3GPP TSG-RAN WG2 Meeting #127	R2-2407304

Maastricht, Netherlands, 19 - 23 August 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on on-demand SSB SCell operation for NES

Agenda Item:	8.5.2

Document for:	Discussion and decision

Introduction

In this contribution, we analyse RAN2 impacts of on-demand SSB SCell operation.

Discussion

Scenarios

RAN1 has discussed the scenarios for on-demand SSB. Apart from Scenario 1 (before SCell is configured), other scenarios are not excluded yet:

|  |  | RAN1 status | Comments |
| --- | --- | --- | --- |
| Scenario 2 | Case1 | supported | On-demand SSB can be used for L3 measurements to determine which SCell to activate. |
|  | Case2 |  | On-demand SSB with a short periodicity can be used for fast L1/L3 measurements, AGC setting, DL synchronisation before SCell activation |
| Scenario 2A | Case1 |  | In both Case #1 and Case #2, the on-demand SSB transmission can be used for cell search, L1/L3 measurements, AGC setting, DL synchronisation. The NES gain for Case #1 is larger, but Case #2 also brings the benefits of SCell activation acceleration. |
|  | Case2 |  |  |
| Scenario 3A | Case1 | FFS | No additional gain (in terms of SCell activation acceleration) compared with Scenario 2/2A. |
|  | Case2 |  | Always-on SSB can serve as QCL-source for TRS, and SSB with shorter periodicity is beneficial for fast SCell activation. |
| Scenario 3B | Case1 |  | Wait for RAN1 to conclude. Motivation needs to be further investigated (e.g. under which case the SSB can be muted or triggered). |
|  | Case2 |  |  |

Proposal 1: RAN2 starts discussion from Scenario #2 (before a configured SCell is activated) and Scenario #2A (upon activating a configured SCell) in combination with Case #1 (no always-on SSB) and Case #2 (always-on SSB). The FFS parts of Scenario #3A and Scenario #3B wait for further RAN1 progress.

There are some related issues on scenario clarification:

- Issue 1: whether on-demand SSB operation applies to CD-SSB or NCD-SSB

- Issue 2: whether to consider combining the R15/R18 SSB-less and on-demand SSB

- Issue 3: whether on-demand SSB in case#2 refers to the adaptation of SSB periodicity or add an additional periodical SSB transmission on top of the always-on SSB

Agreement @ RAN1#116bis - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB

On Issue 1, RAN1 has discussed the corresponding issues with two alternatives, and we think there is no need to restrict the SSB type from the high layer perspective. There could be concerns on impacting legacy UEs, for instance, if an NCD-SSB serves as SCell for a R19 NES UE while serving as PCell for a legacy UE, the on-demand operation may affect the legacy UEs as well. However, there are some implementation methods to avoid such case, for instance, handover can be used to offload RRC_CONNECTED UEs to other cells and cell bar can be used to offload RRC_IDLE/INACTVE UEs to other cells, or TAC in SIB1 can be absent so that this cell cannot serve as PCell (it is captured in the field description of trackingAreaCode that “The absence of the field indicates that the cell only supports PSCell/SCell functionality (per PLMN) or is an NTN cell.”).

On Issue 2, it has been agreed in RAN1 #116 that “RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.” The CA scenarios include both the intra/inter-band CA satisfying SSB-less operation requirements and the CA scenarios not satisfying the SSB-less operation requirements. Actually the CA scenarios not satisfying the SSB-less operation requirements would achieve more benefits from the on-demand SSB operation, while the CA scenarios satisfying the SSB-less operation requirements can simply omit the SSB transmission on the corresponding SCell and only benefit from on-demand SSB in corner cases like when the reference cell is deactivated (it has been agreed by RAN4 that the reference cell should be activated [2]).

On Issue 3, we think the SSB transmission adaptation (including adaptation of periodicity) is discussed in the sub-agenda of “Adaptation of common signal/channel transmissions”. Here, the discussion should be focused on having a periodic SSB transmission for L3 measurements and using on-demand SSB for triggering additional SSB transmission.

Proposal 2: It is up to network implementation to apply on-demand SSB operation to CD-SSB or NCD-SSB.

Proposal 3: RAN2 understands that Case#2 means having a periodic SSB transmission as a baseline and using on-demand SSB for triggering additional SSB transmission.

On-demand SSB configuration

When configuring the SCell, it is useful to notify the UE: 1) whether this SCell supports on-demand SSB transmission; 2) the initial status of the on-demand SSB transmission. Some SSB-related UE behaviours may need to be adjusted accordingly (e.g. SSB-based RRM measurements, SSB-based BFD).

Both RRC-based signalling and MAC CE signalling have been agreed in RAN1#117 to indicate the on-demand SSB transmission:

Agreement (Signalling) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable

Proposal 4: When configuring SCell, whether the SCell supports on-demand SSB and the initial SSB transmission state need to be indicated to the UE.

On-demand SSB triggering

In the WID, three possible triggering method are listed for consideration, UE UL WUS, cell on/off indication via backhaul, and SCell activation/deactivation signalling. They can be categorized into UE-triggered and NW-triggered.

From our perspective, the cell on/off indication via backhaul is more of RAN3 expertise. In R18, RAN3 already introduced inter-node beam activation over F1 and Xn interface, and gNB-CU can request gNB-DU to activate certain SSB beams by indicating the SSB index, and it could be evaluated whether this can be used for on-demand SSB triggering over the F1 interface.

--cite from TS 38.423 v18.0.0 If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

Proposal 5: It is up to RAN3 to decide on the cell on/off indication via backhual.

In RAN1#116 meeting, RAN1 agreed to support NW-triggered on-demand SSB:

Agreement@RAN1#116 Support on-demand SSB SCell operation triggered by gNB. - FFS Details of associated signaling/indication/configuration provided to UE

The remaining issue is whether UE-triggered on-demand SSB needs to be supported in addition. In our understanding, the NW-triggered method is enough, considering the gNB can already obtain the BSR reporting from the UE to determine the UL buffer status, and the gNB is always in control of the DL data scheduling, it is unclear in which use case the triggering cannot be performed by the NW and can only be performed by the UE. Based on the agreed scenario#2 and scenario#2A, UE triggered method is not supported since the motivation is not clear.

Proposal 6: UE-triggered on-demand SSB is not supported at least for Scenario #2 and Scenario #2A.

Regarding the indication signalling design for on-demand SSB triggered by the network, we think MAC CE is an appropriate option. In current spec, the SCell activation/deactivation signaling is based on MAC CE, and this MAC CE can be reused if on-demand SSB is only supported during the SCell activation (i.e. Scenario #2A). For Scenario #2 (i.e. on-demand SSB before SCell activation), a new MAC CE is needed. Considering the above, it would be better to have a unified solution for Scenario #2 and Scenario #2A. The RRC signalling is not preferred since it is less flexible compared with MAC CE and introduces larger delay. As for the DCI, it is up to RAN1’s discussion and the issue on reliability (e.g. confirmation of the L1 signaling) should be considered.

Proposal 7: Introduce a new MAC CE to indicate the on-demand SSB transmission, for both Scenario #2 and Scenario #2A.

If a new MAC CE is introduced, a related question is whether the new MAC CE can also be used for SCell activation/deactivation. From our perspective, the SCell activation/deactivation still uses the legacy MAC CE, in Scenario #2A where the SCell activation and on-demand SSB triggering take place simultaneously, it is up to network implementation to send the new MAC CE and legacy MAC CE together, there is no necessity of complicating the new MAC CE.

Proposal 8: For SCell activation/deactivation, legacy MAC CE is used.

L3 measurements

The existing L3 measurement mechanism can be reused to a large extent. On serving cell measurements, in case1, the UE should stop serving cell measurements when SSB is not transmitted by the serving cell, while in case2 the serving cell measurements can be continuously performed. On neighbour cell measurements, we think on-demand SSB status of neighbour cells should be transparent to the UE (otherwise large signalling overhead in Uu interface and Xn interface are incurred), and it is up to gNB implementation how to handle the reported neighbour cell measurement results which could be degraded based on the intermittent SSB transmission status of neighbour cell.

Proposal 9: On L3 measurements, serving cell measurements in case1 (no always-on SSB) can be stopped when the serving cell is not transmitting SSB; neighbour cell measurements follow legacy L3 measurement behaviour and the SSB transmission status of neighbour cells is transparent to the UE.

Conclusion

In this contribution, we discussed on-demand SSB for intra/inter-band CA, and propose the following:

Proposal 1: RAN2 starts discussion from Scenario #2 (before a configured SCell is activated) and Scenario #2A (upon activating a configured SCell) in combination with Case #1 (no always-on SSB) and Case #2 (always-on SSB). The FFS parts of Scenario #3A and Scenario #3B wait for further RAN1 progress.

Proposal 2: It is up to network implementation to apply on-demand SSB operation to CD-SSB or NCD-SSB.

Proposal 3: RAN2 understands that Case#2 means having a periodic SSB transmission as a baseline and using on-demand SSB for triggering additional SSB transmission.

Proposal 4: When configuring SCell, whether the SCell supports on-demand SSB and the initial SSB transmission state need to be indicated to the UE.

Proposal 5: It is up to RAN3 to decide on the cell on/off indication via backhual.

Proposal 6: UE-triggered on-demand SSB is not supported at least for Scenario #2 and Scenario #2A.

Proposal 7: Introduce a new MAC CE to indicate the on-demand SSB transmission, for both Scenario #2 and Scenario #2A.

Proposal 8: For SCell activation/deactivation, legacy MAC CE is used.

Proposal 9: On L3 measurements, serving cell measurements in case1 (no always-on SSB) can be stopped when the serving cell is not transmitting SSB; neighbour cell measurements follow legacy L3 measurement behaviour and the SSB transmission status of neighbour cells is transparent to the UE.

References

- RP-240170, Revised WID for Enhancements of network energy savings for NR, Ericsson

- R4-2317306, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon

- TS 38.473, v18.0.0