3GPP TSG-RAN WG2 Meeting #127	R2-2407305

Maastricht, Netherlands, 19 - 23 August 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on adaptation of common signal/channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and decision

Introduction

In this contribution, we provide our considerations on adaptation of common signal/channel transmissions.

Discussion

## 2.1 Adaptation of SSB

It is noted in the WID that negative impacts to legacy UEs should be avoided, unless significant benefits are shown.

SSB transmission is important for cell search and initial access. The SSB periodicity is configured in SIB1 or RRC dedicated signalling via field ssb-periodicityServingCell, with a value ranging from 5ms to 160ms. For initial access, the UE always assumes the SSB periodicity is 20ms, as captured in TS 38.213:

A UE can be provided per serving cell by ssb-periodicityServingCell a periodicity of the half frames for reception of the SS/PBCH blocks for the serving cell. If the UE is not configured a periodicity of the half frames for receptions of the SS/PBCH blocks, the UE assumes a periodicity of a half frame. A UE assumes that the periodicity is same for all SS/PBCH blocks in the serving cell. For initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames.

In legacy mechanism, it is already possible by gNB implementation to change the SSB periodicity. In RAN1#116bis, the following has been agreed, which can be taken as a starting point for RAN2 discussion:

Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger

It was further agreed in RAN1#117 that:

Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

RAN1 has agreed to support adaptation of SSB burst periodicity. From RAN2 perspective, we think one simple solution for adapting the SSB transmission in the time domain is to dynamically change the SSB periodicity or activate additional resources. Legacy UEs are configured with a larger periodicity and will not benefit from the additional SSB occasions, while R19 NES UEs can be aware of the additional SSB occasions which are dynamically enabled/disabled by some signalling faster than RRCReconfiguration/SI update.

Proposal 1: For SSB adaptation, SSB periodicity can be dynamically adapted or additional resources are dynamically activated/deactivated for R19 NES UEs, while legacy UEs are configured with a larger SSB periodicity and will not benefit from the additional SSB occasions.

RAN1 has discussed the applicable scenarios for SSB adaptation, and has agreed that the mechanism is supported for the cell with both legacy UEs and R19 NES UEs, and both PCell and SCell are considered for Connected mode UEs. It is still FFS whether UEs in Idle/Inactive mode UEs are taken into account, which might affect whether L1 signalling or MAC CE is used for informing the SSB adaptation, and RAN2 can wait for further RAN1 progress on this. It is also FFS whether the adaptation for SSB is for CD-SSB or NCD-SSB and whether to consider SSBs not on sync raster, we think this is also related to accommodating Idle/Inactive mode UEs, since Idle/Inactive UEs will only consider CD-SSBs on the sync raster for initial access.

Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized

## 2.2 Adaptation of RACH

In RAN#104 meeting, the revised WID has already removed the PRACH adaptation in spatial domain [1].

Regarding RACH adaptation in time domain, RAN1 has agreed to introduce additional PRACH resources for NES-capable UEs in additional to PRACH resources for legacy UEs, the SSB-RO mapping for additional PRACH resources is also separate, but the mapping rule follows legacy SSB-RO mapping rule. It is still FFS whether additional parameters are needed if the PRACH configuration index is the same, and whether additional mechanism is needed if the PRACH configuration index is different. Besides, the signalling for additional PRACH resources also needs to be studied.

Agreement For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least: - a PRACH configuration index - FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources Study further the following - When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource, - Additional parameter(s) for determining the additional PRACH resources e.g. - Scaled/adjusted PRACH configuration period - Additional timing offset - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Muting/masking ROs - When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource - Additional mechanisms (if any) for determining the additional PRACH resources e.g. - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) Agreement For the adaptation mechanism for additional PRACH resources, study further the following: - Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation - Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Strive to re-use existing DCI format(s) - Option 3: Adaptation of PRACH transmission according to predefined condition(s) - FFS: details - Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation - Strive to re-use existing DCI format(s) - Option 5: Enhanced cell DRX

There are other features introducing separate RACH resources, and can be categorized as follows:

- Using RACH partitioning framework, including (e)Redcap, SDT, Coverage enhancement, RAN slicing.

The configuration of RACH resource sets is achieved by separate ROs (via additionalRACH-ConfigList in BWP-UplinkCommon), or separate preamble partitioning with shared ROs (via featureCombinationPreamblesList in RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA).

- Not using RACH partitioning framework, including LTM and twoTA.

The additional RACH configuration for LTM is provided via EarlyUL-SyncConfig in dedicated RRC signalling. The early sync in LTM is RACH towards LTM candidate cell, not serving cell.

The additional RACH configuration for twoTA is provided via additionalRACH-perPCI-ToAddModList and additionalRACH-perPCI-ToReleaseList in BWP-UplinkCommon.

Since RAN1 has agreed in 116bis meeting that the PRACH adaptation is applicable for UEs all RRC states, the additional RACH resources should be provided in SIB1.

Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode

Proposal 2: The additional PRACH resources for NES-capable UEs are provided in SIB1.

On the detailed signalling, we think the first thing to clarify is that whether we incorporate the additional PRACH resources of NES into the RACH partitioning framework. There are two things to be considered: 1) whether there is a need to enable the early identification of NES feature on the network side; 2) whether other RACH-related features like Redcap/SDT/CE can be used jointly with NES.

Take Redcap as an instance, we think it is possible for NES cells to support the access of Redcap UEs, there is no need to exclude the combination of NES and Redcap. RAN2 has discussed whether to prevent legacy UEs from accessing R19 NES cell, and the “legacy UEs” usually refer to the UEs from previous releases not supporting the R19 NES feature. RAN2 can also consider whether UEs of other features (e.g. Redcap UEs) can implement the R19 NES feature and whether these kinds of UEs should be considered when designing the R19 framework.

Proposal 3: RAN2 to discuss whether to incorporate the additional PRACH resources of NES into the RACH partitioning framework, based on two factors: 1) whether there is a need to enable the early identification of NES feature on the network side; 2) whether other RACH-related features like Redcap/SDT/CE can be used jointly with NES.

On CBRA vs CFRA, we think RAN2 should start from CBRA, because UEs in Idle and Inactive states need to be considered and the objective is restricted to common signals. The discussion on CFRA can wait until the mechanism on CBRA is finalized. On 2-step RA vs 4-step RA, we do not see the need to preclude 2-step RA at the current stage.

Proposal 4: CBRA is prioritized for RACH adaptation in time domain.

Proposal 5: Both 4-step RA and 2-step RA are considered for RACH adaptation in time domain.

## 2.3 Adaptation of paging

In this clause, we discuss the two remaining issues from RAN2#125bis meeting: compatibility with legacy UEs, and the solution for adapting paging occasions.

- Compatibility with legacy UEs

In the previous meeting, the following two options were identified for accommodating legacy UEs:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

On Option 1 (barring legacy UEs):

During R18 NES study item phase, RAN1 has discussed common signalling adaptation including paging enhancement, and it is captured in TR 38.864 that legacy UEs follow legacy paging reception procedure in the NES cell:

For (technique A-1-4) the paging enhancement where paging resources are grouped in a compact manner, potential specification impact of the enhancements from paging transmission includes the following: - paging reception procedure (RAN2), i.e., identification of POs and PFs for Rel-18 UEs, - UEs that do not support the technique are expected to follow legacy paging reception procedure in the cell.

From our perspective, the motivation of this objective is to allow both R19 UEs and legacy UEs to camp on the NES cell. The gNB is not prohibited from using the R18 NES cell bar in MIB anyway but no additional spec impact is needed to prevent legacy UEs from receiving paging in the R19 NES cell.

Besides, RAN1 has agreed in the previous meeting that for RACH adaptation, legacy UEs are not excluded, we think the same principle holds for SSB adaptation and paging adaptation as well, considering paging/SSB/RACH resources are all essential components for the Idle/Inactive UEs to access an NES cell:

Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources

Proposal 6: No additional spec impact in R19 to prevent legacy UEs from accessing an R19 NES cell capable of common signal adaptation. The same principle applies to SSB/RACH/Paging adaptation.

On Option 2 (Separate paging configuration):

In our understanding, separate paging resources refer to separate paging configuration/parameters. We think it is reasonable since legacy UEs can only comprehend legacy configuration. The network needs to configure additional paging pattern for R19 UEs, and the NES gain comes from how to distribute the R19 UEs.

Proposal 7: Additional paging configuration/parameter is provided to R19 UEs. R19 UEs only monitor the R19 paging configuration.

- Adaptation of paging occasions

The PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

In legacy mechanism, the PFs and POs are distributed evenly in the paging cycle, and this discontinuous reception of paging is designed for UE power saving, i.e. the UE monitors one PO per cycle.

The paging related parameters from TS 38.331 are excerpted below:

PCCH-Config ::=             SEQUENCE {

defaultPagingCycle                  PagingCycle,

nAndPagingFrameOffset               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15)

},

ns                                  ENUMERATED {four, two, one},

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

...,

[[

nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16        INTEGER (2..4)               OPTIONAL  -- Cond SharedSpectrum2

]],

[[

ranPagingInIdlePO-r17                              ENUMERATED {true}            OPTIONAL,  -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                  OPTIONAL  -- Need R

]]

}

Two solutions have been identified in the previous meeting

- bundle paging frames

This option aims to confine PFs into consecutive frames. It requires changing the PF calculation formula, e.g. (SFN + PF_offset) mod T = (T div N)*(UE_ID SubGroup_ID mod N) [4], or (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) [5].

- extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF

Currently, nAndPagingFrameOffset allows configuring as few as T/16 PFs per paging cycle, some smaller value than T/16 can be introduced to allow fewer PFs per paging cycle; and currently ns allows configuring as many as 4 POs per PF, some larger value can be introduced to allow more POs per PF. Extending the current values will not change the current PF and PO calculation formula and can also decrease the number of PFs leading to large sleep chance.

In both solutions, the NES gain comes from offloading the R19 NES paging messages to condensed POs so the probability for sending paging messages in POs only monitored by legacy UEs will decrease. Take CN paging as an example, the gNB receives paging message from the AMF, and if there is no intended UEs in the upcoming PO (the monitored PO for the intended UEs may occur in later occasions) the gNB does not need to generate a paging message on the radio interface and may have an opportunity for entering sleep states.

Both solutions can only apply to the case where network overload is low, otherwise the legacy UEs may already occupy most of the POs and the paging enhancement will only lead to additional POs brought by NES UEs and the gNB power consumption will be increased instead of reduced.

Among the two solutions, the second option brings less spec impact and is preferred from our perspective.

Proposal 8: Values ranges of nAndPagingFrameOffset and ns can be extended to allow configuring fewer PFs per paging cycle and more POs per PF.

Conclusion

In this contribution, we discussed the adaptation of common signal/channel transmissions, and made the following proposals:

Proposal 1: For SSB adaptation, SSB periodicity can be dynamically adapted or additional resources are dynamically activated/deactivated for R19 NES UEs, while legacy UEs are configured with a larger SSB periodicity and will not benefit from the additional SSB occasions.

Proposal 2: The additional PRACH resources for NES-capable UEs are provided in SIB1.

Proposal 3: RAN2 to discuss whether to incorporate the additional PRACH resources of NES into the RACH partitioning framework, based on two factors: 1) whether there is a need to enable the early identification of NES feature on the network side; 2) whether other RACH-related features like Redcap/SDT/CE can be used jointly with NES.

Proposal 4: CBRA is prioritized for RACH adaptation in time domain.

Proposal 5: Both 4-step RA and 2-step RA are considered for RACH adaptation in time domain.

Proposal 6: No additional spec impact in R19 to prevent legacy UEs from accessing an R19 NES cell capable of common signal adaptation. The same principle applies to SSB/RACH/Paging adaptation.

Proposal 7: Additional paging configuration/parameter is provided to R19 UEs. R19 UEs only monitor the R19 paging configuration.

Proposal 8: Values ranges of nAndPagingFrameOffset and ns can be extended to allow configuring fewer PFs per paging cycle and more POs per PF.

References

- RP-241650, Revised WID: Enhancements of network energy savings for NR, Ericsson

- R2-2403731, Report from session on V2X/SL, R19 NES and MOB, Samsung

- R1-2403692, Summary#4 of AI 9.5.3 for R19 NES, Moderator (Ericsson)

- R2-2402860, Discussion on RAN2 work of common signal transmission adaptation, Apple

- R1-2402889, On adaptation of common signal/channel for NES enhancements, Apple