3GPP TSG-RAN WG2 Meeting #127	R2-2407306

Maastricht, Netherlands, 19 - 23 August 2024

Agenda item:	8.8.2

Source:	Huawei, HiSilicon, Turkcell

Title:	Discussion on DL coverage enhancements

Document for:	Discussion and Decision

- Introduction

In RAN plenary #104 meeting, the revisions to the objective of DL coverage enhancements were agreed [1]:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

RAN2#126 has reached the following agreements:

Agreements;

- Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements.

- RAN2 assumes that both EFC and EMC are supported

In addition to the above agreements, RAN2 also identified some critical issues, on which RAN2 seeks RAN1’s clarifications in LS [2] and RAN1’s replies will serve as the foundation for further RAN2 discussion.

- Ask RAN1 to keep us informed on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values.

- More in general, ask RAN1 whether/how the solution they are investigating is expected to impact common control signalling for UEs in RRC idle / RRC inactive

- Also remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status will have to relate to beams visible to the UE (e.g. SSB beams)

- Also ask RAN1 whether they are also working on UL beam hopping (and whether this is separate from DL beam hopping)

In this contribution, we further discuss the possible system level enhancements from RAN2 perspective.

- Discussion

## 2.1 Cell level vs. beam level

Earlier in RAN2#125bis meeting, RAN2 initially provided the insights on RAN2 aspects of system-level downlink coverage enhancements. The corresponding agreement is listed below:

Agreement: We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input.

RAN1 discussed the system level DL coverage enhancement for the dynamic power sharing among satellite beams, i.e. operating in the granularity of satellite beam. However, only NR cells and NR SSB beams are visible to UEs. In order to transform the satellite beam, we need to figure out the correlation among satellite beam, NR beam, and cell. The following 5 candidate options for the mapping relationship has been provided in [4]：

Option 1: Single NR cell per satellite beam and single NR beam per NR cell;

Option 2: Multi-Satellite beams per NR cell and single NR beams per NR cell;

Option 3: Multi-Satellite beams per NR cell and multi NR beams per NR cell;

Option 4: Multi NR cells per satellite beam;

Option 5: Multi NR cells split across satellite beams.

The mapping issue has remained open for several RAN1 meetings and no conclusion has been reached yet. In our understanding, how the satellite beams map to NR beams is up to network implementation. Unless further down-scoping made in RAN1, RAN2 needs to consider all the above candidate options when analyzing the pros and cons of the cell level Cell DTX/DRX and beam level Cell DTX/DRX.

For Option 1 and Option 4 (where the satellite beam is equal to or larger than an NR cell), the cell level Cell DTX/DRX mechanism is a better choice compared with beam level DTX/DRX, since the satellite beam operation is common to the UEs within the cell coverage. On the other hand, for option 2/3/5 (where the satellite beam is smaller than an NR cell), the satellite beam operation affects a portion of an NR cell, which makes the cell level cell DTX/DRX unsuited for these options.

Beam level DTX/DRX has a finer granularity, and the beam level pattern can take a non-uniform distribution of users and of traffic load into account by setting the active/inactive duration and cycle. Therefore, a beam level DTX/DRX is suitable to address all scenarios. To support beam level DTX/DRX, the network should provide the UE with multiple DTX/DRX patterns associated with each beam. When the UE switches its DL SSB beam to a new DL SSB beam, the UE needs to switch to the DTX/DRX pattern associated with the new DL SSB beam. Since some fundamental assumptions are still pending on RAN1 discussion, we propose RAN2 to wait for sufficient RAN1 progress before further RAN2 discussion.

Proposal 1: RAN2 waits for RAN1 input before further discussion on cell level and beam level Cell DTX/DRX pattern.

## 2.2 Implications of SSB periodicity extension

As the revised WID states, the SSB periodicity extension is further considered. RAN1 is now discussing whether to extend the SSB periodicity to be larger than 160ms, e.g. 320ms. If RAN1 decides to support 320ms for SSB periodicity, RAN2 need to extend the ssb-periodicityServingCell to support 320ms. Besides, some other configuration like SMTC and gap has a periodicity related to SSB periodicity, these values also needs to be extended accordingly.

Proposal 2: If RAN1 decides to support 320ms for SSB periodicity, RAN2 need to extend the ssb-periodicityServingCell, SSB-MTC, SSB-MTC4-r17 and mgrp to support 320ms.

- Conclusion

This contribution discussed the DL coverage enhancements in Rel-19 NR NTN. Proposals are summarized as follows:

Proposal 1: RAN2 waits for RAN1 input before further discussion on cell level and beam level Cell DTX/DRX pattern.

Proposal 2: If RAN1 decides to support 320ms for SSB periodicity, RAN2 need to extend the ssb-periodicityServingCell, SSB-MTC, SSB-MTC4-r17 and mgrp to support 320ms.

- References

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

R2-2405762, LS on DL coverage enhancements, CMCC

R1-2401937, Report of RAN1#116 meeting, ETSI MCC

R1-2400303, Discussion on NR NTN Downlink coverage enhancements, Thales