3GPP TSG-RAN WG2 Meeting #127	R2-2407307

Maastricht, Netherlands, 19 - 23 August 2024

Agenda item:	8.8.4

Source:	Huawei, HiSilicon, Turkcell

Title:	Discussion on MBS broadcast over NTN

Document for:	Discussion and Decision

- Introduction

One of the objectives for Rel-19 NTN WI is the support of broadcast service over NTN:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

During RAN2 #126 meeting, the following agreements were made:

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

In this contribution, we provide our views on the remaining issues.

- Discussion

RAN2 narrowed down the methods for providing the intended area and consider these following possibilities: SIB20, SIB21, or MBSBroadcastConfiguration. Each candidate for provision of intended service area has different implications, since SIB20, SIB21 and MBSBroadcastConfiguration take effect at different stages of the attempt to receive MBS service:

- SIB20 is used to convey the MCCH/MTCH configuration information (the frequency resource for MCCH and MTCH, the time resource for MCCH). If the serving cell sends SIB20, UE will receive the MCCH of this cell.

- SIB21 provides the information related to service continuity of MBS broadcast, i.e., the mapping of MBS FSAI and the current/ neighbor frequencies. If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider the MBS frequency to be the highest priority during the cell selection procedure.

- MBSBroadcastConfiguration message (provided via MCCH) is used to convey the control information for MBS (including the MRB configuration, PDSCH configuration for MTCH per MBS session, and neighbor cells which are also broadcasting this MBS session). UE will apply the corresponding configuration of an MBS session, e.g., the scheduling information and the MRB configuration per MBS session. Besides, the UE also determine whether the MBS session of interest is provided in any neighbor cell or not.

Provision of intended service area by SIB20 and MBSBroadcastConfiguration are used to direct the UE in the current serving cell to further acquire/apply the necessary parameter for the MBS session based on whether the UE is located in the intended service area or not. We observe that it is desirable to associate the intended service area with MBS session. MBSBroadcastConfiguration is able to associate each MBS session with its intended service area naturally since MBSBroadcastConfiguration already has the MBS session ID list. In contrast, when SIB20 provides the intended service area, the UE could only determine whether to further acquire parameters on MCCH based on whether the UE is located in the intended area. Certainly, SIB20 could be enhanced to indicate MBS Session ID list and corresponding intended area of each MBS session, but this incurs more specification impacts.

Proposal 1: Provide the intended service area of each MBS session via MBSBroadcastConfiguration.

As per current specification TS 23.247, only UEs within the MBS service area may receive content data, while UEs outside the MBS service area are not allowed to receive the location specific content. When the UE is not within the MBS service area, there is no need for enforcement in the current RRC specification to forbid the UE to establish the corresponding MRB according to the MBSBroadcastConfiguration, since the UE may enter the service area again shortly after leaving the area, also such detailed UE behaviors like establishing/releasing MRBs should follow legacy MBS broadcast procedure. In our understanding, we do not need to apply this enforcement to the UE outside the intended area of an NTN cell either. When the UE leaves the intended area, the area-specific content data may not be applicable to the UE any more. It is up to UE implementation to deicides whether or not to further receive the content data. No strict enforcement for the UE needs to be specified.

Proposal 2: No strict enforcement for the UE needs to be specified, e.g., no MRB establishment for UE outside the intended area, MRB release for UE leaving the intended area.

Apart from MBS session ID list of the serving cell, MBSBroadcastConfiguration also indicates the MBS session ID list provided by the neighbor cells. Therefore, if MBSBroadcastConfiguration is responsible for associating the MBS session with intended area, both current serving cell’s and neighbor cell’s MBS session specific intended area should be indicated to UEs. This allows the UE to request unicast reception of the interested service before moving to a cell not providing the MBS session in UE’s location area.

Proposal 3: Both current serving cell’s and neighbor cell’s MBS session specific intended area should be indicated to UEs via MBSBroadcastConfiguration.

Proposal 4: Based on the neighbor cell’s MBS session specific intended area, the UE requests unicast reception of the interested service before moving to a cell not providing the MBS session in UE’s location area.

SIB21 is used to guarantee the service continuity of MBS session when the UE initiates the cell reselection procedure. The UE could prioritize an MBS frequency on which MBS session of interest is provided when the UE performs the cell reselection. With the provision of intended service area per MBS frequency by SIB21, the UE know the exact intended service area where a given frequency provides the desirable MBS session. Therefore, it is beneficial for the UE to treat an MBS frequency as the highest priority based on UE’s location and intended area per MBS frequency during cell reselection.

Proposal 5: The UE determine to camp on an MBS frequency as the highest priority based on UE’s location and intended area per MBS frequency during cell reselection.

- Conclusion

In this paper we discuss the support of broadcast service via NR NTN, and propose the following:

Proposal 1: Provide the intended service area of each MBS session via MBSBroadcastConfiguration.

Proposal 2: No strict enforcement for the UE needs to be specified, e.g., no MRB establishment for UE outside the intended area, MRB release for UE leaving the intended area.

Proposal 3: Both current serving cell’s and neighbor cell’s MBS session specific intended area should be indicated to UEs via MBSBroadcastConfiguration.

Proposal 4: Based on the neighbor cell’s MBS session specific intended area, the UE requests unicast reception of the interested service before moving to a cell not providing the MBS session in UE’s location area.

Proposal 5: The UE determine to camp on an MBS frequency as the highest priority based on UE’s location and intended area per MBS frequency during cell reselection.

- References

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

R2-2405703, Report from Break-Out Session on NR NTN and IoT NTN, Session chair (ZTE)