3GPP TSG-RAN WG2 Meeting #127	R2-2407308

Maastricht, Netherlands, 19 - 23 August 2024

Agenda Item:	8.8.5

Source:	Huawei, HiSilicon, Turkcell

Title:	Discussion on regenerative payload

Document for: Discussion and Decision

# 1	Introduction

In the last meeting the following agreements were achieved for the support of regenerative payload in NR NTN [1].

Agreements:

- Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

- Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

In this paper we provide our views on the stage-2 impacts brought by the regenerative payload.

# 2	Discussion

2.1	Common TA and Kmac

Last RAN2 meeting agreed the common TA and kmac would be typically set to zero in case of regenerative payload. In case of transparent payload, considering the UE cannot compensate the whole TA by itself, the reference point (RP) was introduced. The UE is responsible for compensating the TA of UE-satellite-RP, and the TA of RP-gNB is compensated by the gNB. But actually, the UE’s GNSS position may be not precise enough, considering the UE may not turn on the GNSS frequently and the UE is moving. The outdated GNSS position may cause some errors for the TA calculation at the UE, in order to leave some room for NW compensation by sending the NTA in RAR (which can only be a positive value), the common TA is usually set to a relatively smaller value than the common TA relevant to the ideal RP, i.e., the practical RP is set a little closer to the UE compared with the ideal RP.

Observation 1: The practical RP may be set closer to the UE compared with the ideal RP, considering the TA calculation errors at the UE.

Following legacy principle, for the regenerative payload, the ideal RP should be just the same as the gNB/satellite. But considering the above analysis for the TA calculation errors at the UE, the practical RP may be located closer to the UE, i.e., between the UE and the on-board gNB. In this condition, the common TA is not zero but a negative value. Capturing the common TA shall/should be zero brings high requirements for the UE implementation. And introducing the negative common TA has smaller spec impact compared with introducing an additional component in the TA calculation. We propose to not capture in spec that the common TA should be zero, and to agree the common TA can be negative. For the new value range of common TA, we can further prepare LS for asking RAN1/4.

Proposal 1-1: Not to capture in 38.300 that for regenerative the common TA and Kmac shall/should be set to zero.

Proposal 1-2: RAN2 to agree the negative common TA values, and approve the LS to RAN1/4 on common TA value range in the annex.

2.2	RACH-less Optimization

In the last meeting one company proposed a RACH-less optimization solution [2]: the target gNB may be informed the time when the UE is present in the target cell, to reduce the waste of resources at the target cell.

We understand the motivation for such optimization may be mainly for the dynamic grant, in which the target cell attempts to schedule the UE after sending the HO command. And considering the long delay for the inter-satellite Xn exchange (either via ISL or via GW), the target cell may attempt to schedule the UE excessively and waste much resources before the UE actually access the target cell. However, we do not think the Xn delay should be a bottleneck. Even in the transparent scenario, the HO command still has long delay, and that is not a problem for the RACH-less handover. There is no need for any further optimisations.

Proposal 2: Current RACH-less handover can be directly supported in regenerative payload. There is no need for any further optimisations.

# 3	Conclusion

This contribution analyses whether any existing essential features would be affected by the regenerative payload and if there are any enhancements needed for regenerative payload, and proposes the following:

Common TA and Kmac

Observation 1: The practical RP may be set closer to the UE compared with the ideal RP, considering the TA calculation errors at the UE.

Proposal 1-1: Not to capture in 38.300 that for regenerative the common TA and Kmac shall/should be set to zero.

Proposal 1-2: RAN2 to agree the negative common TA values, and approve the LS to RAN1/4 on common TA value range in the annex.

RACH-less optimization

Proposal 2: Current RACH-less handover can be directly supported in regenerative payload. There is no need for any further optimisations.

# 4	References

- R2-2405703, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 126.

- R2-2405148, On the feasibility of the existing NTN features over regenerative architecture, Nokia, RAN2 126.

# Annex – draft LS to RAN1/RAN4

# 1. Overall Description:

Regarding regenerative payload, RAN2 has discovered that there may be a need to broadcast negative common TA values to accommodate the TA estimation errors at the UE, considering the UE may apply a relatively larger value for the UE specific TA.

Question to RAN1/RAN4: Is there any need to define negative values for ta-Common and what could be the specific value range?

# 2. Actions:

To RAN1 and RAN4

ACTION: 	RAN2 kindly asks RAN1 and RAN4 to consider the above question and provide feedback if any.

# 3. Date of Next RAN2 Meetings:

3GPP TSG RAN WG2#127bis	14 - 18 Oct, 2024		Hefei, CN

3GPP TSG RAN WG2#128	18 - 22, Nov, 2024		Orlando, US