3GPP TSG-RAN WG2 Meeting #127	R2-2407309

Maastricht, Netherlands, 19 - 23 August 2024

Agenda item:	8.8.6

Source:	Huawei, HiSilicon, Turkcell

Title:	Discussion on LTE TN to NR NTN mobility

Document for:	Discussion and Decision

- Introduction

In RAN plenary #103 meeting, a work item on IDLE mode mobility from E-UTRA TN to NR NTN was approved [1] with the following objective:

The work item aims at specifying support for the following objectives:

- idle mode mobility based on cell reselection from E-UTRA TN to NR NTN, where E-UTRA TN provides satellite information for NR NTN neighbor cells in a System Information Block. [RAN2]

Note:

- The solutions shall be reusing the existing signaling introduced for IoT-NTN and mobility procedures between NR TN and NR NTN to the maximum extent.

During RAN2 #126 meeting the following agreements were made:

Agreements:

- For idle mode mobility from EUTRA TN to NR NTN, NB-IoT UEs are considered not in the scope.

- For idle mode mobility from EUTRA TN to NR NTN, we don’t consider specific optimizations for BL UEs and UEs in CE.

- SIB24 is reused to provide the NR NTN cell reselection related information (e.g. frequency information, SMTC config, etc.), introducing a satellite ID list in per frequency. The EUTRA cell provides the satellite assistance information for NR neighbor cell per satellite, as identified by the satellite ID.

4.	To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbor cells is needed and should include the following parameters:

-	Satellite ephemeris information

-	TA common information

-	k-Mac

-	epoch time

-	validity duration

-	ntn-PolarizationDL (FFS if mandatory or optional)

5.	The Ephemeris information/epoch time/k-mac/validity duration IEs defined in SIB33 specified in TS36.331 should be reused for NR satellite assistance information.

6.	The signalling format for ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information.

7.	RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur):

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

8.	Introduce the clarification in the field description of measTimingConfig (configured via SIB24 in TS 36.331) that it is configured based on the assumption that the gNB-UE propagation delay equals to 0 ms, and UE can adjust the offset based on the actual propagation delay, when the corresponding frequency is associated with a satellite ID.

Working Assumption:

- NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

In this paper we discuss the remaining issues.

- Discussion

Provision of NR satellite assistance info

We further discuss the following three options for the provision of the NR satellite assistance information:

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

In TS 38.331, the NR satellite assistance information is provided by NTN-Config IE in SIB19, the assistance information for each NTN neighbor cell is as follows:

NTN-NeighCellConfig-r17 ::= SEQUENCE { ntn-Config-r17 NTN-Config-r17 OPTIONAL, -- Need R carrierFreq-r17 ARFCN-ValueNR OPTIONAL, -- Need R physCellId-r17 PhysCellId OPTIONAL -- Need R } NTN-Config-r17 ::= SEQUENCE { epochTime-r17 EpochTime-r17 OPTIONAL, -- Need R ntn-UlSyncValidityDuration-r17 ENUMERATED{ s5, s10, s15, s20, s25, s30, s35, s40, s45, s50, s55, s60, s120, s180, s240, s900} OPTIONAL, -- Cond SIB19 cellSpecificKoffset-r17 INTEGER(1..1023) OPTIONAL, -- Need R kmac-r17 INTEGER(1..512) OPTIONAL, -- Need R ta-Info-r17 TA-Info-r17 OPTIONAL, -- Need R ntn-PolarizationDL-r17 ENUMERATED {rhcp,lhcp,linear} OPTIONAL, -- Need R ntn-PolarizationUL-r17 ENUMERATED {rhcp,lhcp,linear} OPTIONAL, -- Need R ephemerisInfo-r17 EphemerisInfo-r17 OPTIONAL, -- Need R ta-Report-r17 ENUMERATED {enabled} OPTIONAL, -- Need R ... } EpochTime-r17 ::= SEQUENCE { sfn-r17 INTEGER(0..1023), subFrameNR-r17 INTEGER(0..9) } TA-Info-r17 ::= SEQUENCE { ta-Common-r17 INTEGER(0..66485757), ta-CommonDrift-r17 INTEGER(-257303..257303) OPTIONAL, -- Need R ta-CommonDriftVariant-r17 INTEGER(0..28949) OPTIONAL -- Need R } EphemerisInfo-r17 ::= CHOICE { positionVelocity-r17 PositionVelocity-r17, orbital-r17 Orbital-r17 }

As the “note” in the WID states, the solutions shall be reusing the existing signaling introduced for IoT-NTN and mobility procedures between NR TN and NR NTN to the maximum extent. The yellow highlighted IEs could be represented by the current IEs in SIB33 defined in TS36.331, while the green highlighted IEs are different or missing in SIB33 (on the value ranges of common TA parameters, we think it is acceptable to use IoT NTN granularity for NR NTN neighbor cells, and this is manageable by OAM, so the only IE missing is the DL polarization parameter). Since the majority of IEs can reuse SIB33 parameters, we propose to adopt Option 3.

Proposal 1: RAN2 to adopt Option 3, i.e. extend the NeighSatelliteInfo defined for IoT NTN for provision of NR satellite info, so that the signaling for IoT NTN can be reused to the maximum extent.

## 2.2 NR NTN cell reselection evaluation

Currently the inter-RAT cell reselection evaluation for EUTRA-to-NR mobility supports the RRM-based measurement mechanism, i.e. the UE evaluates whether the signal quality of measured cell meets the threshold during a time interval TreselectionRAT. The corresponding inter-RAT Cell Reselection criteria is defined in TS 36.304 as below:

If threshServingLowQ is provided in SystemInformationBlockType3 and more than 1 second has elapsed since the UE camped on the current serving cell and if the measurements are not performed using RSS as specified in [10], cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN, NR or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

We observe that there are some concerns that the common TreselectionRAT for both NR NTN and NR TN neighbor cells would be too large for NR NTN considering that the UE may be more eager to camp on a satellite in case a satellite leaves soon. In our understanding, the concern would be unnecessary. First, the idle mobility is less sensitive to delay compared with connected mode mobility. Secondly, the value range of TreselectionRAT is from 0s to 7s, which is relative short compared to the coverage time of satellite. There is no need to shorten the evaluation period for idle mobility towards NR NTN, and the setting of TreselectionRAT is up to NW implementation. Therefore, no spec impact is foreseen for cell reselection evaluation mechanism.

Proposal 2: Confirm the working assumption: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

- Conclusion

This document discusses LTE TN to NR NTN mobility, and proposes the following:

Proposal 1: RAN2 to adopt Option 3, i.e. extend the NeighSatelliteInfo defined for IoT NTN for provision of NR satellite info, so that the signaling for IoT NTN can be reused to the maximum extent.

Proposal 2: Confirm the working assumption: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

- References

[1] RP-240846, New WID: Inter RAT mobility support from E-UTRAN TN to NR-NTN, Thales, CATT

[2] R2-2405703, Report from Break-Out Session on NR NTN and IoT NTN, Session chair (ZTE)