3GPP TSG RAN2 Meeting #127                                                               R2-2407310

19th Aug. – 23rd Aug. 2024, Maastricht, Netherlands

Agenda item:	8.4.2

Source:	Samsung

Title:	Procedure and Configuration of LP-WUS in RRC Idle /
Inactive Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1]. And during the previous meeting #126, with respect to procedure and configuration of LP-WUS in RRC idle/inactive mode agenda, RAN2 agreed as follows:

Entry/exit condition RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. Sub-grouping RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. SI reception From RAN2 perspective, no new procedure is introduced for SI reception/updates.

And RAN1 reached the following agreements regarding procedures and configurations for idle/inactive modes during meeting #117:

Agreement It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options: - Option 1: UEs monitoring the same PO monitor the same LO - Option 2: UEs corresponding to different POs monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. - Combinations of the above options can be considered. Agreement For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256. - FFS the exact value of X.

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

Based on the findings from the LP-WUS/WUR study conducted during the Study Item (SI) phase in Release 18, it has been proposed that LP-WUS/WUR should be implemented as a power saving mechanism specifically designed for RRC_IDLE and RRC_INACTIVE mode devices. Furthermore, the concept of an "ultra-deep sleep state" has emerged as a potential power saving strategy for both RRC_IDLE and RRC_INACTIVE modes. By leveraging LP-WUS/WUR, a user equipment's (UE) Main Receiver (MR) can effectively transit into an ultra-deep sleep state, thereby achieving significant power savings. While in this ultra-deep sleep state, the UE can use its LP-WUS/WUR functionality to continuously monitor network signaling activities, waking up the MR when necessary.

- Configuration

The UE needs to recognize the resource on which the LP-WUS will be transmitted, regardless of whether continuous or duty cycle-based LP-WUS monitoring is used. It is essential that both the UE and gNB have a mutual understanding of this resource, and therefore, it should be communicated from the network to the UE.

For such configuration, RAN2 agreed to utilize a system information as the container. Considering a cell-specific monitoring occasion (MO), which closely resembles a PRACH configuration, we may regard SIB1 as a potential candidate for signaling LPWUS occasions (Los), as illustrated in Figure 1. It should be noted that SIB1 will be delivered through MR, whereas LP-WUS will be transmitted through LP-WUR.

Figure 1 LO configuration and LP-WUS monitoring

And we propose RAN2 to discuss and determine the configuration of LP-WUS occasions, as in SIB1.

Proposal 1. The LP-WUS related configuration for IDLE/INACTIVE state is provided via SIB1.

- Entry/Exit condition

According to the previous RAN2 discussion, now we have the entry/exit condition(s) of LP-WUS monitoring configured in SIB (e.g., SIB1).

As a baseline, RAN2 agreed the below entry/ exit conditions:

- Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

- Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

For the entry requirement, there is currently debate on whether or not the entry condition should include the low-power receiver (LR) measurement of serving cell quality being above the threshold. Some argue that the LR-based measurement is necessary to prevent any potential pingpong effect, such as exiting (restarting legacy PO monitoring) immediately after meeting the entry condition (stopping monitoring legacy PO).

However, including this additional LR measurement requirement would result in increased UE energy consumption, which contradicts the purpose of this work item. Considering intra-band LP-WUS operating in the main radio (MR) frequency band, it is highly unlikely that the measurements of LR and MR differ significantly. This implies that LR and MR measurements can be used interchangeably, enabling 'offloading via LR' within this work item.

Here, for triggering LP-WUS monitoring and stopping MR paging occasion (PO) monitoring, we suggest using a well-specified threshold of MR to represent reliable LR reception quality. In this case, UEs do not require monitoring both MR and LR to initiate LP-WUS monitoring and save more energy.

Consequently, we propose excluding the LR-based measurement from the entry condition. Proposal 2 states that the LR-based measurement requirement is unnecessary for the entry condition due to its potential additional power consumption and replaceability with MR measurement.

Proposal 2: For the entry condition, the LR-based measurement requirement is unnecessary due to the potential additional power consumption and the fact that it can be substituted with MR measurements.

For the exit requirement from LP-WUS monitoring, in addition to the serving cell measurement result based on LR, the UE must terminate LP-WUS monitoring when either of the following conditions occur:

- Upon receiving an LP-WUS signal whose Subgroup ID matches that of the UE; or

- In the presence of MO traffic.

These conditions are necessary for exiting LP-WUS monitoring, although they might not necessarily belong to the 'exit condition' itself. This behavior could potentially be detailed through LP-WUS reception and subsequent UE actions, as well as MO traffic-originated actions.

Proposal 3: RAN2 discuss and agree the following additional termination criteria for LP-WUS monitoring:

- Upon receiving an LP-WUS signal whose Subgroup ID matches that of the UE; or

- In the presence of MO traffic.

If the serving cell quality is above threshold, UE monitors LPWUS using LR. Even though UE switches to LR and monitors LPWUS, UE have to regularly wakeup the MR for inter frequency measurements wherein the UE measures the SSBs transmitted on neighboring frequency for cell (re-)selection.  Note that LR cannot be used neighbor cell measurements. Waking up MR and again switching to LR after each measurement leads to increased power consumption. Additionally, long time to wake up MR and switching to LR may lead to UE missing paging indication.

So, we propose that if criteria for inter frequency/inter RAT frequency measurement is met, UE does not monitor LPWUS. If UE is already monitoring LPWUS, it stops monitoring LPWUS and wakes up the MR and monitor PEI/PO for paging and also perform inter frequency/inter RAT frequency measurement using the MR.

Proposal 4: If criteria for inter frequency/inter RAT frequency measurement is met, UE does not monitor LPWUS. If UE is already monitoring LPWUS, it stops monitoring LPWUS and wakes up the MR and monitor PEI/PO for paging and also perform inter frequency/inter RAT frequency measurement using the MR.

In line with the RAN1 agreement, 'The entry/exit thresholds can be individually set for various types of LR'. As such, for UEs in Idle or Inactive mode, cells utilizing LP-WUS might necessitate separate configuration of entry and exit thresholds for each distinctive LR type. The sent entry and exit criteria must possess the ability to specify the associated LR.

Proposal 5. The broadcasted entry/exit conditions should be capable of specifying the applicable LR type.

- LP-WUS monitoring occasions

Within the LP-WUS configuration, RAN2 needs to comprehend the characteristics and define the LP-WUS occasions (LOs).

Each LO contains one or more LP-WUS monitoring occasions (MOs), during which the UE can monitor LP-WUS transmissions within every LP-WUS MO. The following characteristics and elements of LP-WUS MOs are considered:

- LP-WUS MOs are cell-specifically configured and utilized

- Different LP-WUS MOs may correspond to different beams, in multi-beam operation [2]

- An LP-WUS configuration may consists of

  - MO information

    - a time window [2]

    - periodicity

    - timing offset

    - frequency information

  - subgrouping information

  - entry/exit condition information

Proposal 6. RAN2 needs to discuss and make decisions on the elements of LP-WUS occasions (LO) comprising of e.g., time window, periodicity, timing offset, frequency information, and subgrouping information.

- Subgrouping

To further enhance the energy saving gain, RAN2 agreed to take PEI subgrouping method as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping.

For UEs in RRC idle/inactive state, LP-WUS can either replace PEI (LP-WUS → PO) or coexist with PEI (LP-WUS → PEI → PO). Thus, in the above two different cases, subgrouping of LP-WUS can limit the monitoring of PF/PO, or PEI occasions, respectively.

- Scenario 1) When LP-WUS replaces PEI and subgroups are applied on LP-WUS, it reduces the number of PF/PO that UEs need to monitor.

- Scenario 2) When both LP-WUS and PEI exist and subgroups are applied on LP-WUS and PEI, it reduces the number of PEI occasions/ PF/PO that UEs need to monitor.

For Scenario 1, where UE monitors PO upon receiving LPWUS signal, UEs monitoring a PO can be divided into several LPWUS subgroups based on UE_ID.  LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +   
                  (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

For Scenario 2, where UE first monitors PEI upon receiving LPWUS signal and then monitors PO (if indicated by PEI).  LP WUS subgroup ID based on UE_ID can be determined using one of the following options:

Option 1 (LPWUS subgroup ID is independent of paging subgroup ID used for PEI):

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +  
(LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

Option 2 (LPWUS subgroup ID is dependent on paging subgroup ID, hierarchical subgroups):LP WUS subgroup ID = paging subgroup ID mod ‘number of UE ID based LPWUS subgroups’.

Option 1 seems simpler as same method can be used for both scenario 1 and scenario 2.

Proposal 7: LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +               (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

Proposal 8: RAN2 to discuss whether the following scenario is supported (if both UE and cell supports PEI)

- UE first monitors PEI upon receiving LPWUS signal and then monitors PO (if indicated by PEI).

- 3. Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. The LP-WUS related configuration for IDLE/INACTIVE state is provided via SIB1.

Proposal 2: For the entry condition, the LR-based measurement requirement is unnecessary due to the potential additional power consumption and the fact that it can be substituted with MR measurements.

Proposal 3: RAN2 to discuss and agree the following additional termination criteria for LP-WUS monitoring:

- Upon receiving an LP-WUS signal whose Subgroup ID matches that of the UE; or

- In the presence of MO traffic.

Proposal 4: If criteria for inter frequency/inter RAT frequency measurement is met, UE does not monitor LPWUS. If UE is already monitoring LPWUS, it stops monitoring LPWUS and wakes up the MR and monitor PEI/PO for paging and also perform inter frequency/inter RAT frequency measurement using the MR.

Proposal 5. The broadcasted entry/exit conditions should be capable of specifying the applicable LR type.

Proposal 6. RAN2 needs to discuss and make decisions on the elements of LP-WUS occasions (LO) comprising of e.g., time window, periodicity, timing offset, frequency information, and subgrouping information.

Proposal 7: LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +               (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

Proposal 8: RAN2 to discuss whether the following scenario is supported (if both UE and cell supports PEI)

- UE first monitors PEI upon receiving LPWUS signal and then monitors PO (if indicated by PEI).

- 4. References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

[2] Draft_Minutes_report_RAN1#116_v030, Draft Report of 3GPP TSG RAN WG1 #116 v0.3.0 (Athens, Greece, February 26th - March 1st, 2024), RAN1