3GPP TSG RAN2 Meeting #127                                                               R2-2407313

19th Aug. – 23rd Aug. 2024, Maastricht, Netherlands

Agenda item:	8.11.2

Source:	Samsung

Title:	Random access in SBFD

Document for:	Discussion & Decision

Introduction

The Rel-19 WID Evolution of Duplex operation introduces NR specification support for gNB-side SBFD within a TDD carrier, enhancements to inter-gNB and inter-UE CLI handling, and BS RF and RRM requirements [1].

In the Rel-19 WID, it is assumed that SBFD is deployed at the gNB side and that the TDD UE operates in half-duplex. Subband full-duplex (SBFD) operation on a time-domain resource is limited to FDM in non-overlapping DL and UL subbands. SBFD enhancements for SBFD-aware UEs assume SBFD operation for NR single carrier in FR1 or FR2-1. From the perspective of the SBFD-aware UE, SBFD operation is assumed within a single configured DL and UL BWP pair with aligned center frequencies. For the semi-static indication of frequency/time locations of the SBFD subbands to SBFD-aware UEs on a legacy DL or F symbol/slot, it is assumed that one UL subband is configured for SBFD operation in an SBFD symbol.

The Rel-19 WID objectives include introduction of specification support for random access in SBFD symbols at least for RRC_CONNECTED mode and a study phase for RRC_IDLE/INACTIVE modes [1].

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2]

- Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2]

  - RAN#104 to check whether to proceed normative work

During the previous meeting #117, RAN1 agreed as follows:

Agreement A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default. A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE: - The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH. - UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols - There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH. - Other assumptions are not precluded. NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB. Agreement Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode. Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options. Conclusion For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1: - The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs. Agreement For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, consider whether additional condition(s) on top of what was agreed in RAN1#116bis for RO validation are necessary. For example, - Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB. - Condition#3: A valid RO does not precede a SSB in the PRACH slot. Agreement Update the following agreement made in RAN1#116-bis meeting: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, adopt the following: - Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - For FR1, consider from the following alternatives: - Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. - Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211).

In this contribution, we would like to discuss the detailed random access in SBFD symbols in RRC Connected and Idle/Inactive mode.

Discussion

To increase the SBFD symbol utilization, random access in SBFD symbols are proposed and studied. Random access in SBFD symbols increases the number of available UL transmission opportunities in the TDD UL-DL frame configuration period for the PRACH/preamble transmissions and reduces the random access latency.

According to RAN1 discussion so far, there could be a RACH Occasion (RO) within a SBFD symbol. And such RO may start from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. Figure 1 illustrates an example of RO across two SBFD slots.

Utilizing such RO in SBFD symbols, RAN1 and RAN2 require to design/ extend the existing random access procedure such as RO configuration, Msg.1/preamble transmission, and UE power ramping behaviour.

Figure 1 An example of RO across SBFD slots

RACH configurations

In RAN1#116, two possible RACH configuration options for SBFD-aware UEs were identified. Note that as by RAN1#116bis agreement, Option 1 does not allow to configure separate prach-ConfigurationIndex.

- Option 1: Use one single RACH configuration with possible enhancement

- Option 2: Use two separate RACH configurations (one legacy and one additional RACH configuration)

In RAN1#117s, the following WA was confirmed and agreed as RACH configuration Options 1 and 2:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

To ensure clarity in RAN2 discussions and progress, we suggest confirming the RAN1#117 Agreement as follows:

Proposal 1: Confirm the RAN1#117 agreement stating support for both RACH Configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH Configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration).

The primary motivation behind RACH Configuration Option 1 with Alt. 1-1 is to minimize signalling overhead. By employing a single RACH configuration, UEs with SBFD capabilities can differentiate whether the RO coincides with separately configured SBFD slots/symbols or within a legacy UL slots. Consequently, these SBFD-aware UEs can utilize the RO on SBFD. On the other hand, UE without SBFD capabilities (legacy UE) may only be able to distinguish ROs on legacy UL slots as the effective ROs.

Proposal 2: Under RACH Configuration Option 1 with Alternative 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), SBFD-aware UEs can determine if the RO aligns with SBFD resources or falls within traditional UL resources.

In order to implement the RACH configuration Option 2 with two distinct RACH configurations, it is logical for RAN2 to define an additional RRC IE specifically for the SBFD based RACH configuration, and signal this configuration through system information. Proposal 3 suggests that for RACH Configuration Option 2 (which involves using two separate RACH configurations, one being a traditional RACH configuration and another being an additional RACH configuration), RAN2 should specify an extra RRC IE dedicated to the novel SBFD based RACH configuration.  Also, considering different aspects of SBFD resources and legacy UL resources, the SBFD RACH configuration may require to have different parameters such as power control related parameters.

Proposal 3: For RACH Configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), RAN2 needs to specify a new RRC IE for the new SBFD based RACH configuration with a new set of parameters.

Similarly, Proposal 4 indicates that for RACH Configuration Option 2 (again involving two separate RACH configurations, one traditional and one additional), the fresh SBFD based RACH configurations will be delivered through system information, as per tradition.

Proposal 4: For RACH Configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), the new SBFD based RACH configurations will be delivered through system information (e.g., SIB1), as per legacy RACH configuration.

And we believe an SBFD capable UE should also be able to utilize not only the SBFD based RACH, but also the legacy RACH.

Proposal 5: The SBFD capable UE may utilize not only the SBFD based RACH, but also the legacy RACH.

Type-2 Random Access

In the Type-2 random access procedure, RACH Msg.1 and Msg.3 (PUSCH) are transmitted together as MsgA. The RACH Msg.2 (RAR) and Msg.4 (for contention resolution) are transmitted together as MsgB. Type-2 is applicable for UEs in RRC_CONNECTED and in RRC_IDLE/INACTIVE modes. Both CBRA and CFRA are possible for Type-2 random access. For MsgA transmission, the gNB configures the MCS and PUSCH allocation. For MsgB reception, a new RNTI is defined. The Type-2 procedure reduces signaling overhead and decreases RA procedure delay.

In Rel-16 NR, the random access causes for Type-1 are considered as valid for Type-2 random access. No new/additional random access cause is introduced for Type-2.

When Type-2 random access is available and configured in the serving cell, the UE selects the Type-1 or Type-2 procedure at the initiation of the random access procedure based on the network configuration.

Most expected design and specification impact to support Type-1 random access procedure in SBFD symbols occur with respect to the Msg.1/preamble transmission such as RO configuration/validation, SSB-RO mapping rules, and UE power ramping behavior. Support of the Type-2 procedure for MsgA transmission shares these common design aspects. For (Type-2) PUSCH in MsgA in SBFD symbols, network configuration can be used similar to existing Rel-16 NR principles.

Therefore, we do not see significantly increased specification effort to also support Type-2 random access in SBFD symbols in addition to the Type-1 procedure.

Proposal 6: For SBFD-aware UEs, support the Type-2 random access procedure in SBFD symbols.

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1: Confirm the RAN1#117 agreement that both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported.

Proposal 2: Under RACH Configuration Option 1 with Alternative 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), SBFD-aware UEs can determine if the RO aligns with SBFD resources or falls within traditional UL resources.

Proposal 3: For RACH Configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), RAN2 needs to specify a new RRC IE for the new SBFD based RACH configuration with a new set of parameters.

Proposal 4: For RACH Configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), the new SBFD based RACH configurations will be delivered through system information (e.g., SIB1), as per legacy RACH configuration.

Proposal 5: The SBFD capable UE may utilize not only the SBFD based RACH, but also the legacy RACH.

Proposal 6: For SBFD-aware UEs, support the Type-2 random access procedure in SBFD symbols.

References

[1] RP-240789 “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”; RAN#103