3GPP TSG-RAN WG2 Meeting #127	R2-2407316

Maastricht, NL, 19 – 23 August 2024

Agenda item:	8.13.2

Source:	Nokia

Title:	Relay discovery and selection for multi-hop relay

WID/SID:	NR_SL_relay_multihop- Release 19

Document for:	Discussion and Decision R2-2407318

# 1	Introduction

In RANP#104, the WID on multi-hop relay is approved in RP-241609. In this contribution, we present our initial thought on the potential/necessary enhancement for relay discovery and reselection to support multi-hop solution.

# 2	Discussion

## 2.1	Relay Discovery

For single-hop (SH) relay operation, transmission of a Relay Discovery (RD) message is determined by RSRP. In detail, the U2N remote UE determines if it can transmit RD messages to U2N relay UE based on a Uu RSRP threshold. U2N relay UE can also perform RD message transmission based on a maximum and/or minimum Uu RSRP thresholds.

For multi-hop (MH) relay operation, as discussed in R2-2407318, the re-use of Rel-17 U2N Relay solution as a baseline simplifies the specification work and deployment of multi-hop relay solution. However, the Rel-17 U2N relay solution might not be optimized for MH relay scenarios. For example, some RRC procedures would not be work well with multi-hop transmission since RRC timer values may not be long enough to cover the latency caused by MH transmission. In addition, the relay UE may not be in coverage of the network, which was not the assumption in Rel-17/18. These limitations might have some impact on RD message transmission, which RAN2 need to discuss further.

Proposal 1: RAN2 discuss the impact on RD message transmission by considering support of Rel-17/18 UEs as remote UE or relay UE for multi-hop operation.

## 2.2	Relay selection

Currently, the U2N relay selection is defined for initial access when the Remote UE is in partial coverage or out of coverage or switching from direct to single-hop (SH) relay, e.g., the remote UE triggers Relay selection procedure if the direct Uu link quality from the remote UE to the gNB is below a threshold. For MH relay operation, the remote UE may be able to reuse the U2N relay selection procedure, i.e., the remote UE triggers the U2N relay selection procedure when the remote UE is out of coverage or the signal quality to the gNB becomes bad.

Proposal 2: RAN2 assumes that the Rel-17/18 U2N relay selection procedure is used as a baseline for multi-hop operation.

Note that service continuity is not to be discussed in this meeting, and thus, relay reselection is not handled in this contribution.

# 3	Conclusion

This contribution has made the following proposals:

Proposal 1: RAN2 discuss the impact on RD message transmission by considering support of Rel-17/18 UEs as remote UE or relay UE for multi-hop operation.

Proposal 2: RAN2 assumes that the Rel-17/18 U2N relay selection procedure is used as a baseline for multi-hop operation.