3GPP TSG-RAN WG2 Meeting #127	R2-2407318

Maastricht, NL, 19 – 23 August 2024

Agenda item:	8.13.3

Source:	Nokia

Title:	Control plane procedure for multi-hop relay

WID/SID:	NR_SL_relay_multihop - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RANP#104, the WID on multi-hop relay is approved in RP-241609. In this contribution, we present our initial thought on the assumptions, based on which we can further discuss what enhancement is required to support multi-hop relay.

# 2	Discussion

The WID on Multi-hop relay [RP-241609] stated to start with one additional hop relay, i.e., 2-hop relay in total, until RAN#107. After checking at RAN#107, we may further consider the scenario with two additional hops, i.e., 3-hop relay.

Figure 1. Multi-hop relay with one additional hop, i.e., 2 hops between the remote UE and the gNB.

Given the intention using the existing framework, it would be good to align the approach:

- Use the Rel-17 U2N Relay solution as a starting point

- Use the Rel-18 U2U Relay solution as a starting point

In the first approach, the first relay UE is acting as a U2N Multi-Hop (MH) relay UE for the remote UE, while simultaneously being a remote UE using the last relay UE as a U2N relay UE. From L2 perspective, it may have an impact on e.g., RRC and/or SRAP, since one UE is acting as a relay UE and a remote UE at the same time. However, the remote and relay UE behavior is already defined in Rel-17 for U2N relay, and can be reused for multi-hop operation. In addition, the UEs beyond the very next hop may be transparent, e.g., the last relay UE is transparent to the remote UE, which would allow flexible extension to N-hop scenario because the additional relays can be added or removed seamlessly from the remote UE perspective.

Figure 2. First approach - Cascaded U2N relay

In the second approach, the remote UE and the last relay UE establish U2U relay connection by using the first relay UE as a U2U relay UE while the last relay UE is directly connected to the gNB. It wouldn’t be a simple combination of U2U relay and direct communication since the last relay UE anyway needs to forward the data received from the first relay UE. Specification impact to U2U relay operation may be significant as multi-hop operation using U2U relay is not yet defined.

Figure 3. The second approach - U2U relay + direct communication

As extending the U2N relay solution to multi-hop U2N operation (especially when there are more than two relays between the remote UE and the gNB) seems less complex, we think that it would be beneficial to use the Rel-17 U2N operation as a baseline for multi-hop U2N relay.

Proposal 1: U2N Relay solution is used as a baseline for the multi-hop relay solution.

If the Rel-17 U2N Relay solution is used as a baseline, then the re-use of the Rel-17 U2N Remote UE and U2N Relay UE functionality is desired as it would simplify both the specification and deployment of multi-hop relay solution. It will also enable wider applicability of UEs to support multi-hop relay. Therefore, we think that RAN2 should attempt to enable the re-use of the Rel-17 U2N Remote and U2N Relay UEs at least for the case when there is only one additional hop.

Proposal 2a: RAN2 working assumption is that the multi-hop relay solution supports Rel-17/18 U2N Remote UEs at least when there is only 1 additional hop.

Proposal 2b: RAN2 working assumption is that the multi-hop relay solution supports Rel-17/18 U2N Relay UEs as Last Relay UE.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: U2N Relay solution is used as a baseline for the multi-hop relay solution.

Proposal 2a: RAN2 working assumption is that the multi-hop relay solution supports Rel-17/18 U2N Remote UEs at least when there is only 1 additional hop.

Proposal 2b: RAN2 working assumption is that the multi-hop relay solution supports Rel-17/18 U2N Relay UEs as Last Relay UE.