3GPP TSG-RAN2 Meeting #127	R2-2407319

Maastricht, Netherlands, Aug 19-23, 2024

Agenda item:	8.2.2

Source:	Qualcomm Incorporated

Title:	Views on Functionality Aspects of Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

Rel-19 Ambient IoT SID [1] describes the objectives including the RAN2 protocol and function studies.

In RAN2 #125bis meeting, the following agreements were made for CP and UP functions separately.

Agreements

- RRC connection management is not supported.  FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)

- RRM L3 measurement reporting is not supported by Ambient IoT devices.

- RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding.

- Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals.

- RAN2 assumes that RRC layer is not necessary between the reader and the device.   RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

Agreement

- SDAP is not supported for UP protocol stack.

- PDCP layer is not needed.  FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities.

- RLC layer is not needed.   FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size).  RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed.

- No HARQ and RLC AM

- FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

- RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL).  FFS how to handle the general QoS requirements from SA2

Furthermore, RAN2#126 agreed the following regarding the function aspects:

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

In this paper, we would like to discuss the functionality aspects of Ambient IoT (AIoT). We will discuss some remaining control plane functionality aspects, followed by user plane functionality aspects and then provide a summary.

# Discussion

The traffic types of DO-DTT and DT with the focus on indoor inventory and command should be considered. The service requirements on inventory procedure and command procedure should be supported from RAN perspective.

In inventory procedure, the AIoT application in core network would like to discover the AIoT devices who are nearby the Readers and sends the inventory request to Readers. The inventory request message may include filter criteria, and other upper layer information.  The Reader that receives the inventory request performs the inventory procedure. The Reader forwards the inventory request to all AIoT devices. The inventory command from Reader could be the group signaling which are sent to all AIoT devices who have harvested enough power and is able to receive.

In RAN2 #125bis meeting, it has been agreed that the AIoT access procedure is triggered by the Reader and Reader may provide the information that the AIoT devices need to respond for the AIoT access trigger. The AIoT devices who are triggered by the inventory request will perform the AIoT access towards the Reader in response to the received inventory request.

Figure 1 Ambient IoT inventory

In command procedure, the AIoT application issues a Command Request including command container, a list of AIoT device identifiers that the command is destined or filter criteria that identify the AIoT devices that are supposed to act upon the command. Therefore, the trigger message sent from Reader may include the device information or filter criteria for triggering AIoT devices to perform AIoT access. The trigger message may also include the command container if AIoT application provides commands to Reader in the command procedure.

## Control plane functionalities

RAN2#125bis discussed about the targeted lower complexity and cost of AIoT devices and agreed that traditional Control Plane functions like RRC connection management, RRM L3 measurement reporting, ASN.1 encoding/decoding, legacy periodical SIB/MIB, legacy paging message, legacy RRC layer, legacy DRX, tracking area update from the device etc. will not be supported by AIoT.

However, RAN2 still has to conclude on some of the remaining CP aspects which will be helpful to resolve the following aspect captured in RAN2#125bis: RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC.

### AIoT Device Registration, Subscription and CN Awareness

Due to the very low target on complexity, cost as well as power consumption, as well as potentially a very large number of AIoT devices may be present in a given vicinity but perform only bursty data transmissions always triggered by the Reader, one of the fundamental architectural differences between AIoT and existing 3GPP technologies may be how the devices are exposed to the Core Network functions.

In our understanding, SA2 would need to discuss and conclude on this. From RAN2 point of view, we think requiring such AIoT devices to support legacy UE-registration-like procedures would be very ambitious not only because of the complexity of the procedure and protocol, but also due to the number of communication steps required. The AIoT devices will not have enough energy source or energy storage to perform multiple steps just for the purpose of registration. Additionally, having a ‘subscription’ similar to legacy UE subscriptions per AIoT device may be a high burden for the core network to manage.

The AIoT device may harvest energy, accumulate it up to the required minimum amount of energy for communication, perform the communication steps and may deplete its energy fully. If the AIoT device runs out of energy after some communication steps, it may not be able to store any configurations, i.e. it will likely ‘forget’ everything. In other words, it is plausible that every communication session starts as if the device has been just powered up from the power off state. Then, it would not make sense to have the AIoT device perform the legacy registration procedures every time it needs to communicate.

It is plausible that for AIoT devices, each communication session may start as if the device has just been powered up from the power off state.

Legacy UE-registration-like procedure is not supported by AIoT. In other words, a specific tag is not required to maintain registration with the CN.

### NAS functions

NAS control protocol in NR performs the functions listed in TS 23.501 such as registration, authentication, mobility management, security control etc. Supporting all these legacy NAS functionalities by the AIoT devices may be infeasible especially given their complexity, cost, processing power and energy/power constraints.

In our view, the functionalities like security, authentication for AIoT communication should be provided by AIoT Application function using a separate command protocol. We understand such details are being discussed in SA2 and SA3.

Legacy NAS or NAS-like functions are not supported by AIoT devices. Consequently, NAS-based data transfer (CP-CIoT-like) is not supported.

RAN2 assumes new AIoT upper layer protocol (e.g. Command Protocol) between AIoT device and AIoT Application Function will be introduced by SA2/3.

## User plane functionalities

According to the AIoT UP agreements made in the previous RAN2 meetings, the legacy NR SDAP/PDCP/RLC protocol layers are not supported for AIoT UP protocol. Some functionalities in these protocols may need further discussion that may need other WG’s further inputs as well. The similar function could be implemented in other UP protocol layer or a new L2 layer.

For the AIoT MAC, not all legacy NR MAC functionalities are needed for AIoT.

### Logical and transport channels

RAN2#126 agreed the following.

Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

There is still FFS on the AIoT logical channel concept. The NR logical channels are used to carry the control signaling and UP data with different QoS requirements. Similar to NR UP data, the AIoT data can be carried in MAC PDU. In AIoT MAC, the upper layer control signaling and AIoT data can be mapped into one single logical channel and all MAC PDUs belonging to this one logical channel instruct the transport blocks to be delivered to the physical layer. Alternatively, the upper layer control signaling and AIoT data can be mapped to two separate logical channels. AIoT MAC can multiplex the MAC PDU from these two logical channels into one transport block.

The concept of logical channels as in NR MAC is supported for AIoT.

Regarding the transport channels, since RAN1 has already agreed to support PRDCH and PDRCH, from AIoT MAC perspective, at least two transport channels mapping to physical channels, i.e., PRDCH and PDRCH should be supported. Similar to NR DL-SCH and UL-SCH, AIoT should support transport channels RD-SCH and DR-SCH.

At least two transport channels, i.e., RD-SCH and DR-SCH are supported for AIoT.

### BSR/PHR

The last RAN2 meeting has agreed that the legacy SR/BSR is not supported for AIoT device. However, FFS is left on whether further indication of device message size/status is needed.

It may be beneficial if AIoT device can indicate assistance information or the buffer/energy status to the Reader. For example, AIoT device may indicate the buffer status to Reader if there are remaining AIoT data stored in the AIoT buffer, or AIoT device may inform Reader the energy status if AIoT device lacks enough power/energy.

In some cases, the full power of some types of AIoT device such as Device type ii may not be always needed if the AIoT device is in close proximity to the Reader. Using lower power for transmission can save the energy to extend the on duration of transmission/reception and reduce the interference to other nearby devices.

Therefore, the functionalities of some of the existing NR MAC features are useful also for AIoT. RAN2 can further study simpler mechanisms but with similar functionalities without using legacy NR MAC CEs, e.g. more-data indication, or low-energy-level indication in D2R communication.

Some functionalities of existing NR MAC CEs are useful for AIoT.

RAN2 to study the simplifications needed to support the similar functionalities as BSR and PHR without using legacy NR BSR/PHR MAC CE, e.g. more-data indication or low-energy-level indication in D2R communication.

### Multi-round AIoT access

Since AIoT device is expected to be ultra-low complexity device with ultra-low power consumption for the very-low end IoT applications, some very low-end AIoT devices, e.g., device type i, may requires more rounds of energy harvesting to accumulate enough energy for inventory response.

In some cases, the AIoT devices may miss the inventory command due to inability of accurate sync signal tracking. In other cases, the congestion may be heavy in the massive IoT access procedure. In that sense, the multiple rounds of AIoT access should be supported by means of the high layer signaling repetition. It will be helpful if Reader could periodically transmit inventory command to discover the nearby AIoT devices.

Noted that in last RAN2 meeting, it has been agreed that reader and device may send the payload again as new transmission. Them multiple rounds of AIoT access could be achieved by the trigger message repetition.

RAN2 to study the multiple rounds of AIoT access procedure, e.g., the periodical transmission of inventory, by Reader.

# Overall protocol stack

Based on the above discussion on the needed functionalities, in our view the overall AS protocol stack may look something like below.

Figure 3 AIoT protocol stack

As discussed above, the NR legacy UP protocols, i.e., SDAP/PDCP/RLC are not needed. NAS protocol would be replaced by simplified command protocol terminating at the AIoT Application Function. Note that the exact protocol layers in the Reader facing the AF side is not shown since that is unchanged and depends on whether Reader is a separate entity or a part of BS or UE based on topology which could be discussed in RAN3.

In our view, a new AIoT layer 2 protocol is a simple and clear design for AIoT device and Reader. The new AIoT L2 protocol sits between AIoT MAC and the application layer/command protocol.

A new simplified AIoT L2 protocol will be introduced.

Simplified AIoT protocol stack is unified for AIoT CP and UP (i.e. no differentiation between CP and UP protocol stacks).

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1.	It is plausible that for AIoT devices, each communication session may start as if the device has just been powered up from the power off state.

Observation 2.	Some functionalities of existing NR MAC CEs are useful for AIoT.

Proposal 1:	Legacy UE-registration-like procedure is not supported by AIoT. In other words, a specific tag is not required to maintain registration with the CN.

Proposal 2:	Legacy NAS or NAS-like functions are not supported by AIoT devices. Consequently, NAS-based data transfer (CP-CIoT-like) is not supported.

Proposal 3:	RAN2 assumes new AIoT upper layer protocol (e.g. Command Protocol) between AIoT device and AIoT Application Function will be introduced by SA2/3.

Proposal 4:	The concept of logical channels as in NR MAC is supported for AIoT.

Proposal 5:	At least two transport channels, i.e., RD-SCH and DR-SCH are supported for AIoT.

Proposal 6:	RAN2 to study the simplifications needed to support the similar functionalities as BSR and PHR without using legacy NR BSR/PHR MAC CE, e.g. more-data indication or low-energy-level indication in D2R communication.

Proposal 7:	RAN2 to study the multiple rounds of AIoT access procedure, e.g., the periodical transmission of inventory, by Reader.

Proposal 8:	A new simplified AIoT L2 protocol will be introduced.

Proposal 9:	Simplified AIoT protocol stack is unified for AIoT CP and UP (i.e. no differentiation between CP and UP protocol stacks).

# References

- RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA