3GPP TSG-RAN WG2 Meeting #127	R2-2407320

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.2

Source: 	Fujitsu

Title:	Discussion on subsequent inter-CU or inter-CU LTM

Document for:	Discussion and Decision

- Introduction

This document discusses basic directions for subsequent inter-CU or inter-CU LTM. Both non-DC case and DC case are considered.

- Discussions

In the WID for NR mobility enhancement Phase 4 [1], the followings are raised as objective for inter-CU LTM.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

This document discusses basic directions for subsequent inter-CU or inter-CU LTM, for both non-DC and DC cases.

2.1	Non-DC case

CU is acting as MN when DC is not configured

In RAN2#126, it was agreed:

The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

In Rel-18 intra-CU LTM, in order to support subsequent LTM cell switch execution, the target configuration is a complete configuration, and the current UE configuration is replaced with the target complete configuration at LTM cell switch execution. In order to generate the target complete configuration, the candidate configurations are either complete configuration or delta configuration with reference configuration.

For Rel-19 intra-CU or inter-CU LTM, subsequent LTM cell switch execution is also one of the objectives. Therefore, the target configuration is required to be a complete configuration. To generate the complete configuration for Rel-19, we propose to apply the same mechanism with Rel-18, i.e., the candidate configurations are either complete configuration or delta configuration with reference configuration, since RAN2 already agreed that the RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline.

Proposal 1: Both complete configuration and delta configuration with reference configuration should be supported for inter-CU LTM candidate configuration as well as intra-CU LTM candidate.

The reference configuration is common for all candidate configurations in Rel-18 intra-CU LTM. For Rel-19 inter-CU or intra-CU LTM, RAN2 needs to discuss whether the reference configuration can be common or not. If the common reference configuration is provided for all candidates belonging to different CUs, the CUs will be required to coordinate with each other to create the common reference configuration. Such coordination will be difficult especially for inter-vender CUs. We propose that more than one reference configurations should be supported for Rel-19 intra-CU or inter-CU LTM.

Proposal 2-a: For Rel-19 intra-CU or inter-CU LTM, more than one reference configurations should be supported.

If RAN2 can agree that more than one reference configurations will be supported for Rel-19 intra-CU or inter-CU LTM, a new IE to cover multiple reference configurations will be necessary for Rel-19. If reference configuration is configured, the network should configure either the Rel-18 reference configuration which covers only one reference configuration or the Rel-19 reference configuration which covers multiple reference configurations depending on how many reference configuration(s) is/are used.

Proposal 2-b: If Proposal 2-a is agreed, a new IE to cover multiple reference configurations should be defined for Rel-19. If reference configuration is configured, the network should configure the Rel-18 reference configuration or the Rel-19 reference configuration depending on how many reference configuration(s) is/are used.

  - DC case

DC case common

In Rel-18 intra-CU LTM, as security key change is not supported, PDCP re-establishment is not supported. Therefore, bearer type change between MN terminated bearer and SN terminated bearer is not implicitly supported as well. In Rel-19 intra-CU or inter-CU LTM, as security key change will be supported, bearer type change between MN terminated bearer and SN terminated bearer can be supported. However, if bearer type change between MN terminated bearer and SN terminated bearer is supported, specification impact will not be small. We propose RAN2 should discuss whether bearer type change between MN terminated bearer and SN terminated bearer will be supported or not.

Proposal 3: RAN2 to discuss whether bearer type change between MN terminated bearer and SN terminated bearer will be supported or not for DC case.

CU is acting as SN and MN is unchanged

In Rel-18, SCG LTM as well as MCG LTM have been standardized. Also, subsequent CPAC (SCPAC) has also been standardized. While Rel-18 LTM is only for intra-CU case (for both MCG LTM and SCG LTM) and does not support security key change, SCPAC supports inter-CU or inter-CU case and therefore support security key change. For Rel-19 inter-CU or inter-CU SCG LTM, we think Rel-18 SCG LTM with Rel-18 SCPAC security handling should be a baseline.

Proposal 4: For the case CU is acting as SN and MN is unchanged, Rel-18 SCG LTM with Rel-18 SCPAC security handling should be a baseline.

CU is acting as MN and SN is unchanged or SN is released

RAN2 is currently waiting for SA3 decision on security handling for inter-CU LTM. For the case CU is acting as MN and SN is unchanged or SN is released, we think RAN2 should start discussion after non-DC case details become stable.

Proposal 5: The case CU is acting as MN and SN is unchanged or SN is released should be discussed after non-DC case details become stable.

- Conclusion

CU is acting as MN when DC is not configured

Proposal 1: Both complete configuration and delta configuration with reference configuration should be supported for inter-CU LTM candidate configuration as well as intra-CU LTM candidate.

Proposal 2-a: For Rel-19 intra-CU or inter-CU LTM, more than one reference configurations should be supported.

Proposal 2-b: If Proposal 2-a is agreed, a new IE to cover multiple reference configurations should be defined for Rel-19. If reference configuration is configured, the network should configure the Rel-18 reference configuration or the Rel-19 reference configuration depending on how many reference configuration(s) is/are used.

DC case common

Proposal 3: RAN2 to discuss whether bearer type change between MN terminated bearer and SN terminated bearer will be supported or not for DC case.

CU is acting as SN and MN is unchanged

Proposal 4: For the case CU is acting as SN and MN is unchanged, Rel-18 SCG LTM with Rel-18 SCPAC security handling should be a baseline.

CU is acting as MN and SN is unchanged or SN is released

Proposal 5: The case CU is acting as MN and SN is unchanged or SN is released should be discussed after non-DC case details become stable.

- References

[1] RP-241515, “Revised Work Item: NR mobility enhancements Phase 4”