3GPP TSG-RAN WG2 Meeting #127	R2-2407321

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:        ZTE Corporation, Sanechips

Title:           Further Clarification on the Reconfiguration Failure Processing

Agenda item:   7.17.3

Document for:  Discussion and Decision

- Introduction

In the current spec, a Note was added to the Reconfiguration failure case for the MUSIM operation.

##### 5.3.5.8.2 Inability to comply with RRCReconfiguration /*********************omit the unrelated part*******************************************/ NOTE 1: The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in clause 10 specifies that the UE shall ignore the message. NOTE 2: If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure, except for the case that the UE is configured to provide MUSIM assistance information for temporary capability restriction. For such case, the UE does not apply above failure handling in case the UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation, and it is up to UE implementation how to apply RRCReconfiguration message. If UE does not perform RRC reconfiguration failure in this case, UE will provide MUSIM assistance information for temporary capability restriction as specified in 5.7.4 and still considers the configuration resulting from RRCReconfiguration message as the current configuration and baseline for delta configuration for future reconfigurations.

However, during offline discussion in the last meeting, there is still a potential issue with the current description. In this paper, we give detail description on this issue and the related TP was also provided.

- Discussion

In this chapter, we first points the problem of the current description in the NOTE2 for the partial failure case, and then give the TP for it.

According to the current NOTE 2, for the case that the UE is configured to provide MUSIM assistance information for temporary capability restriction, there would be some exceptional processing, e.g.do not apply the failure handling.

NOTE 2: If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure, except for the case that the UE is configured to provide MUSIM assistance information for temporary capability restriction.

However, for this case, there are still 2 sub-cases:

- Subcase 1: The UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation;

- Subcase 2: The UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message for the reasons other than the UE temporary capability restriction for MUSIM operation, e.g. the parameters other than MIMO layers/bandwidth/CC numbers.

Obviously, the subcase 1 is aligned with the RAN2’s intention for the MUSIM temporary capability feature, but the subcase 2 is not. For the subcase 2, even the UE reports the UAI with the temporary capability restriction to the network, the network still doesn’t know that the UE can’t comply part of the parameters (the parameters that are unable to be reported by the UE temporary capability UAI). Thus, for the subcase 2, the legacy mechanism should be adopted.

Proposal 1: For the below 2 subcases, only for the subcase 1, the UE could take the exceptional processing, e.g.do not apply the failure handling.

- Subcase 1: The UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation

- Subcase 2: The UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message for the reasons other than the UE temporary capability restriction for MUSIM operation, e.g. the parameters other than MIMO layers/bandwidth/CC numbers.

If the proposal 1 was agreed, a TP was also provided as in the Annex to include only the subcase 1.

Proposal 2: Agree the TP in the Annex.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: For the below 2 subcases, only for the subcase 1, the UE could take the exceptional processing, e.g.do not apply the failure handling.

- Subcase 1: The UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation

- Subcase 2: The UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message for the reasons other than the UE temporary capability restriction for MUSIM operation, e.g. the parameters other than MIMO layers/bandwidth/CC numbers.

Proposal 2: Agree the TP in the Annex.

- Annex

5.3.5.8.2	Inability to comply with RRCReconfiguration

/*********************omit the unrelated part*******************************************/

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in clause 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure, except for the case that the UE is configured to provide MUSIM assistance information for temporary capability restriction and the UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation. For such case, the UE does not apply above failure handling in case the UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation, and it is up to UE implementation how to apply RRCReconfiguration message. If UE does not perform RRC reconfiguration failure in this case, UE will provide MUSIM assistance information for temporary capability restriction as specified in 5.7.4 and still considers the configuration resulting from RRCReconfiguration message as the current configuration and baseline for delta configuration for future reconfigurations.