3GPP TSG-RAN WG2 Meeting #127	R2-2407327

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:         ZTE Corporation, Sanechips

Title:            Consideration on Redundant Prerequisite of UE Features

Agenda item:    7.0.1

Document for:   Discussion and Decision

- Introduction

According to the Ran 1 feature list table [1], there are more than 1 prerequisites for some features. However some prerequisites are redundant, in this paper, we’d like to share our observations on this issue.

- Discussion

In this chapter, we’d like to give some examples on the redundant prerequisites first, and then to discuss whether and how to avoid such kinds of issues.

As shown in the below tables, there are two different cases for the redundant prerequisites:

- Case 1: A feature with prerequisites A or B, but the A is the prerequisite of the B.

Table 1: UE Features of the Case 1

| Index | Feature Group | Prerequisite | IE in the 38331 |
| --- | --- | --- | --- |
| 40-3-1-5 | Basic feature for Rel-17-based CJT type-II codebook | 2-35 | feType2CJT-r18 |
| 40-3-1-5a | Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset | 40-3-1-5 | feType2CJT-FD-IO-r18 |
| 40-3-1-7 | Support of M=2 and R=1 for Rel-17-based CJT codebook | 40-3-1-5 or 40-3-1-5a | feType2CJT-M2R1-r18 |
| 40-3-1-8 | Support R=2 for Rel-17-based CJT codebook | 40-3-1-5 or 40-3-1-5a | feType2CJT-R2-r18 |

- Case 2: A feature with prerequisites A and B, but the A is the prerequisite of the B.

Table 2: UE Features of the Case 2

| Index | Feature Group | Prerequisite | IE in the 38331 |
| --- | --- | --- | --- |
| 40-4-1 | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for scheduling of mapping type A | 2-5 | pdsch-TypeA-DMRS-r18 |
| 40-4-1j | Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for scheduling of mapping type A | 40-4-1 | mappingTypeA-1SymbolFL-DMRS-Addition2Symbol-r18 |
| 40-4-1b | 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1, 40-4-1j | pdsch-1SymbolFL-DMRS-Addition2Symbol-r18 |

Observation 1: There are two cases with the redundant prerequisites:

- Case 1: A feature with prerequisites A or B, but the A is the prerequisite of the B.

- Case 2: A feature with prerequisites A and B, but the A is the prerequisite of the B.

For the case 1, take the feature 40-3-1-7 (feType2CJT-M2R1-r18) in the Table 1 as an example, it has two prerequisites “40-3-1-5 or 40-3-1-5a”. However, the feature 40-3-1-5 is also the prerequisite of the 40-3-1-5a, which means that if the UE supports 40-3-1-5a, it must support 40-3-1-5. Based on this logic, these two prerequisites “40-3-1-5 or 40-3-1-5a” has the same effect as the only one prerequisite, i.e. 40-3-1-5.

Observation 2: For the case 1, if a feature has prerequisites A or B, but the A is the prerequisite of the B, the prerequisite B is redundant.

For the case 2, take the feature 40-4-1b (pdsch-1SymbolFL-DMRS-Addition2Symbol-r18) in the Table 2 as an example, it has two prerequisites “40-4-1 or 40-4-1j”. However, the feature 40-4-1 is also the prerequisite of the 40-4-1j, which means that if the UE supports 40-4-1j, it must support 40-4-1. Based on this logic, these two prerequisites “40-4-1 or 40-4-1j” has the same effect as the only one prerequisite, i.e. 40-4-1j.

Observation 3: For the case 2, if a feature has prerequisites A and B, but the A is the prerequisite of the B, the prerequisite A is redundant.

If we go through the RAN1 feature list carefully, we can also find some other similar examples. Furthermore, if we go back to the Rel15, these cases also exist, take the table 3 as an example:

Table 3: R15 UE Features of the Case 1

| Index | Feature Group | Prerequisite |
| --- | --- | --- |
| 1-7 | CSI-RS based RLM | 1-1, CSI-RS |
| 1-8 | RLM based on a mix of SS block and CSI-RS signals within active BWP | 1-3 and 1-7 |
| 1-14 | Maximal number of CSI-RS resources within a slot per PCell/PSCell for CSI-RS based RLM | 1-7 or 1-8 |

In which, the feature 1-14 has prerequisite “1-7 or 1-8”. However, the feature 1-7 is also the prerequisite of the 1-8, thus it can be simplified to be 1-7.

Observation 4: The case (at lease case 1) with redundant prerequisite even exists from the Rel-15.

Based on the above observations, RAN2 can discuss whether and how to avoid such kinds of issues, generally, there are 3 options:

- Option 1: Take no action on it, for that it only has impacts on the field description part of the UE capability spec;

- Option 2: Avoid the redundant prerequisite by RAN2 UE capability review for the Rel-18, and send LS to RAN1/4 to avoid such kinds of the redundant prerequisites in the feature list table from Rel-19;

- Option 3: Send LS to RAN1/4 to check and update the Feature list table from the Rel-18;

- Option 4: Keep the R18 as it is, but send LS to RAN1/4 to avoid such kinds of the redundant prerequisites in the feature list table from Rel-19.

Among these 4 options, option 1 is quite negative, for that in the UE capability spec implementation, RAN2 would include all of the prerequisites listed in the RAN1/4 feature list table, which leads to the redundant prerequisite in the UE capability field description. The option 2 and 3 may introduce additional work in RAN1 or RAN2, which is unsuitable at this stage and thus the option 4 is slightly preferred.

Proposal 1: Based on the above 4 observations, RAN2 can keep the Rel-18 as it is, but send LS to RAN1/4 to ask RAN1/4 to avoid such kinds of the redundant prerequisites in the feature list table from Rel-19.

A draft LS was also provided in the [2].

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: There are two cases with the redundant prerequisites:

- Case 1: A feature with prerequisites A or B, but the A is the prerequisite of the B.

- Case 2: A feature with prerequisites A and B, but the A is the prerequisite of the B.

Observation 2: For the case 1, if a feature has prerequisites A or B, but the A is the prerequisite of the B, the prerequisite B is redundant.

Observation 3: For the case 2, if a feature has prerequisites A and B, but the A is the prerequisite of the B, the prerequisite A is redundant.

Observation 4: The case (at lease case 1) with redundant prerequisite even exists from the Rel-15.

Proposal 1: Based on the above 4 observations, RAN2 can keep the Rel-18 as it is, but send LS to RAN1/4 to ask RAN1/4 to avoid such kinds of the redundant prerequisites in the feature list table from Rel-19.

- Reference

[1] R2-2405564 Updated RAN1 UE features list for Rel-18 NR after RAN1#117.

[2] R2-2407328 Draft LS on the Redundant Prerequisite ZTE Corporation, Sanechips