3GPP TSG RAN WG2#127	R2-2407328

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Title:	Draft LS on the Redundant Prerequisite

Response to:

Release:	Rel-18

Work Item:	         NR_MIMO_evo_DL_UL-Core

Source:	TSG RAN WG2

To:	TSG RAN WG1, TSG RAN WG4

CC:

Contact person:	Wenting Li (li.wenting@zte.com.cn)

Attachments:	None

1	Overall description

During RAN2 discussion,  for the prerequisites of the UE feature, there are two cases with the redundant prerequisites:

- Case 1: A feature with prerequisites A or B, but the A is the prerequisite of the B.

Table 1: UE Features of the Case 1

| Index | Feature Group | Prerequisite | IE in the 38331 |
| --- | --- | --- | --- |
| 40-3-1-5 | Basic feature for Rel-17-based CJT type-II codebook | 2-35 | feType2CJT-r18 |
| 40-3-1-5a | Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset | 40-3-1-5 | feType2CJT-FD-IO-r18 |
| 40-3-1-7 | Support of M=2 and R=1 for Rel-17-based CJT codebook | 40-3-1-5 or 40-3-1-5a | feType2CJT-M2R1-r18 |
| 40-3-1-8 | Support R=2 for Rel-17-based CJT codebook | 40-3-1-5 or 40-3-1-5a | feType2CJT-R2-r18 |

- Case 2: A feature with prerequisites A and B, but the A is the prerequisite of the B.

Table 2: UE Features of the Case 2

| Index | Feature Group | Prerequisite | IE in the 38331 |
| --- | --- | --- | --- |
| 40-4-1 | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for scheduling of mapping type A | 2-5 | pdsch-TypeA-DMRS-r18 |
| 40-4-1j | Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for scheduling of mapping type A | 40-4-1 | mappingTypeA-1SymbolFL-DMRS-Addition2Symbol-r18 |
| 40-4-1b | 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1, 40-4-1j | pdsch-1SymbolFL-DMRS-Addition2Symbol-r18 |

RAN2 observes that for the case 1, if a feature has prerequisites A or B, but the A is the prerequisite of the B, the prerequisite B is redundant, while for the case 2, if a feature has prerequisites A and B, but the A is the prerequisite of the B, the prerequisite A is redundant. In the UE capability spec implementation, RAN2 would include all of the prerequisites listed in the RAN1/4 feature list table. To solve this kind of redundant issue, RAN2 would like to ask ran1/4 to avoid such kinds of the redundant prerequisites in the feature list table from Rel-19.

2	Actions

To RAN4

ACTION: 	RAN2 kindly requests RAN4 to take the above information into account.

3	Dates of next TSG WG2 meetings

TSG-RAN WG2 Meeting #127bis	14 Oct - 18 Oct 2024	Hefei, CN

TSG-RAN WG2 Meeting #128	18 Nov - 22 Nov 2024	Orlando , US