3GPP TSG-RAN WG2 Meeting #127	R2-2407329

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:         ZTE Corporation, Sanechips

Title:            Consideration on the intra-band EN-DC Channel Spacing

Agenda item:    7.24.1

Document for:   Discussion and Decision

- Introduction

About the capability for the intra-band EN-DC Channel Spacing, a LS [1] was received on the intra-band EN-DC channel spacing.

With the UE capability, it is noticed that in the field, some UEs indicating ‘non-contiguous’ via intraBandENDC-Support cannot access to the NW configuring intra-band contiguous EN-DC. The UE can only access to single carrier rather than intra-band EN-DC, which degrades the UE performance. Considering the capability of intraBandENDC-Support has been implemented for several releases, to make up for the problem, RAN4 agrees to introduce a new capability to support intra-band non-contiguous with nominal channel spacing. In RAN4 spec, nominal channel spacing between E-UTRA carrier and an adjacent NR carrier is defined for intra-band contiguous EN-DC in TS 38.101-3.

In this paper, we’d like to share our views on this LS.

- Discussion

In this chapter, we first share some observations on the problem described in this LS, and then give the proposals on this issue.

From the LS, it mentioned that until now the UE can only support two cases (i.e. contiguous and both), but the RAN2 defined 3 cases.

In Rel-15, RAN4 sent the LS R4-1813862 to RAN2 for intra-band MR-DC band combination signaling, with the assumptions that if a UE supports intra-band non-contiguous EN-DC, it also supports contiguous in the same band. And then the capability of intraBandENDC-Support was introduced in TS 38.306 as follows, with three states, including: contiguous (not include), non-contiguous and ‘both’. However, from RAN4’s perspective, UE implementing ‘non-contiguous’ needs more stringent capability. In this case, if a UE supports ‘non-contiguous’, it should not preclude the support of ‘contiguous’ in the same band. The capability UE reports should reflect the implementation UE can support, rather than just for configurations.

However, from the RAN2 aspect, when design the ASN.1 signalling, the forward compatibility shall also be considered. Take the intraBandENDC-Support as an example, the non-contiguous only case can be included for the forward compatibility. Furthermore, even “non-contiguous only” case would not be implemented in the future, it would not affect the UE to report the other 2 cases, i.e. contiguous and both.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraBandENDC-Support Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC only in DL, this field indicates the DL capability. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC in DL and UL, this field indicates the common capability for both DL and UL. If intrabandENDC-Support-UL is included, intraBandENDC-Support indicates the DL capability. | BC | No | N/A | N/A |

Observation 1: From forward compatibility aspect, RAN2 signalling can include more cases than the current UE supported.

From the LS, we also observed that the problem happens when the UE supports both contiguous and non-contiguous but reports non-contiguous only.

With the UE capability, it is noticed that in the field, some UEs indicating ‘non-contiguous’ via intraBandENDC-Support cannot access to the NW configuring intra-band contiguous EN-DC. The UE can only access to single carrier rather than intra-band EN-DC, which degrades the UE performance.

Then the problem is why the UE doesn’t report the “both” as it has supported for this case. It has been clearly clarified in the filed description that the non-contiguous is used for the case that the UE supports non-contiguous only. Obviously, if the UE reports the capability correctly according to the RAN2 spec, there would be no problem.

Observation 2: The problem happens when the UE supports both contiguous and non-contiguous but reports non-contiguous only. If the UE reports the capability correctly according to the RAN2 spec, there would be no problem.

In the LS, it further suggests to introduce new capability, however, if a new capability was introduced, it means updating would be needed at both the UE and network side.

Considering the capability of intraBandENDC-Support has been implemented for several releases, to make up for the problem, RAN4 agrees to introduce a new capability to support intra-band non-contiguous with nominal channel spacing. In RAN4 spec, nominal channel spacing between E-UTRA carrier and an adjacent NR carrier is defined for intra-band contiguous EN-DC in TS 38.101-3.

Observation 3: The new capability mechanism requires updating at both the UE and network side, which means it can’t work when the new UE enters into the old gNB.

Furthermore, before introducing the new capability, at least the issues as below need to be further discussed and clarified:

- Issue 1: Which requirements (contiguous or non-contiguous) shall be used for the case that the UE reports “intraBandENDC-Support: non-contiguous” and the new capability?

- Issue 2: Are there any different between the below 2 cases:

- Case 1: UE reports “intraBandENDC-Support: non-contiguous” and the new capability

- Case 2 UE reports “intraBandENDC-Support: both”.

- Issue 3: There is also a corresponding capability for the uplink (i.e. intrabandENDC-Support-UL), whether to extend this new capability to the Uplink?

- Issue 4: Whether extend the new capability for the multiple intra-bands case as in [2][3][4][5]?

Observation 4: Before introducing the new capability, at least the issues as below need to be further discussed and clarified:

- Issue 1: Which requirements (contiguous or non-contiguous) shall be used for the case that the UE reports “intraBandENDC-Support: non-contiguous” and the new capability?

- Issue 2: Are there any different between the below 2 cases:

- Case 1: UE reports “intraBandENDC-Support: non-contiguous” and the new capability

- Case 2 UE reports “intraBandENDC-Support: both”.

- Issue 3: There is also a corresponding capability for the uplink (i.e. intrabandENDC-Support-UL), whether to extend this new capability to the Uplink?

- Issue 4: Whether to extend the new capability for the multiple intra-bands case [2][3][4][5]?

Based on the above observations, we give the below proposal.

Proposal 1: Ran2 would not introduce the new capability for the case that the UE supports both contiguous and non-contiguous. According to the RAN2 spec, the UE should report the “both” for this case with the existing signalling.

Ran2 can send the reply LS to Ran4 with the above observations and the proposal. The draft LS was also provided in the [6].

Proposal 2: Include the above observations and the proposal in the reply LS [6].

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: From forward compatibility aspect, RAN2 signalling can include more cases than the current UE supported.

Observation 2: The problem happens when the UE supports both contiguous and non-contiguous but reports non-contiguous only. If the UE reports the capability correctly according to the RAN2 spec, there would be no problem.

Observation 3: The new capability mechanism requires updating at both the UE and network side, which means it can’t work when the new UE enters into the old gNB.

Observation 4: Before introducing the new capability, at least the issues as below need to be further discussed and clarified:

- Issue 1: Which requirements (contiguous or non-contiguous) shall be used for the case that the UE reports “intraBandENDC-Support: non-contiguous” and the new capability?

- Issue 2: Are there any different between the below 2 cases:

- Case 1: UE reports “intraBandENDC-Support: non-contiguous” and the new capability

- Case 2 UE reports “intraBandENDC-Support: both”.

- Issue 3: There is also a corresponding capability for the uplink (i.e. intrabandENDC-Support-UL), whether to extend this new capability to the Uplink?

- Issue 4: Whether to extend the new capability for the multiple intra-bands case [2][3][4][5]?

Proposal 1: Ran2 would not introduce the new capability for the case that the UE supports both contiguous and non-contiguous. According to the RAN2 spec, the UE should report the “both” for this case with the existing signalling.

Proposal 2: Include the above observations and the proposal in the reply LS [6].

- Reference

- R4-2410693    LS on intra-band EN-DC channel spacing  To RAN2

- R2-2405975	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-17	38.331	17.8.0	4750	4	F	NR_newRAT-Core, TEI17

- R2-2405976	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-17	38.306	17.8.0	1084	4	F	NR_newRAT-Core, TEI17

- R2-2405977	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-18	38.331	18.1.0	4751	4	A	NR_newRAT-Core, TEI17

- R2-2405978	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-18	38.306	18.1.0	1085	4	A	NR_newRAT-Core, TEI17

- R2-2407330	Draft Reply LS on the intra-band EN-DC Channel Spacing	 ZTE Corporation, Sanechips