3GPP TSG-RAN WG2 Meeting #127							R2-2407331

Maastricht, Netherlands, 19 - 23 August, 2024

Title: 	Discussion on NW-sided data collection for training

Source: 	Huawei, HiSilicon

Agenda item:	8.1.3

Document for:	Discussion/Decision

- Introduction

In the last meeting, the following agreements are achieved for NW-sided data collection [1]:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

There are still some FFSs that need to be analyzed. In this paper, we will analyze the network-sided data collection and the FFSes.

- Discussion

Discussion on details of periodical reporting

In the last meeting, periodical reporting was agreed to be introduced but the detailed solution was not discussed. In this section, we will analyze the periodical reporting.

We need to consider what should be configured for periodical reporting of data collection. First, what should be collected needs to be specified. Due to the content of collected data being discussed in RAN, this part should be left to RAN1 and it should be discussed based on use cases.

Proposal 1: The contents of collected data should be specified by RAN1, and then RAN2 can follow RAN1's conclusions. The data collection requirements are discussed and decided based on use cases.

Secondly, the logging configuration also needs to be specified. We should determine when the UE should start logging data and how long the UE should log. In our opinion, the following reporting parameters can be considered:

(1) L1 measurement result (e.g. data sample) collection period. This parameter indicates the interval between L1 measurement results

(2) Reporting interval. This parameter indicates the interval between periodical reports, and the current IE ReportInterval is an example (defined in TS 38.331)

If the UE is configured with (1) and (2), the UE continuously collects L1 measurement results, and the UE reports the logged L1 measurement results to network for each reporting interval. The L1 measurement result collection period should be smaller than the reporting interval.

Proposal 2: The L1 measurement result collection period and reporting interval are configured by the network.

Based on the above analysis, the general procedure for periodical reporting is as shown in Fig.1:

Fig.1 The general procedure of training data collection for periodical report

Step 1: The network sends the data collection configuration to the UE via RRC signaling, which includes the content of data to be collected, and the reporting configuration.

Step 2: UE starts to collect the data based on the configurations.

Step 3&4: When the reporting period passes, the UE sends the report to the network.

Discussion on event-based reporting

For event-based reporting, the content of the collected data is similar to the periodical reporting and it depends on RAN1 conclusion. Similarly as for periodical reporting, the UE may need to log some L1 measurement results, and then the logging configuration also needs to be considered.

What is most important for the event-based reporting is the definition of an event. In our opinion, the data volume of stored L1 measurement results reaching a certain threshold can be considered as such event. In this approach, the network would configure the data volume threshold to the UE, e.g., 500 bits. When the UE has collected 500 bits of data, the triggering condition of the event is met and then the UE immediately sends a report to the network.

However, we think that compared with the periodical reporting, the benefit of the data volume-based event is unclear. The end effect is very similar to the periodical reporting, but it has some drawbacks. Firstly, if the data volume threshold is inappropriate, it may lead to frequent UL RRC messages, and secondly it puts some complexities to UE side. So we think the necessity of data volume-based event should be discussed in RAN2.

Proposal 3: It is proposed RAN2 to discuss the necessity of data volume-based event.

Discussion on network request based reporting

Another option for data reporting which was considered was reporting upon explicit network request. For this option, the logging configuration and content of the collected data are configured by the network in advance, i.e., these configurations have been sent before the network requests the collected data. The content of this configuration are similar to the other two reporting types.

In this approach, when the data collection is finished, the UE firstly sends an indicator to the network, which shows that the L1 measurement results are available. The trigger for the UE to send an indication may be related to the network configuration, e.g. it may be about the data volume exceeding some threshold, or it can be left to UE implementation.

Once the network receives the indicator from the UE, the network can initiate another RRC procedure to request logged L1 measurement results from the UE.

Compared with the periodical reporting, the benefit of the network request based reporting is unclear. At the same time, this option consumes more RRC signaling and introduces higher latency. In addition, it also puts some complexities to UE side. So we think the necessity of network request based reporting should be discussed in RAN2.

Proposal 4: It is proposed RAN2 to discuss the necessity of network request reporting.

Discussion on RRC message capacity

Issue analysis and solutions

This section is to address the FFS "FFS how to handle the case when single RRC message is not sufficient".

According to [2], the maximum supported size of a PDCP SDU is 9000 bytes, and then one RRC message is at most 9000 bytes (72 kbits).

If the network can provide suitable reporting configurations to UE, the logged L1 measurement results should not exceed the upper limit of one RRC message. However, if the reporting configurations are not suitable, the UE may log lots of logged L1 measurement results, and then it may bring transmission issue of the RRC message.

According to the RAN1 LS [3], if we take Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits) if all beams in Set A were to be collected. Assume that 1000 data samples are included in one RRC measurement report, the total size of the data is up to 500 kbits (500 bits * 1000), which is more than the upper limit of one RRC message (i.e. 72 kbits).

In order to solve the issue, there may be two ways: the UE can do segmentation on the RRC message, or the UE can stop logging any new L1 measurement results. It is noted that even for the case where RRC segmentation is not enabled for the logged measurement collection, the UE may still need to stop logging if the stored measurements exceed the limit.

Proposal 5: For the issue of RRC message capacity due to collection of logged L1 measurement results, it is proposed RAN2 to discuss the following solutions: the UE can do segmentation of the RRC message, or the UE can stop logging any new measurements.

Discussion on data compression for logged L1 measurement results

Currently, UDC (Uplink Data Compression) has been defined, and DEFLATE based UDC solution could achieve higher compression efficiency which would save more uplink resources and reduce the transmission latency. The NG-RAN node can configure the UE to use UDC or not. If UDC is enabled, one byte UDC header is used to indicate whether the PDCP SDU is compressed by UDC or not. For UDC, the feature summary can be found in [4].

As shown in the performance evaluation [5], the KPI compression efficiency is defined as below:

Compression efficiency			1 – (output compressed data size / input uncompressed data size)

And then, UDC can achieve 40% ~ 80% compression efficiency for different service types.

The simulation results of UDC solution based on DEFLATE (solution 3) and Solution 4 (APDC) have shown a similar trend in terms of the compression efficiency for uplink on various input traffic profiles simulated including FTP, SIP signalling, video and web surfing in case 1 and case 2 type traffic scenarios. Wherein, about 40% to 50% compression efficiency is shown for FTP traffic, over 80% of compression efficiency is shown for SIP signalling and about 60% to 75% compression efficiency is shown for video traffic. Similarly, over 60% of compression efficiency can be obtained with web surfing data. From technical point of view, solution based on DEFLATE and solution based on APDC have shown significant and similar compression efficiency.

If UDC can be applied for RRC message carrying logged L1 measurement results, the RRC signaling capacity problem can be alleviated. In other words, more logged L1 measurement results can be transmitted with the same RRC signaling.

Proposal 6: It is proposed RAN2 to discuss the necessity of data compression for logged L1 measurement results.

- Conclusion

In this paper, we mainly address the FFSes left from RAN2#126. We have the following proposals:

Proposal 1: The contents of collected data should be specified by RAN1, and then RAN2 can follow RAN1's conclusions. The data collection requirements are discussed and decided based on use cases.

Proposal 2: The L1 measurement result collection period and reporting interval are configured by the network.

Proposal 3: It is proposed RAN2 to discuss the necessity of data volume-based event.

Proposal 4: It is proposed RAN2 to discuss the necessity of network request reporting.

Proposal 5: For the issue of RRC message capacity due to collection of logged L1 measurement results, it is proposed RAN2 to discuss the following solutions: the UE can do segmentation of the RRC message, or the UE can stop logging any new measurements.

Proposal 6: It is proposed RAN2 to discuss the necessity of data compression for logged L1 measurement results.

- References

RAN2#126 Chair Notes

TS 38.323

R2-2311720/R1-2310681, Reply LS on Data Collection Requirements and Assumptions, Source: RAN1, To: RAN2

RP-220142, Summary for WI NR Uplink Data Compression (UDC), CATT

TR 36.754, Study on UL data compression for E-UTRA