3GPP TSG-RAN WG2 Meeting #127							R2-2407332

Maastricht, Netherlands, 19 - 23 August, 2024

Title: 	Discussion on UE-sided data collection for training

Source: 	Huawei, HiSilicon, CAICT

Agenda item:	8.1.4

Document for:	Discussion/Decision

- Introduction

In the last RAN2#126 meeting, there are some agreements were achieved for UE-side data collection as follows [1]. After that meeting, the analysis table was discussed in [2], and here is the latest version as below:

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure FFS: impact to other layers | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. |  |  |  |  |

In this paper, we analyzed a few more technical aspects, i.e., FFS. In addition, as indicated in the RAN2#127 agenda, we also provide potential recommendation for this study objective.

- Discussion

Discussion on FFSes from RAN2#126

Discussion on Option 1a

Description of Option 1a

For Option 1a, UE collects and directly transfers training data to the Over-The-Top (OTT) server (3GPP transparent). In TR 38.843, it has been mentioned that the training data is generated by the UE, while the termination point for training data is the UE or OTT server, as follows.

- Model Training: o For UE-side models, training data can be generated by the UE, while the termination point for training data may include the UE or a UE-side OTT server. Note: RAN2 identified the cases in which OAM or Core Network may be used for UE-side model training. However, no study was conducted since this is beyond the scope of this Working Group. Note: RAN2 identified the case in which gNB may be used for UE-side model training. However, no conclusion was reached, as this depends on the RAN1 progress.

This means the UE-sided OTT server which belongs to a particular UE vendors collects the data and trains the model. Thus, the UE-sided OTT server knows what data it needs. Option 1a can leave the UE-sided data collection to implementation, which means the OTT server can collect the data that it needs more directly. The needed data type doesn’t need to be specified, and when/what to transmit is leave to UE, which provide full flexibility for UE/chipset vendors to train and implement their specific AI/ML models.

Last but not least, Option 1a doesn’t have any specification impacts (the RAN2#126 agreement is put as below), so it can save a lot of standardization work.

Agreements

1	Solution 1a has no specification impact.

Business implications of Option 1a

Currently, the transmission from a UE to the UE vendor’s server (or some cooperated application server) through 3GPP network is supported. The UE can use the same method to send training data to the UE-side OTT server for model training. Such procedure is supported in a transparent way to the network side, without need of control/visibility for the network/MNO. The UE itself should be responsible to protect data privacy and acquire the user consent. In this procedure, as the transparent tunnel, neither the network vendors or operators is to be blamed for any privacy reason.

More specifically, the training data is reported via UP tunnel, i.e. from application level data collection clients at the UE to the application server (i.e. UE-sided OTT server). It is typical that the data collection application client and the application server are paired (i.e. from the same vendor). A UE vendor may install its own data collection client at the UE to collect lower layer data and report to its own application server for AI/ML model training.

We should know that UE is always a whole from 3GPP perspective. The interaction between UE and any third-party server, is never been discussed in 3GPP. The interaction should be leave to implementation, e.g. business cooperation between companies.

To protect user privacy according to local regulations, some user consent request/provision procedure should be needed at application level, which is also out of 3GPP standardization scope.

Observation 1: In Option 1a, with the Data collection application client at the UE, the data transmission from UE to UE vendor’s application server can be supported without involvement of network side, by following regulations. The collected data type/format doesn’t need to be specified, which provides full flexibility for UE to train and implement their specific AI/ML models.

Discussion on Option 1b

Description of Option 1b

For the Option 1b, the description in the email discussion [2] as follows:

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). FFS whether according to RAN2 there might be 3GPP specification impact.

We can see that the UE collects the data and transfers it to the server for data collection for UE-side model training (inside the MNO, for simplicity, we mark it as the first entity) and then the first entity can choose whether to send the collected data to the OTT server (outside the MNO).

The UE can transfer the collected data by UP tunnel to the first entity inside MNO. From the transport protocol perspective, this Option is no different from Option 1a. The only difference is that the first entity (i.e. the server for data collection for UE-side model training) can optionally forward the collected data to the OTT server, but the transmission from a server to another server is completely out of 3GPP scope.

Observation 2: From the transport protocol perspective, Option 1b is no different from Option 1a. The only difference is the ownership of the server, and in Option 1b, the server for data collection for UE-side model training which is owned by MNO can optionally forward the collected data to the OTT server, but the transmission from a MNO’s server to OTT server is completely out of 3GPP scope.

Business implications of Option 1b

Different from Option 1a, the updated Option 1b implies that the server for data collection for UE-side model training (inside the MNO) is a node belonging to MNO, i.e. like a CN node. However, like Option 1a, the training data is reported via UP tunnel, and data collection application clients are still needed at the UE side to collect the data and send the data to the server for data collection for UE-side model training (inside the MNO).

As said, it is typical that the data collection application client and the application server are paired (i.e. from the same vendor). As the server for data collection belongs to MNO, it requires operators/MNOs to install their own paired application clients on UEs to collect data from lower layers, and send the collected data to the server. It is unclear how this can work, e.g. how the operator’s data collection client can collect the data from UE’s lower layers, how the operator’s application client is installed on UEs, and so on.

Observation 3: From business perspective, Option 1b requires operators/MNOs to have data collection application clients on UEs to collect data from lower layers, and send the collected data to their own server. It is unclear how this business can work, e.g. how the operator’s data collection client can collect the data from UE’s lower layers, how the operator’s data collection client is installed on UEs, and so on.

On the other hand, even if all the complicated business as mentioned above is possible for a MNO, Option 1b basically is Option 1a applied by MNO, plus forwarding data to OTT server (which is out of 3GPP). From controllability perspective, as it is MNO’s sever, MNO can control the data collection. From visibility perspective, like Option 1a, there is no standardized visibility for Option 1b.

Observation 4: From business perspective, Option 1b is like MNO applying Option 1a (which is not practical according to Observation 3), plus forwarding data to OTT server (which is out of 3GPP).

Proposal 1: For Option 1b, for "FFS: level of controllability" in controllability of MNO on data transfer aspect, it is proposed RAN2 to remove it.

Proposal 2: For Option 1b, for "FFS" in Possible Options for Visibility of data content aspect, it is proposed RAN2 to change it into: No standardized visibility.

Furthermore, like 1a, as it is only application layer communication based on UP tunnel. There is no need to specify anything in 3GPP. Therefore, we propose to remove “SA2, SA3, RAN2, RAN3, CT1” from the involved WGs.

Proposal 3: For Option 1b, it is proposed RAN2 to remove “SA2, SA3, RAN2, RAN3, CT1” from the involved WGs.

Discussion on Option 2 and Option 3

UP tunnel and protocol

According to [2], UP tunnel are listed as FFS for Option 2 and Option 3. We analyze these in this section.

For the UP tunnel, we think it depends on the data volume of the collected data. According to the RAN1 LS [3], we can calculate the data volume of the collected data roughly.

Assume that 1000 data samples are included in one measurement report, for training for the BM case, the data size for L1-RSRP can be up to 500 bits. Then the total size of the data is up to 500 Kbits, i.e., 62.5 Kbytes. For the training data collection for CSI prediction, the data size of the one data sample can be up to 1.5 Mbits.

For training data collection, there is no latency requirement according to RAN1 evaluation, and then the data can be transmitted from UE to network for multiple times. If the training data is transferred via NAS signalling, it should be technically feasible, and thus the necessity of using UP tunnel is unclear.

For the positioning case, according to the RAN1 LS, the data size of one data sample is as follows.

Note 3: The measurement size of one data sample = (measurement data size of one PRS/SRS resource)*(number of PRS/SRS resources needed for model input). The label size of one data sample = (label data size of one PRS/SRS resource)*(number of PRS/SRS resources needed for model output). The quantization and bit representation of time, power, and phase information (including their necessity) still need to be further discussed. Existing specification allows reporting of up to 64 PRS/SRS resources per frequency layer for one positioning fix. For evaluations, most companies considered up to 18 TRPs. It should be noted that AI/ML positioning is not restricted to work only with maximum of 18 TRPs. - Example of calculation on a potential lower bound on measurement size per PRS/SRS resource: - A potential lower bound on measurement size per PRS/SRS resource can be calculated as follows (assuming timing only for 9 measurements per PRS/SRS resource): 16 + 9*8 = 88 bits. The total lower bound can be 88*N bits, where N is number of PRS/SRS resources used as model input for obtaining a positioning fix. This is based on the assumption of timing info as 16 bits for first arrival and 9 bits for relative timing. - Example of calculation of a potential upper bound on measurement size per PRS/SRS resource: - A potential upper bound on measurement size per PRS/SRS resource can be calculated as follows (assuming timing, power, and phase for 256 measurements per PRS/SRS resource and assuming 8 bit representation of each real number): 2*(8*256) = 4096 bits. The total upper bound can be 4096*N bits, where N is number of PRS/SRS resources used as model input for obtaining a positioning fix. - For location coordinates (corresponding to model output) - The bit representation of location coordinates depends on the type of shape, resolution, and uncertainty used to indicate the location (e.g., ellipsoid point, ellipsoid point with uncertainty circle, high accuracy ellipsoid with uncertainty ellipsoid, etc.) as listed in TS 23.032. The range of bit representation for location coordinates can be 7 bytes to 18 bytes (i.e., 56 to 144 bits). The location information report in existing specifications may contain additional information besides location coordinates (e.g., velocity, location error, integrity info, etc.) - For intermediate positioning measurement (corresponding to model output): - The quantization and bit representation of time, [RSRP/RSRPP], and LOS/NLOS information (including their necessity) as model output still need to be discussed in an appropriate working group. As a reference to existing timing representation in Rel17 [TS 37.355], an example on the label size can be of 21 bits per PRS/SRS resource while assuming model output produces one timing of 21 bits per PRS/SRS resource. The label size can be 21*N bits, where N is number of PRS/SRS resources for which intermediate positioning measurement has been generated. If LOS/NLOS indicator (1 bit per PRS/SRS resource assuming hard value for LOS/NLOS indicator) is included, the label size becomes 22*N bits.

We can see that multiple cases and measurements need to be considered. For simplicity, we use the maximum data size to calculate the data volume for data collection. Similarly, assume that there are 1000 data samples in one measurement report, the total size of one measurement report is 4.096 Mbits.

Similar to the analysis for NAS signalling, it should be technically feasible since there is no latency requirement for training data collection. Thus, the necessity of using UP tunnel for LPP is unclear.

Observation 5: For training data collection, there is no latency requirement according to RAN1 evaluation, so the data can be reported from UE to network in multiple transmission, and the transmission by using control plane is technically feasible.

On the other hand, there is no difference between Option 1b (or 1a) and UP options for Option 2/3, from data transmission perspective. In UP option for Option 2/3, the node in CN (in Option 2) and OAM (in Option 3) would be like MNO’s application servers, and like 1b, it still requires operators/MNOs to install paired application clients on UEs to collect data from lower layers, and send the collected data to the server, which is basically unpractical.

Observation 6: From the transport protocol perspective, there is no difference between UP options for Option 2/3 and Option 1a/1b.

Proposal 4: For Option 2/3, there is no need to discuss UP options (no different from Option 1a/1b).

Proposal 5: For Option 2, it is proposed RAN2 to agree the following:

- for "FFS: UP tunnel" in UP/CP tunnel aspect, it is proposed RAN2 to remove it.

- for "FFS: the protocol layer for UP tunnel" in Protocol layer for data transfer, it is proposed RAN2 to remove it.

- for "FFS: impact to other layers" in Solution for network controllability, it is proposed RAN2 to remove it.

Proposal 6: For Option 3, it is proposed RAN2 to agree the following:

- for "FFS: UP tunnel" in UP/CP tunnel aspect, it is proposed RAN2 to remove it.

- for "FFS: the protocol layer for UP tunnel" in Protocol layer for data transfer, it is proposed RAN2 to remove it.

Visibility of data content in MNO and Data format

There are three options for the visibility of the data content.

Opt A) Full visibility for standardized data content.

FFS: Opt B) Partial visibility for partially standardized data content.

FFS Opt C) No standardized visibility.

For Option 2 and 3, as discussed above, there is no clear need to specify containers to deliver non-standardized data via control plane option. Any non-standardized data can be based on UP options, i.e. Option 1a.

Proposal 7: For Option 2/3, it is proposed RAN2 to remove the following 3 FFSes in Possible Options for Visibility of data content aspect:

- FFS: Opt B) Partial visibility for partially standardized data content.

- FFS Opt C) No standardized visibility.

- FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility.

Based on our proposal 4 ~ 7, we see no need to involve SA2 for Option 3, so "FFS SA2" can be removed.

Proposal 8: For Option 3, for "FFS SA2" in involved WGs aspect, it is proposed RAN2 to remove it.

Business implications of Option 2/3

For Option 2/3, different from UP option, there is no need to have data collection application client at the UE to report the data, that is relatively simpler compared to UP options. RRC or NAS or other lower layers in UE can report the data to RAN/CN, and then the data can be forwarded from a MNO’s node to outside OTT server. However, the business is still complex, as it requires per-vendor per-MNO business discussion on the required data.

Observation 7: From business perspective, Option 2/3 with CP tunnel is relatively simpler to MNO compared to UP options, but still requires per-vendor per-MNO business discussion on the required data.

Recommendation

According to our observations shown above, we don’t see a difference between those options based on UP tunnel, i.e. Option 1a/1b or Option 2/3 with UP tunnel. We suggest to only adopt Option 1a as the candidate solution.

Proposal 9: For the study objective of UE-sided model training data, we recommend to only adopt Option 1a as the candidate solution.

- Conclusion and Recommendation

According to our observations above, we provide the following proposals to address the FFSes in the analysis table:

FFSes for Option 1b

Proposal 1: For Option 1b, for "FFS: level of controllability" in controllability of MNO on data transfer aspect, it is proposed RAN2 to remove it.

Proposal 2: For Option 1b, for "FFS" in Possible Options for Visibility of data content aspect, it is proposed RAN2 to change it into: No standardized visibility.

Proposal 3: For Option 1b, it is proposed RAN2 to remove “SA2, SA3, RAN2, RAN3, CT1” from the involved WGs.

FFSes for Option 2/3

Proposal 4: For Option 2/3, there is no need to discuss UP options (no different from Option 1a/1b).

Proposal 5: For Option 2, it is proposed RAN2 to agree the following:

- for "FFS: UP tunnel" in UP/CP tunnel aspect, it is proposed RAN2 to remove it.

- for "FFS: the protocol layer for UP tunnel" in Protocol layer for data transfer, it is proposed RAN2 to remove it.

- for "FFS: impact to other layers" in Solution for network controllability, it is proposed RAN2 to remove it.

Proposal 6: For Option 3, it is proposed RAN2 to agree the following:

- for "FFS: UP tunnel" in UP/CP tunnel aspect, it is proposed RAN2 to remove it.

- for "FFS: the protocol layer for UP tunnel" in Protocol layer for data transfer, it is proposed RAN2 to remove it.

Proposal 7: For Option 2/3, it is proposed RAN2 to remove the following 3 FFSes in Possible Options for Visibility of data content aspect:

- FFS: Opt B) Partial visibility for partially standardized data content.

- FFS Opt C) No standardized visibility.

- FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility.

Proposal 8: For Option 3, for "FFS SA2" in involved WGs aspect, it is proposed RAN2 to remove it.

We also have analyzed technical and business perspectives of each solution, with the following observations:

Technical and business observations

Observation 1: In Option 1a, with the Data collection application client at the UE, the data transmission from UE to UE vendor’s application server can be supported without involvement of network side, by following regulations. The collected data type/format doesn’t need to be specified, which provides full flexibility for UE to train and implement their specific AI/ML models.

Observation 2: From the transport protocol perspective, Option 1b is no different from Option 1a. The only difference is the ownership of the server, and in Option 1b, the server for data collection for UE-side model training which is owned by MNO can optionally forward the collected data to the OTT server, but the transmission from a MNO’s server to OTT server is completely out of 3GPP scope.

Observation 3: From business perspective, Option 1b requires operators/MNOs to have data collection application clients on UEs to collect data from lower layers, and send the collected data to their own server. It is unclear how this business can work, e.g. how the operator’s data collection client can collect the data from UE’s lower layers, how the operator’s data collection client is installed on UEs, and so on.

Observation 4: From business perspective, Option 1b is like MNO applying Option 1a (which is not practical according to Observation 3), plus forwarding data to OTT server (which is out of 3GPP).

Observation 5: For training data collection, there is no latency requirement according to RAN1 evaluation, so the data can be reported from UE to network in multiple transmission, and the transmission by using control plane is technically feasible.

Observation 6: From the transport protocol perspective, there is no difference between UP options for Option 2/3 and Option 1a/1b.

Observation 7: From business perspective, Option 2/3 with CP tunnel is relatively simpler to MNO compared to UP options, but still requires per-vendor per-MNO business discussion on the required data.

In summary, we have the following recommendation:

Recommendation

Proposal 9: For the study objective of UE-sided model training data, we recommend to only adopt Option 1a as the candidate solution.

We also have some text proposal for TR 38.843 in section 5.

- References

Draft_R2_126_Meeting_Report_v1

R2-24XXXXX RAN2 inputs to TR 38843_v22_Rapp (related to [POST126][034][AIML PHY] TP for data collection  (Ericsson))

R2-2311720/R1-2310681, Reply LS on Data Collection Requirements and Assumptions, Source: RAN1, To: RAN2

- Text proposals for TR 38.843

- 7.2.1.3	Data collection

[Partially omitted]

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel | RRC layer for CP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | No standardized visibility | Opt A) Full visibility for standardized data content. | Opt A) Full visibility for standardized data content. |
| Involved WGs | N/A | N/A | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. |  |  |  |  |

[Partially omitted]

- Business implications for different options:

- In Option 1a, with the Data collection application client at the UE, the data transmission from UE to UE vendor’s application server can be supported without involvement of network side, by following regulations. The collected data type/format doesn’t need to be specified, which provides full flexibility for UE to train and implement their specific AI/ML models.

- Option 1b requires operators/MNOs to have data collection application clients on UEs to collect data from lower layers, and send the collected data to their own server. It is unclear how this business can work, e.g. how the operator’s data collection client can collect the data from UE’s lower layers, how the operator’s data collection client is installed on UEs, and so on. From business perspective, Option 1b is like MNO applying Option 1a, plus forwarding data to OTT server (which is out of 3GPP).

- Option 2/3 with CP tunnel is relatively simpler to MNO compared to UP options, but still requires per-vendor per-MNO business discussion on the required data.

- Comparison between different options:

- From the transport protocol perspective, Option 1b is no different from Option 1a. The only difference is the ownership of the server, and in Option 1b, the server for data collection for UE-side model training which is owned by MNO can optionally forward the collected data to the OTT server, but the transmission from a MNO’s server to OTT server is completely out of 3GPP scope.

- From the transport protocol perspective, there is no difference between UP options for Option 2/3 and Option 1a/1b.

- Recommendation:

- For the study objective of UE-sided model training data,  it is recommended to only adopt Option 1a as the candidate solution.