3GPP TSG-RAN WG2 Meeting #127							R2-2407333

Maastricht, Netherlands, 19 - 23 August, 2024

Title: 	Discussion on MRO enhancements for Rel-18 mobility features

Source: 	Huawei, HiSilicon

Agenda item:	8.10.2

Document for:	Discussion/Decision

- Introduction

In this paper, we mainly analyze possible RAN2 impacts for LTM, CHO with candidate SCGs and subsequent CPAC.

- Discussion

Discussion on LTM

Per the agreement from RAN2#125bis, RAN2 has agreed on the definition of too early LTM, too late LTM and LTM to wrong cell, and to focus on the MCG LTM first. RAN2#126 meeting has achieved the following agreements:

We correct the definition of scenario 3a as follows: Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

To facilitate the further discussion, we illustrate the steps during the LTM procedure to have a retro on the design of Rel-18 LTM feature, compared to those during legacy L3 handover.

Figure 1: Handover procedures for Legacy L3 HO and LTM

The division of failure cases for LTM is similar as those for handover, since the LTM is built and augmented on top of L3 handover. For handovers, the SON features including the radio link failure (RLF) report and successful handover report (SHR) provides the UE feedback information. The RLF and SHR could be augmented to also provide useful feedback information to address the LTM mobility issues. We will discuss the possible MRO enhancement for those SON reports.

RLF report for LTM

The existing timeConnFailure IE is used to indicate the time elapse since the last HO execution until connection failure, i.e., since the last execution of the last RRC reconfiguration message including the reconfigurationWithSync. This time information is used for the network to determine the failure type, e.g., too late, too early and wrong cell. In our understanding, only the field description associated to the IE needs to be updated accordingly as below. Otherwise, no specification impact is foreseen.

timeConnFailure This field is used to indicate the time elapsed since the last HO execution or the cell switch execution until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

The reconnectCellId indicates a cell in which UE comes to connected after failing to perform the reestablishment including the possible LTM recovery and normal RRC re-establishment. The field description and the procedural text applies to LTM scenario. No enhancement for reconnectCellId is foreseen for LTM.

Proposal 1: only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, no specification impact for timeConnFailure and reconnectCellId is foreseen.

The choCellId indicates the CHO candidate cell which the UE selects for CHO based recovery instead of RRC Reestablishment after the connection failure. Similar to the CHO recovery, the UE may perform LTM recovery instead of re-establishment procedure, i.e., if the UE configured with LTM configurations for some candidate cells, selects a candidate cell to execute the HO using the LTM candidate cell configuration. Therefore, a new field in RLF report is needed to indicate the LTM recovery cell.

Proposal 2: introduce a new field in RLF report to indicate the LTM recovery cell.

- When the HOF/RLF occurs in the target cell, the specific access type should be available to the gNB, i.e. whether it is RA-based or RA-less cell switch.

Proposal 3: RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch.

Per current specification, the UE could perform the UE-based TA measurement and perform PDCCH ordered TA acquisition simultaneously. If there are two valid TA, i.e. one from cell switch command and the other measured by UE itself, the UE applies the former one. If there is only either one TA of those two, the UE simply applies it. In addition to access type, the UE also indicates that whether the network provides the TA in the cell switch command, in case that there are two valid TAs simultaneously.

Proposal 4: RAN2 include in the RLF report whether the network provides the TA in the cell switch command.

In order for the network the evaluate the TA freshness of target cell, the UE could indicate the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

Proposal 5: RAN2 include in the RLF report the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

SHR report for LTM

Another distinct nature of LTM is that the UE could perform a RACH-less cell switch towards the target cell with valid TA acquired from the PDCCH order’s TA acquisition or UE-based TA measurement. It is beneficial for the UE to recognize the RA-based cell switch as a near failure, since the UE is supposed to benefit from the LTM feature by performing a RACH-less access. Therefore, a RA-based access could be viewed as a sub-optimal LTM mobility incident. We recommend, the RA-based access is a SHR trigger for LTM feature.

If the UE is configured with LTM config, the UE has no available valid TA upon reception of cell switch command and has to perform RA-based access towards target cell, which causes long UP interruption time. It is beneficial to recognize the RA-based cell switch as a near failure, since the UE is supposed to benefit from the LTM feature by performing a RACH-less access and thus avoiding long UP interruption time according to Rel-18 WID of LTM.

One thing we would like to clarify, the UE attempt the RA-based access until T304 expiry, which is equivalent to declaration of HOF. Before T304 expiry, there is no such thing as fall back from RA-less to RA-based. Only HOF occurs, the UE may perform LTM based recovery, which is based on RA using LTM cell configuration. However, we call it an LTM recovery procedure instead of fallback procedure.

Observation 1: Before T304 expiry, there is no such thing as fall back from RACH-less to RACH-based. Only HOF occurs, the UE may perform LTM based recovery, which is based on RA using LTM cell configuration. However, we call it an LTM recovery procedure instead of fallback procedure.

Proposal 6: In case the UE is configured with LTM config, the RA-based access could be one triggering condition for the SHR report.

According to TS 38.300, the LTM defines the UP/HO interruption as depicted in below Figures in order to measure the latency of RACH-less and RACH-based procedure:

Figure 2: Mobility Latency for RACH-based LTM

Figure 3: Mobility Latency for RACH-less LTM

HO interruption time for LTM is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. The HO interruption time could be set as a trigger for SHR report to reflect the sub-optimal latency case caused by LTM mobility.

Proposal 7: In case the UE is configured with LTM config, the HO interruption time could be one triggering condition for the SHR report.

Discussion on CHO with candidate SCGs

About assumptions on configurations

According to RAN2#126 Chair Notes [1], for the assumptions on configurations, the following options were proposed for discussions:

Option 1: R18 configurations of CHO with candidate SCGs

Option 2: R17 configurations of CHO with target SCG

Option 3: R16 configurations of CHO

Some companies proposed to discuss Option 1 only, while some companies proposed to discuss mixed configurations, e.g. Option 1 + Option 2, or Option 1 + Option 3. Since the Option 1 is the target case for this Rel-18 feature objective, the usage of mixed configurations are just a supplement to Option 1 in case that these parameters of Option 1 could not be configured properly and thus raise the risk of mobility failure issues.

We think RAN2 can focus on Option 1 first, and other options (including mixed configuration) can be de-prioritized.

Proposal 8: RAN2 focus on only R18 configurations of CHO with candidate SCGs first.

Discussions on scenarios and issues

At RAN2#126, we submitted [2] and we showed 9 failure cases for CHO with candidate SCGs. As below, we provide more technical analysis  on these scenarios and possible issues.

Discussions on Case 2)

For Case 2), it is about too late handover, and then UE will suffer RLF on PCell.

Figure 4: Handover procedure for Case 2)

At RAN2#126 meeting, RAN2 made the following agreements, which can be used for Case 2).

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

For RLF report, currently the IE lastHO-Type is used to indicate the type of the last executed handover before the last detected connection failure, and it has defined CHO and DAPS. For Case 2), if a new HO type CHO with candidate SCGs can be added, the network can do optimizations based on this kind of handover. For now, we see no strong need to enhance SCGFailureInformation for Case 2).

Proposal 9: For Case 2) where execution conditions for PCell/PScell are met but handover to PCell/PScell fails, it is proposed RAN2 to discuss the need of introducing a new HO type CHO with candidate SCGs in the IE lastHO-Type in RLF report.

Discussions on Case 7)

For Case 7), both PCell mobility and PScell mobility are triggered. And then, either PCell or PScell or both fail.

Figure 5: Handover procedure for Case 7)

For Case 7), if PCell fails and PScell succeeds, the UE will do RRC re-establishment and also generate RLF report. In this case, the UE can indicate the cause (e.g. PCell failure+PScell success) to the network, so that the network can optimize configuration parameters correspondingly.

If PCell succeeds and PScell fails, the UE will change its PCell successfully, but PScell change fails. Later, The network can do something to handle it. So SCGFailureInformation enhancement may be needed. For example, MN associated with failed SCG can be considered in order to reflect CHO with candidate SCGs.

If both PCell and PScell fail, similar to the first case for Case 7), the cause can be indicated to the network for optimizations.

Proposal 10: For the case of PCell failure and PScell success, and the case of PCell failure and PScell failure, it is proposed RAN2 to discuss the need of introducing the causes in the RLF report.

Proposal 11: For the case of PCell success and PScell failure, it is proposed RAN2 to discuss the need of introducing MN associated with failed SCG in SCGFailureInformation message.

Discussions on Case 8)

For Case 8), both PCell and PScell change successfully, and then either PCell or PScell or both fail in a short time. This belongs to "failure shortly after a successful handover".

Figure 6: Handover procedure for Case 8)

We think Case 8) can be categorized into near failure case, and then successful handover report can be enhanced for PCell/PScell mobility.

Firstly, for the time period between CHO execution time and CPC execution time, it can be included in either SHR or SPR. In addition, the trigger based on the time period can be considered, e.g. if the time period is above a threshold, it may be near failure, and such successful handover reports should be triggered.

Secondly, for Case 8), the UE may generate SHR and SPR. Then whether/how to co-ordinate both reports may be discussed. For example, the UE may generate SHR for PCell mobility and generate SPR for PScell mobility in Case 8), and two reports are actually related to one single R18 mobility procedure (i.e. CHO with candidate SCGs).

Proposal 12: Time period between CHO execution time and CPC execution time can be added in SHR or SPR.

Proposal 13: For a successful CHO with candidate SCGs, time period between CHO execution time and CPC execution time could be one triggering condition for SHR or SPR.

Proposal 14: For a successful mobility procedure of CHO with candidates SCGs, RAN2 to discuss whether SHR and SPR need to be correlated or not.

Other scenarios (see no issues and no enhancements needed)

Apart from the above scenarios, there are more scenarios but we observe no issues and thus no enhancements are needed.

For Case 1), it is about too late handover, and PCell/PScell mobility is not triggered. In this case, we think legacy SON reporting can work.

Figure 7: Handover procedure for Case 1)

For Case 4), we think CHO failure reporting can be followed. In this case, no candidate SCG mobility is triggered, so no extra information needs to be collected.

If R16 CHO fails, legacy SON reporting is followed.

Figure 8: Handover procedure for Case 4)

For Case 5), it is CHO only and SCG is not changed. This assumes that UE has both R16 CHO and R18 CHO with candidate SCGs. If R16 CHO shortly RLF, legacy SON reporting is followed.

Figure 9: Handover procedure for Case 5)

For Case 6), it is CHO only and SCG is not changed. This assumes that UE has both R17 (CHO+SCG)+R18 CHO with candidate SCGs. If R17 CHO+SCG succeeds, and SCG failure happens quickly, legacy SCGFailureInformation is followed.

Figure 10: Handover procedure for Case 6)

For 9d, there are two failures during the access procedure, priority of this case? According to internal discussion, when UE is in DC and stable state, Fast MCG recovery can happen. So 9b does not exist.

Figure 11: Handover procedure for Case 9)

Discussion on subsequent CPAC

For subsequent CPAC discussion, RAN2 reached the following agreement at the last meeting.

RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

Since RAN3 is discussing the failure and near failure scenarios of SCPAC and the MRO enhancements. We think that RAN2 should fully consider and keep consistency with the RAN3’s agreements.

Proposal 15: For S-CPAC, RAN2 mainly to wait for RAN3 progress and do the high-level discussion on which cases supported by legacy and which cases needs enhancement.

From RAN2 side, we notice that several companies discussed the issue of overwritten SPR for near failure scenarios. Since further enhancements might required frequent report recording and sending, there may exists report overwritten if they are not sent immediately. Therefore, the impacts of the overwritten issues and whether it needs to be tackled should be discussed. In our opinion, some out-of-time reports may help the network to know the details of the historical states. However, if the SPR is related to the old configurations or some sub-optimal solutions that are not adopted, we see no big problems if these reports are overwritten. Therefore, the overwritten issue should justified first.

Proposal 16: RAN2 to discuss the impacts and the potential mechanism of overwritten issues, e.g., in what cases it matters and should be enhanced.

- Conclusion

Based on the discussion, we have the following observations and proposals:

LTM

Proposal 1: only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, no specification impact for timeConnFailure and reconnectCellId is foreseen.

Proposal 2: introduce a new field in RLF report to indicate the LTM recovery cell.

Proposal 3: RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch.

Proposal 4: RAN2 include in the RLF report whether the network provides the TA in the cell switch command.

Proposal 5: RAN2 include in the RLF report the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

Observation 1: Before T304 expiry, there is no such thing as fall back from RACH-less to RACH-based. Only HOF occurs, the UE may perform LTM based recovery, which is based on RA using LTM cell configuration. However, we call it an LTM recovery procedure instead of fallback procedure.

Proposal 6: In case the UE is configured with LTM config, the RA-based access could be one triggering condition for the SHR report.

Proposal 7: In case the UE is configured with LTM config, the HO interruption time could be one triggering condition for the SHR report.

CHO with candidate SCGs

Proposal 8: RAN2 focus on only R18 configurations of CHO with candidate SCGs first.

Proposal 9: For Case 2) where execution conditions for PCell/PScell are met but handover to PCell/PScell fails, it is proposed RAN2 to discuss the need of introducing a new HO type CHO with candidate SCGs in the IE lastHO-Type in RLF report.

Proposal 10: For the case of PCell failure and PScell success, and the case of PCell failure and PScell failure, it is proposed RAN2 to discuss the need of introducing the causes in the RLF report.

Proposal 11: For the case of PCell success and PScell failure, it is proposed RAN2 to discuss the need of introducing MN associated with failed SCG in SCGFailureInformation message.

Proposal 12: Time period between CHO execution time and CPC execution time can be added in SHR or SPR.

Proposal 13: For a successful CHO with candidate SCGs, time period between CHO execution time and CPC execution time could be one triggering condition for SHR or SPR.

Proposal 14: For a successful mobility procedure of CHO with candidates SCGs, RAN2 to discuss whether SHR and SPR need to be correlated or not.

Subsequent CPAC

Proposal 15: For S-CPAC, RAN2 mainly to wait for RAN3 progress and do the high-level discussion on which cases supported by legacy and which cases needs enhancement.

Proposal 16: RAN2 to discuss the impacts and the potential mechanism of overwritten issues, e.g., in what cases it matters and should be enhanced.

References

Draft_R2_126_Meeting_Report_v2

R2-2405334, MRO enhancements for Rel-18 mobility features, Huawei, HiSilicon