3GPP TSG-RAN WG2 Meeting #127						R2-2407334

Maastricht, Netherlands, 19 - 23 August, 2024

Title: 	Discussion on support of the Rel-18 leftovers

Source: 	Huawei, HiSilicon

Agenda item:	8.10.5

Document for:	Discussion/Decision

1   Introduction

The RAN2#126 meeting has achieved the following agreements:

For the purpose of SON enhancements for SDT, include RSRP/data volume related information

Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

Reply to RAN3 that we will only do EN-DC. RAN2 understands that whether also supporting (NG)EN-DC has no additional RAN2 impact hence RAN3 can decide. If later we get time we can consider other options.

In this paper, we will discuss the leftover issues from Rel-18 SON/MDT.

2   Discussion

2.1 RACH optimization for SDT

RA report for SDT has been standardized in Rel-18, which is one means to detect the SDT related RACH resource issues. The Rel-19 WID allows the further development of RACH optimization for SDT.

RAN2 has agreed to introduce the downlink RSRP and buffered data volume at the time of deciding whether to perform SDT or not in order to optimize the condition parameters for triggering the SDT. Some company were also interested in the RSRP and data volume at the time of SDT failure. Between the time of SDT initiation and SDT failure, the RSRP and buffered data volume may change a lot, which is dynamic random variation. In our understanding, the RSRP value and data volume when SDT failure happens is not helpful for network to adjust the condition parameter for triggering SDT or not.

Proposal 1: It is proposed RAN2 to investigate the necessity of including RSRP threshold and data volume threshold when SDT failure happens.

2.2 MRO for MR-DC SCG failure

Rel-17 SON/MDT WI developed the solution for MRO for NR-DC SCG failures according to TS 38.331 specification [2]. In Rel-17, SON/MDT WI considered primarily two SCG failure scenarios as follows:

- SCG RLF, and

- Legacy PSCell change or addition failure.

EN-DC

For EN-DC scenario, the UE report the NR SCG failure information to LTE MN node, and then the LTM MN node may further forward the NR SCG failure information to NR SN node. The SCGFailureInformationNR message reported from UE to LTE node needs to be enhanced in TS 36.331 accordingly to align with the SCGFailureInformation defined in TS 38.331.

Table 1 Enhancement to align with the NR-DC SCG failure report in TS 38.331

| Parameter | IE/value & condition |
| --- | --- |
| failedPSCellId | Frequency and PCI |
| previousPSCellId | Frequency and PCI |
| timeSCGFailure | 0-1023 |
| perRAInfoList | Only can be included if failureType is set to synchReconfigFailureSCG, or is set to randomAccessProblem while T304 is running |

NE-DC

For NE-DC scenario, the UE report the LTE SCG failure information to NR MN node, and then the NR MN node may further forward the LTE SCG failure information to LTE SN node. Therefore, the SCGFailureInformationEUTRA message in TS 38.331 needs to be enhanced accordingly.

NGEN-DC

Similar as EN-DC case, the SCGFailureInformationNR message defined in TS 36.331 needs to be enhanced accordingly, as above discussed.

Proposal 2: For EN-DC and NGEN-DC, the same enhancement to SCGFailureInformationNR message in TS 36.331 is introduced, i.e., failedPSCellId, previousPSCellId, timeSCGFailure and perRAInfoList.

Proposal 3: For NE-DC, the enhancement to SCGFailureInformationEUTRA message in TS 38.331 is introduced, i.e., failedPSCellId, previousPSCellId, timeSCGFailure and perRAInfoList.

3   Conclusion

In this paper, we provide the following proposals:

RACH optimization for SDT:

Proposal 1: It is proposed RAN2 to investigate the necessity of including RSRP threshold and data volume threshold when SDT failure happens.

MRO for MR-DC SCG failure:

Proposal 2: For EN-DC and NGEN-DC, the same enhancement to SCGFailureInformationNR message in TS 36.331 is introduced, i.e., failedPSCellId, previousPSCellId, timeSCGFailure and perRAInfoList.

Proposal 3: For NE-DC, the enhancement to SCGFailureInformationEUTRA message in TS 38.331 is introduced, i.e., failedPSCellId, previousPSCellId, timeSCGFailure and perRAInfoList.