3GPP TSG-RAN WG2 Meeting #127	R2-24xxxxx

# Maastricht, Netherlands, 19 - 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | DraftCR | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on area scope checking for MBS QoE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE_enh-Core |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Regarding the area scope checking for QoE, we have spotted two issues: Issue 1: For the area scope checking for Idle/Inactive QoE, it is only done when the QoE session starts in application layer, and once it is started, there is no need to do the area scope checking again. However, currently the UE always performs the area scope checking regardless of QoE session status, which leads to unnecessary AS<->APP layer interactions. Issue 2: At RAN2#126 meeting, it was discussed whether the MBS broadcast service area should be checked as part of the area scope checking. To be more specific, for cell re-selection procedures, the majority view was to let the UE check the re-selected cell for the area scope checking, based on which RAN2 agreed to keep the current description for QoE MBS area scope checking (as below). 2> if the UE re-selects to a new cell which is part of the area indicated by qoe-AreaScope and the previous cell was not part of the area indicated by qoe-AreaScope: 3> inform upper layers of being inside the area; 2> if the UE re-selects to a new cell which is not part of the area indicated by qoe-AreaScope and the previous cell was part of the area indicated by qoe-AreaScope: 3> inform upper layers of being outside the area. However, we find that during the state transitions, the UE still checks the area scope based on a cell where the UE receives MBS broadcast service, which is not aligned with RAN2#126 progress. The UE behaivours should be uniform between state transitions and cell re-selection cases. 2> if the cell where the UE receives the service when the UE transits from RRC_CONNECTED state to RRC_INACTIVE or RRC_IDLE state is part of the area indicated by qoe-AreaScope: 3> inform upper layers of being inside the area; 2> if the cell where the UE receives the service when the UE transits from RRC_CONNECTED state to RRC_INACTIVE or RRC_IDLE state is not part of the area indicated by qoe-AreaScope: 3> inform upper layers of being outside the area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: For Issue 1, update the conditions for UE doing the area scope checking by adding the following to ensure that the area scope is only checked when the service has not yet started or when the service status is not known at AS layer: 1> if the transmissionOfSessionStartStop is set to true and the related service is not ongoing, or the transmissionOfSessionStartStop is set to false: Change 2: For Issue 2, the change is that if qoe-AreaScope is configured, when state transitions happen, the UE checks the cell which the UE selects for the area scope checking. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: QoE measurement collection Inter-operability: for change 1 There is no inter-operability issue as the CR only impacts UE. for change 2 There is no inter-operability issue as the CR only impacts UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not agreed, the following issues still remain in this specification: If qoe-AreaScope is configured, the UE always checks the area scope regardless of the QoE session status, which leads to unnecessary AS<->APP layer interactions at the UE side. If qoe-AreaScope is configured, when state transitions happen, the UE still checks MBS broadcast service cell for the area scope checking, which causes complexities to UE and is misaligned with the UE behaviour for area scope checking during UE mobility between the cells. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5b.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.5b	Application Layer Measurements in RRC_IDLE/RRC_INACTIVE

5.5b.1	Area handling and storing of Application Layer Measurement reports in RRC_IDLE/RRC_INACTIVE

5.5b.1.1	General

This procedure specifies the area handling and storing of application layer measurement report containers by a UE in RRC_IDLE and RRC_INACTIVE that has an application layer measurement configuration with appLayerIdleInactiveConfig configured.

5.5b.1.2	Initiation

While in RRC_INACTIVE and RRC_IDLE state, the UE shall:

1>	store any previously or subsequently received application layer measurement report containers associated with the measConfigAppLayerId for which the successful transmission of the message or at least one segment of the message has not been confirmed by lower layers;

NOTE:	The UE continues storing application layer measurement report containers for MBS broadcast service in RRC_CONNECTED as long as stored application layer measurement configurations/reports have not been retrieved.

1>	if the memory reserved for storing application layer measurement report containers becomes full:

2>	if the reports are associated with a configuration including appLayerMeasPriority:

3>	discard reports in priority order where reports with the lowest priority are discarded first, and among reports with equal associated priority, discard reports in the order they were received, where older reports are discarded first;

2>	else:

3>	discard reports in the order they were received, where older reports are discarded first;

1>	if qoe-AreaScope is configured; and

1>	if the transmissionOfSessionStartStop is set to true and the related service is not ongoing, or the transmissionOfSessionStartStop is set to false:

2>	if the cell which the UE selects when the UE transits from RRC_CONNECTED state to RRC_INACTIVE or RRC_IDLE state is part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being inside the area;

2>	if the cell which the UE selects when the UE transits from RRC_CONNECTED state to RRC_INACTIVE or RRC_IDLE state is not part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being outside the area.

2>	if the UE re-selects to a new cell which is part of the area indicated by qoe-AreaScope and the previous cell was not part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being inside the area;

2>	if the UE re-selects to a new cell which is not part of the area indicated by qoe-AreaScope and the previous cell was part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being outside the area.