3GPP RAN WG2 Meeting #127                                                     R2-2407342

Maastricht, Netherland, Aug 19th – 23rd, 2024

Agenda Item:	8.2.2

Source:	HONOR

Title:	Discussions on functionalities required for A-IoT Communication

Document for:	Discussion and Decision

1. Introduction

In RAN2#126, the following aspects were agreed for functionality aspects [1]:

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed)

In this contribution, we will discuss the possible functionality for A-IoT device, focusing on resource allocation, padding, segmentation and reassemble.

2 Discussion

2.1 Need of resource allocation

In past RAN1 meeting, it has been agreed that both TDMA and FDMA are candidate transmission mode for D2R transmission and PRDCH should provide scheduling information for PDRCH transmission, which are highlighted in blue.

Agreements of RAN1#116bis [2]

Agreements of RAN1#117 [3]

Observation 1: RAN1 has agreed that the PDRCH transmission should be performed based on PRDCH scheduling information.

Therefore, the resource allocation is a necessary A-IoT functionality. The scheduling information may include time-domain information or time-domain/frequency-domain information which depends on the D2R transmission mode. From RAN2 perspective, the transmission mode should be taken into consideration for the resource allocation.

Proposal 1: Resource allocation is a necessary A-IoT functionality. RAN2 to study how to allocate resource for A-IoT device supporting different transmission mode, e.g., TDMA, FDMA.

Rel-19 A-IoT WI focuses on the DT and DO-DTT traffic types. Firstly, the reader should trigger the D2R transmission via triggering message and then the A-IoT device could transmit the packets using the allocated resource. Therefore, the reader could allocate D2R resource separately or along with the triggering message, e.g., initial message or A-IoT paging. Considering TDM is the only R2D option, resource allocation along with the triggering message could be more efficient especially when the triggering message aims to several A-IoT devices.

Proposal 2: RAN2 to take resource allocation along with the triggering message as the baseline, and to discuss whether and when to allocate resource separately, e.g., transmit the triggering message and perform the resource allocation separately.

Since whether the message size is visible is still FFS, there exists three options for resource allocation:

- Option1: allocating resources according to accurate message size.

- Option2: allocating resources for up to maximum message size.

- Option3: allocating resources based on reader implementation.

In the following section 2.2, 2.3 and 2.4, the required functionality will be analyzed based on the possible resource allocation schemes.

2.2 Message size visible

- Option1: allocating resource according to accurate message size

It will be a relatively efficient allocation scheme if the reader knows the accurate message size before performing the resource allocation. However, the reader could not know the message size from the A-IoT device when allocating resources along with triggering message.

Since the A-IoT traffic is not so complicated, it is proposed SA2 to evaluate whether the uplink message size for corresponding downlink inventory/command could be visible to the reader. The information will be extremely useful for D2R resource allocation.

Proposal 3: RAN2 to ask SA2 whether uplink message size corresponding to downlink inventory/command could be visible to the reader to assist the reader in resource allocation for A-IoT device(s).

2.3 Padding

- Option2: allocating resources for up to maximum message size

In RAN TR 38.848[4], the RAN design targets are defined and the maximum message size to be received or to be transmitted is approximately 1000 bits. Without accurate message size, the reader could also allocate as much resource as possible to cover all the D2R transmission, e.g. resources for 1000bits.

Observation 2: For A-IoT, the maximum message size to be received or to be transmitted is approximately 1000 bits. The reader could allocate resource for transmitting up to 1000bits data to cover all the D2R transmission scenario.

If a certain number of resources will be allocated by the reader, the device should add padding to ensure the D2R packets length consistent with the expected length in the reader side, especially for CRC in RAN1.

Proposal 4: The reader could allocate resources for up to maximum message size. A-IoT device should take padding into account if the actual message size could not match the allocated resources.

2.4 Segmentation

- Option3: allocating resources based on reader implementation

The concept of transmission duration and bandwidth is under discussion in RAN1. Without prior information, the reader can determine the allocated duration and bandwidth based on their own algorithms. From device side, the allocated resource may be not enough to carry the whole message.

Observation 3: Without message size, the reader could allocate resource blindly and it is possible that the allocated resource could not carry the whole message waiting to transmit.

From the perspective of resources utilization, segmentation should be performed to transmit the partial that could be transmitted by the allocated resources.

Proposal 5: Segmentation is necessary if the reader doesn’t know the accurate D2R message size and the allocated resources are not enough to transmit the whole message.

Since there is on HARQ/ARQ, the device can’t retransmit the lost segments. Thus, the reader should discard the whole received segments when lost happens.

Observation 4: Since there is no HARQ/ARQ, the reader should discard the received segments if lost one of them.

Restricted by the A-IoT device capability, the D2R transmission reliability could not be guaranteed very well. Therefore, the less segments, the higher transmission success rate.

Meanwhile, more segments imply multiple scheduling and transmission which will consume more energy of the A-IoT device. The transmission may fail if the energy is not enough to support all the segments transmission.

Proposal 6: RAN2 to evaluate to at most divide the message into two segments to avoid unnecessary energy consumption and loss of segments.

To support accurate resource allocation for the left segment, the device should be able to report the left message size to the reader. Considering complexity, RAN2 has agreed to not support the legacy NR BSR/SR. It is proposed to include the left message size information with the D2R message, e.g. indicated in the header of MAC Data PDU or new AS-layer Data PDU.

Proposal 7: The left message size information is useful for following scheduling and it could be transmitted along with the first segment.

2.4 Reassembly

Generally, reassemble is necessary if there is more than one message transmitted via once resource allocation. DT or Do-DTT could only feedback the downlink inventory/command and there may be only one message for uplink for one downlink triggering. Since DO-A is also in the work list, there could be message waiting for transmission before receiving downlink triggering message. For example, the sensor may have periodical or event-triggered report before receiving the downlink inventory/command. The stored message and required message could be transmitted simultaneously and the AS-layer reassembly should be supported to transmit the two messages via once scheduling. To keep unified design, it is proposed that

Proposal 8: RAN2 to evaluate whether it is a potential case that the A-IoT device will have more than one message waiting for transmit simultaneously. If there are more than one message existing in the device buffer, the reassembly should be considered as one functionality of A-IoT device.

3 Conclusions

In this contribution, we analyze the possible A-IoT device functionalities and made the following observations and proposals:

Observation 1: RAN1 has agreed that the PDRCH transmission should be performed based on PRDCH scheduling information.

Observation 2: For A-IoT, the maximum message size to be received or to be transmitted is approximately 1000 bits. The reader could allocate resource for transmitting up to 1000bits data to cover all the D2R transmission scenario.

Observation 3: Without message size, the reader could allocate resource blindly and it is possible that the allocated resource could not carry the whole message waiting to transmit.

Observation 4: Since there is no HARQ/ARQ, the reader should discard the received segments if lost one of them.

Resource Allocation

Proposal 1: Resource allocation is a necessary A-IoT functionality. RAN2 to study how to allocate resource for A-IoT device supporting different transmission mode, e.g., TDMA, FDMA.

Proposal 2: RAN2 to take resource allocation along with the triggering message as the baseline, and to discuss whether and when to allocate resource separately, e.g transmit the triggering message and perform the resource allocation separately.

Message Size Visible

Proposal 3: RAN2 to ask SA2 whether uplink message size corresponding to downlink inventory/command could be visible to the reader to assist the reader in resource allocation for A-IoT device(s).

Padding

Proposal 4: The reader could allocate resources for up to maximum message size. A-IoT device should take padding into account if the actual message size could not match the allocated resources.

Segmentation

Proposal 5: Segmentation is necessary if the reader doesn’t know the accurate D2R message size and the allocated resources are not enough to transmit the whole message.

Proposal 6: RAN2 to evaluate to at most divide the message into two segments to avoid unnecessary energy consumption and loss of segments.

Proposal 7: The left message size information is useful for following scheduling and it could be transmitted along with the first segment.

Reassemble

Proposal 8: RAN2 to evaluate whether it is a potential case that the A-IoT device will have more than one message waiting for transmit simultaneously. If there are more than one message existing in the device buffer, the reassembly should be considered as one functionality of A-IoT device.

4. References

[1]	R2-24xxxx    Report of 3GPP TSG RAN WG2 meeting #126

[2]	R1-2403821	Report of RAN1#116bis meeting

[3]       R1-24xxxx    Report of 3GPP TSG RAN WG1 meeting #117

[4]	TR 38.848-v18.0.0, Study on Ambient IoT (Internet of Things) in RAN, 2023-09