3GPP RAN WG2 Meeting #127	R2-2407343

# Maastricht, Netherlands, Aug 19th–23rd, 2024

Agenda Item:	8.2.3

Source:	HONOR

Title:	Discussion on A-IoT Paging

Document for:	Discussion and Decision

1. Introduction

In RAN2#126, the following agreements were agreed [1]:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details

In this contribution, we focus on the remaining issues of A-IoT paging.

2 Discussion

- Resources related

For inventory or DL command procedure, the reader needs to send A-IoT paging message to find devices. After receiving A-IoT paging message, device may send random ID or device ID to respond reader. Since RAN1 has agreed to use FDM for D2R message, there may be multiple frequency resources in one access slot. Unlike RFID, dedicated resources could be multiple access occasions in one slot. Thus, the A-IoT paging message can include scheduling information to indicate frequency/time resources for device(s) to respond. To support CBRA and CFRA, the resource information included in A-IoT paging message can be shared resources or dedicated resources. Since RAN1 is discussing the design of the resource, RAN2 can further discuss the parameters included in AIoT paging message after receiving RAN1 conclusions on resource.

Proposal 1: A-IoT paging message includes dedicated resources or shared resources for device(s) to initiate RA procedure. Detailed resource-related parameters depend on RAN1.

The following agreement was made in the last meeting:

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

The CFRA applies to single and multi-device case. For one device case, dedicated resource(s) could be included in one paging message for this device. For multiple device case, multiple dedicated resources could be included in one paging message for these devices.

Proposal 2: Dedicated resource(s) of one or more devices can be included in one A-IoT paging message.

For the case of dedicated resources for multiple devices, device needs to determine which access occasion is allocated to it. Since device uses the ID to identify itself, ID can be also used to determine the specific access occasion. For example, when the configuration of an access occasion associated with a ID is included in A-IoT paging message, device could use the ID to determine the specific access occasion according to the association in the configuration.

Proposal 3: A-IoT paging message includes a mapping between each ID and dedicated resource(s).

- RA type related

It is agreed that CBRA and CFRA are supported and CBRA may contain 2-step RA/3-step RA procedure. If a device is not be able to determine which type of RA procedure should be triggered, there should be an indication of RA type in A-IoT paging message. As the A-IoT paging message format is still under study, this indication could be implicit or explicit.

Proposal 4: RA type, e.g., 2-step/3-step CBRA or CFRA, is provided implicitly or explicitly in A-IoT paging message.

- DL command related

The procedures for inventory + DL command or inventory are identified as following：

1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer.

As the first trigger message and respond are same for these two procedures, a unified A-IoT paging message can be defined. For example, the command is not included in the A-IoT paging for the “inventory and command” case.

For DL command only procedure, since whether the DL command can be included in A-IoT paging message depends on SA2 and SA3 progress, the discussion on format and content of A-IoT paging message for “DL command only” case can be postponed until there is clear definition.

Proposal 5: A unified A-IoT paging message can be defined for inventory only and inventory + DL command procedures.

3 Conclusions

Proposal 1: A-IoT paging message includes dedicated resources or shared resources for device(s) to initiate RA procedure. Detailed resource-related parameters depend on RAN1.

Proposal 2: Dedicated resource(s) of one or more devices can be included in one A-IoT paging message.

Proposal 3: A-IoT paging message includes a mapping between each ID and dedicated resource(s).

Proposal 4: RA type, e.g., 2-step/3-step CBRA or CFRA, is provided implicitly or explicitly in A-IoT paging message.

Proposal 5: A unified A-IoT paging message can be defined for inventory only and inventory + DL command procedures.

4. References

[1]	RAN2 #126 Chairman notes