3GPP TSG-RAN WG2 Meeting #127	R2-2407344

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.2.4

Source:	HONOR

Title: 	Discussion on A-IoT random access

Document for:	Discussion and decision

1	Introduction

The RAN finished the SI for Ambient IoT and approved the TR 38.848 as the outcome of the SI. During the initial discussion related to the Rel-19 Ambient IoT, RAN confirmed the general scope of this SI with some common understandings and limitations related to device type, deployment scenarios and traffic types [1].

During RAN2#125 bis meeting and RAN2#126 meeting, RAN2 discussed the AIoT random access and made several initial agreements as:

- RAN2#125bis: Agreements - RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access - We will study the support for access triggering for a single device, group of devices, or all devices. RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions. - Random Access is triggered by the reader - Reader provides the information that the device needs to respond to the random access trigger. FFS what those parameters are - Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure. FFS the details on each procedure and how we call it. - Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access. FFS details - For the very first access message from the device to reader in random access an ID is included. RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well). - RAN2#126 Agreements on “4 step” RA - A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements - A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request) - The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. - “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. - RAN2 will not use “Msg4” term for further discussion of the random access. Agreements on 2 step CB RA - A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information. - A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed Agreement - From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In this contribution, we analyze the remaining issues of the AIoT random access based on the use cases and assumptions mentioned above and provide proposals related to AIoT random access procedure and the failure handling.

- Discussion

Ambient IoT, which is introduced as a new 3GPP IoT technology, aims to rely on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications and address some representative use cases. AIoT random access is not the same as the NR random access or RFID access procedure since the overall device assumption and the design target are different based on the previous discussions in 3GPP working groups.

In the current NR specification [2], different kinds of random-access procedure are supported: 4-step CBRA as shown in Figure 1(a), 2-step CBRA as shown in Figure 1(b), 4-step CFRA as shown in Figure 1(c), 2-step CFRA as shown in Figure 1(d).

Figure 1: Random Access Procedures in NR

Slotted-ALOHA is employed in RFID to reduce the access collision. The general access procedure for RFID comprises of basic operations like select, inventory, access.

\

Figure 2: General access procedure for RFID

RAN2 confirmed that slotted-ALOHA is the baseline for Ambient IoT random access in the first meeting and analysed the basic steps for 3-step random access CBRA, 2-step CBRA and contention-free access.

2.1 A-IoT random access steps

3-step CBRA, as the baseline access procedure for AIoT, is similar to the 4 step CBRA in NR and the inventory procedure in RFID. During the RAN2#126 meeting, RAN2 agreed that the first two messages were for contention resolution and the device sends the Msg3 to complete the random-access procedure. To take an example of inventory use case, we provide the general procedure for 3-step CBRA based on the current agreements.

Figure 3: General procedure of 3-step CBRA

During last meeting, RAN2 confirmed that the subsequent R2D transmission after Msg3 was not always sent in random access but companies failed reach an agreement about what the message is used for. As mentioned in the agreement, the R2D transmission after Msg3 can be considered to handle the Msg3 transmission failure. Based on our understanding, the device will not perform re-access in the following rounds if it is not aware of whether the Msg3 is not well-received received by the reader. The reader needs to trigger another procedure to find the failed device which results in more signalling overhead and longer latency to complete the service request. In addition, if the failure is informed timely, the failure handling could be performed with less latency. Thus, the R2D transmission after Msg3 could be used to inform and handle the Msg3 transmission failure and provide some performance improvement. For example, the R2D transmission after Msg3 could confirm the failure/success reception of the Msg3 and the absent of it indicates the success/failure reception of Msg3.  For the use case of “inventory and command”, R2D data transmission will be sent to send the command to the device after the Msg3. Thus, the subsequent R2D transmission after Msg3 could be the scheduling information for the following command (e.g., new RN as RFID) or higher layer data (e.g., command). Thus, based on our understanding,

Proposal 1: For 3-step CBRA, the subsequent R2D transmission after Msg3 could be used for the following one or both potential cases:

Confirm the failure/success reception of Msg3.

Scheduling/transmission for the following higher layer data.

For the use case of “inventory only”, the random-access procedure is completed from the point view of the device when it sends Device ID and/or any other upper layer data in Msg3. As mentioned above, to handle the Msg3 transmission failure, subsequent R2D transmission after Msg3 is present to confirm the failure/success reception of the Msg3 while the absent of it indicates the success/failure reception of Msg3. That is, the subsequent R2D transmission after Msg3 is not always present in 3 Step CBRA for the use case of “inventory only”.

Proposal 2: RAN2 confirms that the subsequent R2D transmission after Msg3 is not always present in 3 Step CBRA (e.g., present to confirm the failure reception of the Msg3 while the absent of it indicates the success reception of Msg3).

Similar to the 3-step CBRA, the first two steps are also used for contention resolution for the 2-Step CBRA. In another word, the step 2 in 2-step CBRA needs to be mandatory to confirm the reception of the Msg1. Thus, the present of Msg2 means the Msg1 is well received from the perspective of device.

Proposal 3: For 2 Step CBRA, the Msg2 is always present to indicate whether the Msg1 is well received.

CFRA procedure for AIoT could be employed by the reader when the reader is aware of the specific device(s) to respond. In this case, the device could send the device ID and/or upper layer data in the first message with the dedicated occasion/resource(s) assigned by the reader.

Proposal 4: For CFRA, device determines its dedicated access occasion/resource and sends the device ID and/or any other upper layer data in the first message.

For 2 step CBRA, Msg2 is used for collision resolution which need to be always present. But for 2-step CFRA, the device with dedicated access occasion/resource will not collides with other devices. Thus, it is not necessary to keep Msg2 as mandatory for 2-step CFRA.

Proposal 5: For CFRA, the “Msg2” is not always present to echo the Msg1.

The Msg1 in CFRA is sent by the device without contention. Similarly, the Msg3 in 3-step CBRA is only sent after the AIoT device considers the contention resolution as successful. The “Msg2”, i.e., subsequent R2D transmission after Msg1, is similar to the subsequent R2D transmission after Msg3 in 3-step CBRA, which could be used to inform and handle the Msg1 transmission failure.  If there is following data transmission, the subsequent R2D transmission after Msg1 could also be employed for scheduling/transmission of the following higher layer message.

Proposal 6: For CFRA, the subsequent R2D transmission after Msg1 could be used for the following one or both potential cases:

Confirm the failure/success reception of Msg1.

Scheduling/transmission for the following higher layer message.

2.2 Failure Detection and handling

As discussed before, for 3-step CBRA, the device considers the contention resolution as successful only when the reader ack the same ID received in Msg1. Similarly, the device considers the contention resolution as failed, if the Msg2 including the same ID in Msg1 is not received before the next occasion/slot. It could not distinguish the cause of the failure: collision with other devices or Msg1 decoding problem. From the perspective of reader, it may consider the Msg2 transmission as failed if there is no Msg3 from the device. Thus, to complete the whole random access procedure, the failure determination needs to be from perspective of both reader and device.

Proposal 7: RAN2 to study the failure detection from perspective of both reader and device.

In addition to contention resolution, the subsequent R2D transmission after Msg3 for 3-step CBRA or Msg1 for CFRA could be detected as failure. What’s more, considering the limited power for the AIoT devices, the failure may be caused by the un-availability. The handling should be studied for all the failure cases mentioned above.

Proposal 8: RAN2 to consider the following failure cases to handle: contention resolution failure, the subsequent R2D transmission failure, failure related to availability due to power.

3	Conclusions

In this contribution, we discussed the issues related to the random access procedure and failure handling aspects on Ambient IoT. Based on the  discussion, the following observation and proposals are concluded:

Proposal 1: For 3-step CBRA, the subsequent R2D transmission after Msg3 could be used for the following one or both potential cases:

Confirm the failure/success reception of Msg3.

Scheduling/transmission for the following higher layer data.

Proposal 2: RAN2 confirms that the subsequent R2D transmission after Msg3 is not always present in 3 Step CBRA (e.g., present to confirm the failure reception of the Msg3 while the absent of it indicates the success reception of Msg3).

Proposal 3: For 2 Step CBRA, the Msg2 is always present to indicate whether the Msg1 is well received.

Proposal 4: For CFRA, device determines its dedicated access occasion/resource and sends the device ID and/or any other upper layer data in the first message. Proposal 5: For CFRA, the “Msg2” is not always present to echo the Msg1.

Proposal 6: For CFRA, the subsequent R2D transmission after Msg1 could be used for the following one or both potential cases:

Confirm the failure/success reception of Msg1.

Scheduling/transmission for the following higher layer message.

Proposal 7: RAN2 to study the failure detection from perspective of both reader and device.

Proposal 8: RAN2 to consider the following failure cases to handle: contention resolution failure, the subsequent R2D transmission failure, failure related to availability due to power.

4	References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR.

- TR 38.300