3GPP TSG-RAN WG2 Meeting #127	R2-2407346

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.8.4

Source:	HONOR

Title: 	Discussions on the support of broadcast service

Document for:	Discussion and decision

1	Introduction

During the previous meetings for Rel-19 NTN, RAN2 clarified use cases of the broadcast service in NTN system and made some basic agreements as follows.

Agreements: - RAN2#125bis: - For MBS broadcast service we don’t restrict the work to any satellite constellation type - We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS - We will cover at least the case where the indicated intended service area covers a portion of a NTN cell - The intended service area can cover the area of more than one NTN cells (or portions thereof) - Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area) - At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area - RAN2#126: - For MBS broadcast service, both EFC and EMC are supported. - RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. - MBS broadcast intended service area is provided via system information - FMBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

In this contribution, we would further discuss how to provide the broadcast service via NTN based on the previous agreements.

- Discussion

Based on the current MBS procedure, a UE can receive a broadcast communication service in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. For now, the ETWS is supported by NR connected to 5GC by means of system information broadcast capability. What is more, a UE not registered even could receive some kind of service via broadcast. In conclusion, it is scarcely possible for the NW to limit the reception of the UE. Besides, NW is only aware of the coarse location of the UE in RRC_IDLE, RRC_INACTIVE state. The RRC_IDLE/RRC_INACTIVE UE could re-select the camping cell within the preconfigured tracking area/RNA and only triggers the registration update when the new camped cell does not belong to the tracking area/RNA. The NW was not sure which RRC_IDLE/RRC_INACTIVE UE is out of the intended service area and is not supposed to receive the broadcast service. One potential solution is to provide the intended service area to the UE and the UE verified the service availability based on its location.

Proposal 1: The intended service area could be provided to UE for availability verification based on its own location.

In the TN, the coverage is formulated with the circles-like format since the coverage area is a circle around the base station. Since the intended area is the related to the service but not the coverage of the network, it is hard to determine the reference point for the centre and radius with the circles-like format. At least, the reference point for the centre is not toggled by the coverage centre of the NTN cell. For the broadcast service, the area is usually divided by administrative regions. As stated in the agreement, geographical area information, e.g., via polygons, could better approximate the intended shape of service area. Thus, we prefer the geographical area information as the baseline area format.

Proposal 2: The intended service area could be provided to UE in the format of explicit geographical area.

The intended service area related information could be sent along with the broadcast service data or broadcast related configurations. Thus, the content of the service-related SIBs could be extended by mapping different services with different intended service areas. Thus, explicit geographical area or index mapping to the pre-configured geographical area could be potential information to be carried in the system information.

Proposal 3: For MBS broadcast, one intended service area could be associated with multiple broadcast services and one broadcast service could be associated with multiple intended service areas.

As agreed in the RAN2#126 meeting, service area information might be included in SIB20/ SIB21/ MBSBroadcastConfiguration for MBS broadcast with FFS for ETWS. For PWS, the related system information blocks are SIB6 which contains an ETWS primary notification and SIB7 which contains an ETWS secondary notification.

Proposal 4: For ETWS, the intended service area related information could be provided to UE via system information, and the specific SIB(s) could be SIBs related to broadcast service (e.g., SIB6/7).

3	Conclusions

In this contribution, we discussed the issues related to intended service area in NTN. Based on the discussion, the following proposals are concluded:

Proposal 1: The intended service area could be provided to UE for availability verification based on its own location.

Proposal 2: The intended service area could be provided to UE in the format of explicit geographical area.

Proposal 3: For MBS broadcast, one intended service area could be associated with multiple broadcast services and one broadcast service could be associated with multiple intended service areas.

Proposal 4: For ETWS, the intended service area related information could be provided to UE via system information, and the specific SIB(s) could be SIBs related to broadcast service (e.g., SIB6/7).

4	References