3GPP TSG-RAN WG2 Meeting #127	R2-2407349

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.3

Source:	HONOR

Title: 	Discussion on measurement event evaluation

Document for:	Discussion and decision

1	Introduction

For mobility enhancement WID in the Release 19[1], the following agreements related to L1 measurement enhancement were made during last RAN2 meeting.

- Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

The L1 measurement enhancements are related to scenario extension, event triggered measurement reports and CSI-RS measurements. In this contribution, we give our initial thinking about the L1 measurement event evaluation.

- Discussion

In Rel-18 mobility enhancement WI, L1/L2 triggered mobility measurement (LTM) is introduced as a cell switch procedure while the gNB may trigger the procedure after receiving the L1 measurement report from UE. Enhancements on L1 measurement only realized the basic function with the support of the SSB-based L1 measurement for neighbour cells in the intra CU scenario. For the Rel-18 LTM, the legacy CSI report mechanism is reused as much as possible. All SSBs from all candidate cells are numbered so that the report only needs to include the SSBRI. Compared to the fixed number in the legacy report for the serving cell (and potential the neighbour cell as the additional PCI), the number of SSBRI reported in one CSI report is modified to L*M where L is the number of reported cells and M is the number of reported SSBRI/RSRP pair per cell. And the selection of the reported beam is up to UE implement.

Observation 1: In Rel-18, the CSI report only includes RS index and the corresponding RSRP for the SSB-based L1 measurement in intra-CU scenario.

For Rel-19, the applicable scenario is extended to inter-CU MCG/SCG in addition to intra-CU MCG/SCG in Rel-18. And, the discussion about CSI-RS based L1 measurement is re-triggered in Rel-19 since the study of Rel-18 LTM only finished the SSB-based L1 measurement and measurement report. What is more, event triggered enhancement is introduced for triggering L1 measurement reporting and conditional LTM execution. Considering the extension of scenarios and RS type mentioned above, impacts on the configuration, performing measurements and measurement reports are not evitable. Enhancements related to need to be introduced with the balance of impacts on the spec and availability/effectiveness. Separate discussions on the three sub-objectives may lead to some overlaps. We prefer a unified solution for L1 measurement enhancement in Rel-19 which covers event triggered L1 measurement reporting and LTM execution, SSB or CSI-RS based L1 measurement and both intra-CU and inter-CU scenario. That is, the UE could evaluate conditions for triggering LTM and the L1 measurement report by SSB-based L1 measurement or CSI-RS based L1 measurement in both inter-CU and intra-CU MCG/SCG scenario.

Proposal 1: RAN2 strives to have a unified solution for L1 measurement enhancements in Rel-19 covering scenario extension for both inter/intra-CU MCG/SCG LTM with SSB based or CSI-RS based L1 measurement on candidate cells.

Proposal 2: Event triggering L1 measurement report could be reused for the evaluation of the conditional LTM execution.

RAN2 already confirmed that the event triggered L1 measurement is designed for LTM early synchronization and LTM cell switch procedure. Before the LTM cell switch execution, L1 measurement is one of the important parts in the preparation phase. Considering the L1 measurement is performed for candidate cells for both inter/intra-CU LTM, the suspend and trigger of the L1 measurement is one of the potential use cases for the event-triggered L1 measurement report. Especially, CSI-RS based L1 measurement causes lots of signalling overhead and power consumption, it is not necessary to perform the L1 measurements for all candidates when the serving cell becomes good enough or for all RS types when the candidate cell is not good enough.

Proposal 3: Event triggered L1 measurement should be designed for the one additional LTM purposes: Select the target beam/RS and trigger L1 measurement.

And CSI-RS could be configured in L1 measurement resource configuration for LTM. For the report criterion of intra-RAT L3 measurement, A1~A5 events are introduced as the trigger condition for the measurement events. For L1 measurement, Event LTM2~5  have been confirm during RAN2#126 meeting. However, RAN2 left the event LTM1 open since some companies haven’t found the use case for the good enough event. We could see that different events could be applied in different scenario. For example, event LTM1 indicates the good quality of the serving cell which could be used to trigger the suspend of the inter-cell measurement. For event LTM2, when the serving cell becomes worse than absolute threshold, the NW could trigger the UL/DL sync to prepare for the following cell switch or just configure the UE with event LTM3/5. Similarly, when the serving cell becomes better than absolute threshold, the NW could suspend UL/DL sync since the serving cell is good enough to maintain the connection. Event LTM1 could be employed for the use case mentioned above.

Proposal 4:Introduce LTM1 events to trigger the L1 measurement reporting for cell switch.

We could specify multiple events for L1 measurement in addition to A3/A5 like events since reports triggered by different events could provide different kind of information. Thus, to achieve different effects via different events, multiple events may be configured to one beam which is similar to the L3 measurement report. Furthermore, in CHO, two conditions associated with one event could be configure to UE and the event is considered as fullfilled only when both conditions are met, which improve the robustness for the handover. However, take the latency caused by the event evaluation into account, it still needs some discussions about whether one event could be associated with two conditions could be introduced into L1 measurement event evalutaion.

Proposal 5: Multiple events may be configured simultaneously for one beam.

3	Conclusions

In this contribution, we discussed the L1 measurement enhancement for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Observation 1: In Rel-18, the CSI report only includes RS index and the corresponding RSRP for the SSB-based L1 measurement in intra-CU scenario.

Proposal 1: RAN2 strives to have a unified solution for L1 measurement enhancements in Rel-19 covering scenario extension for both inter/intra-CU MCG/SCG with SSB based or CSI-RS based L1 measurement on candidate cells.

Proposal 2: Event triggering L1 measurement report could be reused to evaluate the condition for LTM execution.

Proposal 3: Event triggered L1 measurement should be designed for the one additional LTM purposes: Select the target beam/RS and trigger L1 measurement.

Proposal 4: Introduce LTM1 events to trigger the L1 measurement reporting for cell switch.

Proposal 5: Multiple events may be configured simultaneously for one beam.

4	References

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Rel-19,