3GPP TSG-RAN WG2 Meeting #127	R2-2407350

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.4

Source:	HONOR

Title: 	Discussion on L1 measurement enhancements

Document for:	Discussion and decision

1	Introduction

For mobility enhancement WI in the Release 19[1], the following agreements related to L1 measurement enhancement were made during last RAN2 meeting.

- Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

The L1 measurement enhancements are related to scenario extension, event triggered measurement reports and CSI-RS measurements. In this contribution, we focus on the L1 measurement reporting and provide some proposals for the format and contents for the report accordingly.

- Discussion

For L3 measurement, multiple RS types could be included in one report based on measurements with different RS type. For L1 measurement, RAN2 already confirmed the support for the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. The content of the L1 measurement should be discussion when both SSB and CSI-RS are configured in L1 measurement resource configuration.

For event LTM2 and event LTM4, the L1 measurement report includes the result(s) for the serving cell or candidate cell. Considering the limited size of the L1/L2 signalling, measurement results based on different RS type in one L1 measurement report may introduce more complexity for the design of the signalling format. For event LTM3 and event LTM5, the L1 measurement report includes the result(s) for one candidate cell and serving cell. Based on our proposal in [another contribution], the comparison needs to be done based on the same RS type. Thus, we propose that

Proposal 1:  Measurement results based on same RS type can be included in one L1 measurement report.

Unlike the SSBRI in the Rel-18 L1 measurement report and the indexes reported in L3 measurement, based on different L1 measurement configuration in the inter-CU scenario, the associated index could be different. For example, similar to the SSBRIs in Rel-18 LTM L1 measurement report, RS among cells in intra-DUs intra-CU are unified numbering. If the RS index is configured following the mechanism in Rel-18, we may need to extend the number of the candidate beams to cover the candidates from both intra-CU and inter-CU. Another solution is to take the same mechanism as Rel-18 which means the RSs are unified numbering among the CU. Last but not least, format similar to L3 measurement report may also works fine based on detail information like candidate configuration index and RS index.

Proposal 2: RAN2 to discuss the two formats of the beam related information:

- beam index based on unified numbering between CUs.

- Candidate configuration index and beam index.

For the reporting content, except the L1 measurement result per beam and the associated RS index that UE includes in the L1 measurement report, other associated information could also be needed. For example, the information of the triggered event could provide more information of the beam satisfying the L1 measurement event. For this meeting, we propose to only confirm the potential contents for the event triggered L1 measurement reporting as follows:

Proposal 3: RAN2 to confirm the at least following contents for the event triggered L1 measurement report: beam information, measurement results per beam, event index, RS type.

RAN1 had listed the signalling content of the UE-initiated/event-driven beam reporting with variable or fixed number of beam(s) satisfying the event or at least one of the beams satisfying the event:

Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, the following options are provided for down-selection (other options are not precluded) in RAN1#117 - Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - The N beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - FFS: Whether the indication of payload size should be provided additionally. - Option-1a (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - At least one of N reported beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - FFS: Whether the indication of payload size should be provided additionally. - FFS: Details on how value of N is determined by the UE - Option-1b: N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - The N beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - Payload size does not vary as a function of N - FFS: Zero-padding can be provided if N is less than Nmax. - Option-2: Only N=1 beam is reported in the report instance - The reported beam should satisfy the condition of Event-2 - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - Other options are not precluded. - FFS: Whether the measurement results for current beam is always reported or can be enabled by RRC. - FFS: When current beam is reported, whether the current beam is counted in the N reported beams. - The selected option shall satisfy Event-2.

Similarly, it is reasonable to have these beams included in the L1 measurement report. Firstly, beam(s) of neighbour satisfying the event is the basic option since it is useful to help the NW determine the target beam. Secondly, the quality of the serving beam is needed when determining cell switch since the determination should be made based on the comparison between the serving and the neighbour. Considering the variety of L1 measurement, the result of the serving beam could be carried in the same report instance with the result of beam(s) of neighbour satisfying the event. Last but not least, beams of neighbour with at least one beam satisfying the event is another option for the signalling content. As the current L3 measurement and report mechanism, the UE measures several beams to derive the cell quality to ensure that UE could receive good service under the cell with more good beams. Even the LTM is based on beam measurement, it provides more robust if the target has multiple good beams within the cell or CU.

Proposal 4: RAN2 to confirm the following beams to be included in the report: the beam(s) of neighbour satisfying the event, serving beam, beams of neighbour with at least one beam satisfying the event.

Considering the UE-initiated/event-driven beam report for beam management, RAN1 listed the potential issues such as the general procedure, signalling for the beam report transmission, signalling content in the first R19 meeting. During RAN1#116 meeting, they further studied the UCI-based L1 measurement report and agreed the following modes for the beam report transmission procedure:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details.

During the RAN1#117 meeting, companies focused on the Mode A solution and made some progress. However, the intentions are not exactly the same between the MIMO WI and Mobility WI. Rel-19 MIMO focuses on the beam management while Rel-19 Mobility is designed for the cell switch. Considering the large amount of potential information carrying in the event triggered L1 measurement reporting, we propose to take the MAC CE as the baseline for the L1 measurement report in Rel-19 mobility WI.

Proposal 5: Confirm that MAC CE carrying L1 measurement report is the starting point for RAN2. FFS UCI based beam report from RAN1 could be reused.

3	Conclusions

In this contribution, we discussed the L1 measurement reporting for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1:  Measurement results based on same RS type could be included in one L1 measurement report.

Proposal 2: RAN2 to discuss the three formats of the beam information:

- beam index based on unified numbering between CUs.

- Candidate configuration index and beam index.

Proposal 3: RAN2 to confirm the at least following contents for the event triggered L1 measurement report: beam information, measurement results per beam, event index.

Proposal 4: RAN2 to confirm the following beams to be included in the report: the beam(s) of neighbour satisfying the event, serving beam, beams of neighbour with at least one beam satisfying the event.

Proposal 5: Confirm that MAC CE carrying L1 measurement report is the starting point for RAN2. FFS UCI based beam report from RAN1 could be reused.

4	References

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Rel-19,