3GPP TSG-RAN WG2 Meeting #127	R2-2407352

Maastricht, NL, Aug 19th – 23rd, 2024

Agenda item:	8.5.4

Source:	HONOR

Title:	Discussion on adaptation of common signal/channel transmissions

Document for:	Discussion and Decision

- Introduction

New Rel-19 work item on enhancements of network energy savings was approved at RAN#102 and revised at RAN#103 [1]. One of the objectives in the WID is to specify adaptation of common signal/channel transmissions:

Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

At RAN2#125bis meeting, paging adaptation was discussed and the following agreements were achieved [2]:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle. - For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. - For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

This objective was also discussed at RAN1#117and some agreements were made [3]. In this document, we focus on potential RAN2 impact based on RAN1 and RAN2 conclusion.

- Discussion

2.1 Adaptation of SSB

The following RAN1 agreements for SSB adaption were made in the last RAN1 meeting:

Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

For SSB adaption, One or more SSB burst periodicity value(s) has already agreed from RAN1.It seems only RAN2 impact that can be identified for the time being is corresponding configuration. Other RAN2 impact, if any (e.g. trigger mechanism), can be studied and discussed after further RAN1 progress.

Proposal 1: Wait for further RAN1 progress before discussing RAN2 impact of SSB adaptation.

2.2 Adaptation of PRACH

The following RAN1 agreements for PRACH adaption were made in the last RAN1 meeting:

Agreement For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least: - a PRACH configuration index - FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources Study further the following - When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource, - Additional parameter(s) for determining the additional PRACH resources e.g. - Scaled/adjusted PRACH configuration period - Additional timing offset - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Muting/masking ROs - When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource - Additional mechanisms (if any) for determining the additional PRACH resources e.g. - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) Agreement For the adaptation mechanism for additional PRACH resources, study further the following: - Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation - Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Strive to re-use existing DCI format(s) - Option 3: Adaptation of PRACH transmission according to predefined condition(s) - FFS: details - Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation - Strive to re-use existing DCI format(s) - Option 5: Enhanced cell DRX

For PRACH adaption, several mechanisms were listed for further study, which can be categorised as bellow:

- New configuration for PRACH, e.g. PRACH configuration index, Additional parameter(s) for determining the additional PRACH resources, etc.

- Dynamic adaption for PRACH configuration/occasion and corresponding trigger mechanisms.

Apart from corresponding configuration, for some PRACH adaption mechanism, there will be RAN2 procedure impacts. For example, PRACH resource selection for NES-UEs and UE request for PRACH adaption (if L2 signaling is needed). According to RAN1 agreements, all the listed mechanisms are for further study, RAN2 can start to discuss detailed RAN2 impacts after RAN1 makes down selections on the listed mechanisms.

Proposal 2: Wait for further RAN1 progress before discussing RAN2 impact of PRACH adaptation.

2.2 Adaptation of Paging

For paging adaptation, two options were agreed at RAN2#125bis:

- Option 1: bundle PFs in one paging DRX cycle in time domain.

- Option 2: extend interval between PFs and increase POs per PF to avoid decreasing paging capacity per paging DRX cycle.

Both options can restrict all POs in one paging DRX cycle to limited time domain for gNB energy saving. In Option 2, for each paging DRX cycle, the number of PFs is decreased but the number of POs per PF is increased. There may be impact on gNB scheduling flexibility for paging transmission as the gNB may need to schedule more paging transmission in one PF. Besides, increasing POs per PF may have impact on features linked to PO. For example, one DCI 2_7 for PEI can be associated with up to 8 POs of two consecutive PFs. Increasing POs per PF may have impact on PEI reception procedure. Thus, Option 1 is preferred.

In TS38.304, PF is determined by the following formulae:

The PF and PO for paging are determined by the following formulae: SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) Index (i_s), indicating the index of the PO is determined by: i_s = floor (UE_ID/N) mod Ns …… T: DRX cycle of the UE. …… N: number of total paging frames in T Ns: number of paging occasions for a PF

One straightforward way to bundle PFs to certain time domain within one paging DRX cycle is to update the formula for PF determination.

Proposal 3: For NES-UEs, revise the legacy PF determination formula to bundle PFs in one paging DRX cycle.

- Conclusion

In this contribution, we focus on potential RAN2 impacts of adaptation of common signal/channel transmissions for network energy saving. Proposals are summarised as follows.

Proposal 1: Wait for further RAN1 progress before discussing RAN2 impact of SSB adaptation.

Proposal 2: Wait for further RAN1 progress before discussing RAN2 impact of PRACH adaptation.

Proposal 3: For NES-UEs, revise the legacy PF determination formula to bundle PFs in one paging DRX cycle.

- References

- RP-240170, Revised WID for Enhancements of network energy savings for NR.

- R2-2403731, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)

- Draft Report of 3GPP TSG RAN WG1 #117 v0.2.0