3GPP RAN WG2 Meeting #127	R2-2407354

# Maastricht, Netherlands, Aug 19th–23rd, 2024

Agenda Item:	8.7.4

Source:	HONOR

Title:	Discussion on UL scheduling enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#126, the following agreements were agreed [1]:

For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH We try to avoid RAN1 impacts. RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial The solutions should not disallow non-delay critical data from using an UL grant. LCP prioritization within a logical channel will not be considered in RAN2 discussions Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

In this contribution, we focus on the UL scheduling enhancements with delay information.

2 Discussion

2.1 The delay-critical data transmission

- LCH priority

In the last meeting, it was agreed to use additional priority configured to LCHs in case of these LCHs with delay-critical data. When the LCH becomes delay-critical, the additional priority should be higher than the legacy one to improve scheduling priority.

Proposal 1: A additional higher priority is configured to LCHs in case of these LCHs with delay-critical data.

There may be delay-critical data and non-delay critical data in one LCH at the same time. If the additional priority is configured to the LCH, there are two options to use it:

Option 1: the priority only applies to delay-critical data within the LCH.

Option 2: the priority applies to the whole LCH.

With the first option, UE needs to perform two rounds of LCH prioritization for selected UL grant, i.e. it first allocates resources for delay critical data from all logical channels based on additional priority, then repeats the same procedure with non-delay critical data based on legacy priority. Meanwhile, it is agreed LCP prioritization within a logical channel will not be considered in RAN2 discussions. Thus, the option 2 can be adopted.

Proposal 2: The additional priority applies to the whole LCH in case of the LCH with delay-critical data.

- LCH mapping restrictions

The current LCH mapping restrictions control the LCH to use suitable grants for its traffic. For example, if there are more than one set of grants configured to UE and the LCH1 can use the first set of grants according to LCH restrictions. There could be a case that the first set of grants are not enough to fill data burst available in LCH1. While waiting for the next available grant, the remaining data may become delay-critical data. These data will be discarded if the next available grant is not arrived in time. Therefore, the LCH restrictions could be temporarily relaxed for the LCH1 to use other UL grants for remaining delay-critical data.

Proposal 3: LCH mapping restrictions configured to LCH can be relaxed in case of the LCH with delay-critical data.

LCH mapping restrictions include the following parameters:

- allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission; - maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission; - configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission; - allowedServingCells which sets the allowed cell(s) for transmission; - allowedCG-List which sets the allowed configured grant(s) for transmission; - allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission; - allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The above SCS, PUSCH duration and CGtype1 is introduced for URLLC traffic, which can be extended to XR traffic. These parameters can guarantee packets transmitted on shorter TTI and with higher reliability, which may not be simply relaxed. Therefore, RAN2 can evaluate which LCH mapping restrictions can be relaxed.

Proposal 4: RAN2 evaluates which LCH mapping restrictions can be relaxed.

- PDUs transmission in RLC

For UL data transmission in one LCH, if notified of a transmission opportunity by the lower layer, the transmitting UM RLC entity shall submit the UMD PDU to lower layer. It is up to UE implementation that the UM RLC entity can submit UMD PDU considering delay information, e.g. based on the packet arrival order.

For the AM RLC entity, it should consider the PDU type when submitting PDUs to lower layer, i.e.：

- control PDU is prioritized over AMD PDU

- AMD PDU containing retransmitted RLC SDU(s) is prioritized over AMD PDU containing new RLC SDU(s).

Therefore, there is no packet delay information considered in current UL data transmission.

For RLC AM mode and RLC UM mode, when the PSI-based discard is active, the lower important PDU set data arrived formerly may be transmitted first and filled in UL resources. If UL resources are not enough, the higher important PDU set data with low remaining time may be delayed transmitting and further be discarded. And allocated UL resources are expected to be used by the higher important PDU set data in such case. Therefore, at least for the case PSI-based discard is active, it is reasonable to consider delay information for efficient scheduling when submitting the PDUs to lower layer.

Proposal 5: At least for PSI-based discard is active, the AMD PDU with low remaining time can be transmitted prior to other AMD PDU in RLC.

2.2 DSR enhancements

In the last meeting, it was agreed to enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. In current specification, DSR is triggered when the shortest remaining time of data for LCG is below remainingTimeThreshold. If enhanced DSR is configured, it is simple and work well that the legacy threshold can be reused for triggering enhanced DSR. The specification is as the following text:

1> if the smallest remaining value of the running PDCP discardTimers among all the SDUs buffered for the LCG that has not been transmitted in any MAC PDU and has not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR or multiple DSR pending for the LCG: 2> if multiple DSR is configured to report: 3> trigger a multiple DSR for the LCG. 2> else: 3> trigger a DSR for the LCG.

Proposal 6: The legacy remainingTimeThreshold and triggering condition are reused for multiple DSR. 
In additional, the content of multiple DSR can be further discussed. The buffer size whose remaining time is below a reporting threshold is reported in multiple DSR, reporting thresholds can include remainingTimeThreshold. Reporting threshold can be obtained from the network or a predefined rule. Current value r of remaining time indicates a remaining time within the range of (r, r + 1] msec, there may be a method to report the shortest few pairs of information for each LCG by using current format of remaining time.

Proposal 7: Multiple DSR includes the buffer size whose remaining time is below a reporting threshold.

Proposal 8: RAN2 discusses how to obtain reporting thresholds:

- configuring by the network;

- using a predefined rule, e.g. report the shortest few pairs of information for each LCG by using current format of remaining time.

3 Conclusions

Proposal 1: A additional higher priority is configured to LCHs in case of these LCHs with delay-critical data.

Proposal 2: The additional priority applies to the whole LCH in case of the LCH with delay-critical data.

Proposal 3: LCH mapping restrictions configured to LCH can be relaxed in case of the LCH with delay-critical data.

Proposal 4: RAN2 evaluates which LCH mapping restrictions can be relaxed.

Proposal 5: At least for PSI-based discard is active, the AMD PDU with low remaining time can be transmitted prior to other AMD PDU in RLC.

Proposal 6: The legacy remainingTimeThreshold and triggering condition are reused for multiple DSR.

Proposal 7: Multiple DSR includes the buffer size whose remaining time is below a reporting threshold.

Proposal 8: RAN2 discusses how to obtain reporting thresholds:

- configuring by the network;

- using a predefined rule, e.g. report the shortest few pairs of information for each LCG by using current format of remaining time.

4. References

[1]	RAN2 #126 Chairman notes