3GPP RAN WG2 Meeting #127	R2-2407355

# Maastricht, Netherlands, Aug 19th–23rd, 2024

Agenda Item:	8.7.5

Source:	HONOR

Title:	Discussion on RLC enhancements

Document for:	Discussion and Decision

1. Introduction

In this contribution, we discuss the potential solutions related to RLC enhancements for avoiding unnecessary retransmissions and ensuring timely RLC retransmissions.

2. Discussion

2.1 Avoiding unnecessary retransmissions

The following agreements were made for avoiding unnecessary retransmissions:

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

For the current RLC AM mechanism, once the RLC PDU is submitted to lower layer, it cannot be discarded when the PDCP discard the corresponding PDCP SDU. Therefore, these obsolete packets may be retransmitted in radio link. The above agreements identify clear procedures for Tx initiated approach and Rx initiated approach to solve this issue. And the FFS is how Rx side determines that an SDU should be abandoned. There are some solutions provided by companies [1]:

- Option 1: using a discard timer in Rx side, the SDU is assumed discarded when such timer expires.

- Option 2: using maximum number of retransmissions in Rx side, the SDU is assumed discarded when such number is reached.

- Option 3: according to Rx PDCP side indication when t-reordering timer expires.

For the option 2, since that SDU may not be discarded in PDCP, it cannot be assumed that the corresponding SDU is actually discarded or unnecessary. For the option 1 and option 3, PDCP will not deliver those unreceived SDUs whose COUNT value is less than the updated RX_DELIV when t-reordering timer expires. So the RLC does not need to retransmit those SDUs. Though Rx side method works for the case where PDCP cross-deliver but RLC still retransmits, the discarded SDU in PDCP Tx side may continue retransmitting in radio link. And one solution is that the PDCP Rx side can use PDCP SN gap report to indicate RLC Rx side which packet is discarded. But this method may not be timely due to this report is triggered when discarded SDU is not submitted to RLC Tx side.

For the Tx side method, RLC can determine that a SDU is discarded when the discard indication is received from PDCP. RLC discards this SDU regardless of whether this SDU is submitted to lower layer. Meanwhile, the RLC Tx side notifies the receiving RLC side about the obsolete SDUs. Therefore, the Tx side method is more accurate and timelier.

Proposal 1: Apply Tx initiated approach for avoiding unnecessary RLC AM retransmissions.

2.2 Timely RLC retransmissions

The following agreements were made for timely RLC retransmissions:

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

Current RLC retransmission is triggered by status report which needs to be triggered by transmitter polling or PDU loss (SN gap). Those procedures will bring some delay of XR services. In the last meeting, there were provided three solutions to study and one is autonomous retransmission without status report.

There are two methods for autonomous retransmission without status report:

- Method 1: A timer can be introduced to control the RLC retransmission, e.g. the retransmission of the data can be triggered when the timer expires.

- Method 2: Current indicator from PDCP to RLC to notify delay-critical RLC SDU is used to trigger the corresponding RLC SDU retransmission.

For Method 1, a timer is introduced in RLC for retransmitting SDU. RLC entity starts the timer when receiving the RLC SDU. When the timer expires and the SDU has been sent, RLC retransmits the corresponding SDU. This timer is shorter than PDCP discard timer for avoiding retransmitting discarded SDU.

Fort Method 2, if a PDCP SDU has low remaining time, and if the corresponding PDCP Data PDU has already been submitted to lower layers, the delay-critical indication for the PDCP Data PDU is provided to RLC. This indication means the corresponding RLC SDU is about to time out, RLC can trigger the corresponding SDU retransmission when receiving the indication.

For the above two methods, both can be used to trigger timely RLC retransmission but method 2 reuses the current mechanism. So, RAN2 can apply method 2.

Proposal 2: Current indication of delay-critical RLC SDU from PDCP to RLC is reused to trigger the corresponding RLC SDU autonomous retransmission.

After the delay-critical RLC SDUs are retransmitted, the poll for these SDUs can be transmitted for confirming whether these SDUs are successfully received. Therefore, the polling can be enhanced considering delay-critical RLC SDUs.

Proposal 3: Enhance polling based on the delay-critical data.

3. Conclusions

Proposal 1: Apply Tx initiated approach for avoiding unnecessary RLC AM retransmissions.

Proposal 2: Current indication of delay-critical RLC SDU from PDCP to RLC is reused to trigger the corresponding RLC SDU autonomous retransmission.

Proposal 3: Enhance polling based on the delay-critical data.

4. References

[1]	RAN2 #126 Chairman Notes