3GPP RAN WG2 Meeting #127	R2-2407356

# Maastricht, Netherland, Aug 19th – 23rd, 2024

Agenda Item:	8.7.2

Source:	HONOR

Title:	Discussion on multi-modality support

Document for:	Discussion and Decision

1. Introduction

In RAN#103 meeting, the following objectives of Rel-19 XR work item were agreed [1]:

Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption.

In RAN2#126 meeting, the following agreements on XR Multi-modality have been achieved [2].

- Support Multi-Modality awareness in RAN in Rel-19 for UL and DL. - Ask SA2 whether/what information could be provided to RAN - We clarify that we are still in study phase and that we need to understand whether/what information we can get from SA2 (e.g. MMSID, synchronization thresholds) to conclude the study.

In this contribution, we will discuss how to identify whether the Multi-Modal service fulfils the synchronization requirement.

2. Discussion

2.1. General

RAN2 has agreed to support Multi-Modality awareness in RAN in Rel-19 for UL and DL in #126 meeting and sent LS to SA2 to ask whether/what information could be provided to RAN (e.g. MMSID, synchronization thresholds).

For XR applications, synchronization between different media components is critical to user experience. Lack of synchronization can cause users to become dizzy. Packets that have exceeded the synchronization thresholds are useless to users. Discarding these packets is also beneficial to increase system capacity. So, awareness of synchronization thresholds in RAN and UE is very important for the user experience.

In addition, we think how to identify whether multimodal services fulfil the synchronization thresholds also needs to be discussed.

2.2. Multi-modality support

From our perspective, UE and gNB could detect synchronization status by the field “timestamp” in the RTP header. The following figure shows the structure of the RTP header.

The description in the box below is the definition of timestamp, which quotes from RFC1889[3].

The timestamp reflects the sampling instant of the first octet in the RTP data packet. The sampling instant must be derived from a clock that increments monotonically and linearly in time to allow synchronization and jitter calculations. The resolution of the clock must be sufficient for the desired synchronization accuracy and for measuring packet arrival jitter (one tick per video frame is typically not sufficient). The clock frequency is dependent on the format of data carried as payload and is specified statically in the profile or payload format specification that defines the format or may be specified dynamically for payload formats defined through non-RTP means. If RTP packets are generated periodically, the nominal sampling instant as determined from the sampling clock is to be used, not a reading of the system clock. As an example, if an audio application reads blocks covering 160 sampling periods from the input device, the timestamp would be increased by 160 for each such block, regardless of whether the block is transmitted in a packet or dropped as silent. The initial value of the timestamp is random, as for the sequence number. Several consecutive RTP packets may have equal timestamps if they are (logically) generated at once, e.g., belong to the same video frame.

From the above definition, we can see that UE can calculate the time interval of each packet relative to the first packet based on timestamp. For example, assuming the Multi-Modal service contains video and audio streams. The synchronization threshold is 30ms. The video sampling period is 90Khz, and the time stamp corresponding to the first video packet is 10000. The audio sampling period is 8Khz, and the time stamp corresponding to the first audio packet is 20000, then the interval of each packet relative to the first packet is shown in the figure below.

Assuming the state of the buffer is as shown in the following figure after a period. Since the relative time difference between data4 in the audio buffer and data2 in the video buffer is greater than the synchronization threshold, the two streams are lack of synchronization.

Proposal 1: RAN2 assume that the timestamp field in the RTP header can be used to identify whether Multi-Modal service fulfils synchronization thresholds.

To enhance the user experience, gNB can configure a synchronization status reporting threshold, just like the DSR mechanism. Before the Multi-Modal service is about to exceed the synchronization threshold, UE reports the event to the network. The network can use this report to enhance scheduling, for example, allocating more resources and preferentially scheduling the packets that are about to exceed the synchronization threshold.

Proposal 2: UE report the synchronization status to gNB before the Multi-Modal service is about to exceed the synchronization threshold to enhance gNB scheduling.

3. Conclusions

In this contribution, we discussed how to identify whether the Multi-Modal service fulfils the synchronization requirement. These proposals are summarised as below.

Proposal 1: RAN2 assume that the timestamp field in the RTP header can be used to identify whether Multi-Modal service fulfils synchronization thresholds .

Proposal 2: UE report the synchronization status to gNB before the Multi-Modal service is about to exceed the synchronization threshold to enhance gNB scheduling.

4. References

[1] RP-234014 New WID: XR (eXtended Reality) for NR Phase 3

[2] Draft_R2_126_Meeting_Report_v1

[3] RFC1889