3GPP RAN WG2 Meeting #127	R2-2407357

# Maastricht, Netherland, Aug 19th – 23rd, 2024

Agenda Item:	8.4.2

Source:	HONOR

Title:	Discussion on LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

1. Introduction

In RAN#103 meeting, the following objectives of Rel-19 LP-WUS work item were agreed [1]:

- For IDLE/INACTIVE modes (RAN1, RAN2, RAN3, RAN4) - Specify procedure and configuration of LP-WUS indicating paging triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point.

In RAN2#126 meeting, LP-WUS was discussed and the following agreements on LP-WUS monitoring conditions and subgrouping were made [2]:

- Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

- RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

In this contribution, we focus on the discussion of the following issues for LP-WUS in RRC_IDLE/INACTIVE.

- Monitoring condition of LP-WUS

- Subgrouping

2. Discussion

2.1. Monitoring condition of LP-WUS

The baseline definition for monitoring conditions has been made in RAN2#126 meeting. We go further to discuss the relationship between entry/exit thresholds based on last meeting agreement.

The agreement in RAN2#126 meeting says that the entry threshold is based on MR measurement, and exit threshold is based on LP-WUR measurement. Now there are two types of LP-WUR identified by RAN1, that is, OFDM-based LP-WUR and OOK-based LP-WUR.

For OFDM-based LP-WUR, its RRM measurement metrics are based on SSS, RAN1 has assumed that its RRM measurement metrics have the same definition of SS-RSRP and SS-RSRQ. In theory, the entry threshold should be the same as exit threshold for OFDM-based LP-WUR. However, considering that the RRM measurement accuracy of OFDM-based LPWUR is lower than that of MR, it is reasonable to have a gap between entry/exit thresholds.

For OOK-based LP-WUR, its RRM measurement metrics are based on LP-SS, which is different from reference signal for MR. And its RRM measurement accuracy is lower than that of MR either. Consequently, exit threshold for OOK-based LP-WUR should be different from entry threshold for MR.

Proposal 1: The entry/exit thresholds for LP-WUS monitoring in IDLE/INACTIVE mode could be different.

In RAN1 116bis meeting, it was assumed that FFS other conditions need to be satisfied. We observe some scenarios that have impact on the use of LP-WUS.

- When the ramp-up time of MR exceeds DRX cycle, it will affect the paging latency.

- High effective paging rate will affect power saving gain.

- High false alarm rate will affect power saving gain.

- LP-WUS decoding failure will affect paging reception.

In our perspective, we should discuss other entry/exit conditions of LP-WUS monitoring based on these scenarios. For example, when the ramp-up time of MR exceeds a threshold, UE may not monitor LP-WUS. The threshold could be I-DRX cycle or a value configured by gNB. If UE reports the ramp-up time of MR to the network in UE capability transfer procedure, the network can also decide whether to send LP-WUS to UE based on this threshold.

Proposal 2: RAN2 should discuss other entry/exit conditions of LP-WUS monitoring, e.g., when the ramp-up time of MR exceeds a threshold, UE may not monitor LP-WUS.

2.2. Subgrouping

RAN2 assumes the maximum number of subgroups is no less than 8 in #126 meeting. From RAN1#117 meeting agreements we can see the maximum number of subgroups is consistent with RAN2 assumption [3]. Thus, we can further discuss other factors that affect subgroup design.

- Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24 - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Option 3: A LP-WUS indicates multiple codepoint values with each corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256

In our view, another factor that have impact on the subgroup design is the association between LO and PO. RAN1 has identified three options for association between LO and PO [3].

- Option 1: UEs monitoring the same PO monitor the same LO - Option 2: UEs corresponding to different POs monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. - Combinations of the above options can be considered.

Option1 is a special case of Option2, that is, when Option2 is configured as one LO associated with one PO, it becomes Option1. These two options are similar to the association between the MO and PO in the PEI feature. We can directly reuse subgroup method of PEI for these two options.

For Option3, RAN2 needs to consider how to assign UE to a subgroup set. For example, we can assign UEs with the same MR ramp-up time to one set. This has the benefit of reducing the paging latency for UEs with relatively short MR ramp-up time.

Proposal 3: If Option1 or Option2 is adopted, RAN2 assumes reusing PEI subgroup method. If Option3 is adopted, RAN2 should discuss how to assign UE to one subgroup set.

3. Conclusions

In this contribution, we discussed monitor condition of LP-WUS and Subgrouping, and the proposals are summarised as below.

Proposal 1: The entry/exit thresholds for LP-WUS monitoring in IDLE/INACTIVE mode could be different.

Proposal 2: RAN2 should discuss other entry/exit conditions of LP-WUS monitoring, e.g., when the ramp-up time of MR exceeds a threshold, UE may not monitor LP-WUS.

Proposal 3: If Option1 or Option2 is adopted, RAN2 assumes reusing PEI subgroup method. If Option3 is adopted, RAN2 should discuss how to assign UE to one subgroup set.

4. References

[1] RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] Draft_R2_126_Meeting_Report_v1

[3] Draft_Minutes_report_RAN#117_v020