3GPP RAN WG2 Meeting #127	R2-2407358

# Maastricht, Netherland, Aug 19th – 23rd, 2024

Agenda Item:	8.4.4

Source:	HONOR

Title:	Discussion on LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

1. Introduction

In RAN#103 meeting, the following objectives of Rel-19 LP-WUS work item were agreed [1]:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

In RAN2 126 meeting, the following agreements on procedures for LP-WUS in RRC_CONNECTED were made [2]:

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

- For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

- The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

RAN2 has agreed to further discuss the impacts of LP-WUS operation methods identified in RAN1. Option1-1 has been fully discussed and some agreements have been reached. In this contribution, we will further discuss Option1-2 and Option1-3.

2. Discussion

2.1. Discussion on Option1-2 and Option1-3

For Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring [3]. It has two sub-options:

- Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

2)  Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

These two sub-options all have benefit for latency. For example, supposing a service contains a voice stream with 10ms periodicity and a video stream with 16.67ms periodicity, current CDRX mechanism can only adapt to one of the periodicities. If CDRX cycle is configured to be the same as the voice periodicity, the maximum delay for receiving video may be 6ms, which is unacceptable for low-latency services (e.g. XR). The following figure shows the issue.

Option 1-2 can solve this problem, as LP-WUS could trigger PDCCH monitoring outside C-DRX active time. Theoretically, it means that downlink data can be sent to the UE at any time.

Since PDCCH monitoring is triggered only by LP-WUS in Option1-2-2, this can reduce unnecessary PDCCH monitoring compared to Option1-2-1, which will bring monitoring power saving gains. Besides, RAN1 has another note to restrict the Option 1-2-1 to be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX, which will bring extra complexity comparing to Option 1-2-2. Thus, from the perspective of power consumption and complexity, we suggest excluding Option1-2-1 and adopting Option1-2-2.

For Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring [3]. It has benefit for service with large jitter.

For example, for periodic service with large jitter, current CDRX mechanism needs to be configured with a long drx-onDurationTimer to match with the real arrival time of data burst. However, it means that part of the PDCCH monitoring in the drx-onDurationTimer is useless, which will lead to high monitoring power consumption.

Option1-3 is the perfect solution to this issue, as UE can monitor LP-WUS instead of PDCCH from inside drx-onDurationTimer. And UE starts PDCCH monitoring only if LP-WUS is detected and indicates UE to monitor PDCCH. In this way, UE will benefit in terms of power consumption.

However, we note that Option1-3 is described as LP-WUS monitoring inside the CDRX active time. In our opinion, it has no benefit to monitor LP-WUS inside the active time other than the drx-onDurationTimer. For example, UE must use MR to monitor PDSCH inside drx-InactivityTimer. Therefore, UE can directly use MR to monitor PDCCH in this time, rather than triggering PDCCH monitoring by LP-WUS. It is better to clarify that the LP-WUS monitoring of Option1-3 is only inside the drx-onDurationTimer.

The following table summarizes the applicable scenarios and benefit of option1-2 and opiton1-3.

| LP-WUS monitor options | Applicable scenario | Benefit |
| --- | --- | --- |
| Option1-2 | Service with Multi-periodicity and delay sensitive | Option1-2-1: low DL transmission delay |
|  |  | Option1-2-2: - low DL transmission delay, - lower power consumption and complexity than Option1-2-1 |
| Option1-3 | Service with large jitter | reduce unnecessary PDCCH monitoring inside drx-onDurationTimer |

Proposal 1: RAN2 adopt Option1-2-2 and Option 1-3.

Proposal 2: For Option1-3, UE monitors LP-WUS only inside drx-onDurationTimer.

3. Conclusions

In this contribution, we discussed the pros and cons of Option 1-2 and Option 1-3. These proposals are summarised as below.

Proposal 1: RAN2 adopt Option1-2-2 and Option 1-3.

Proposal 2: For Option1-3, UE monitors LP-WUS only inside drx-onDurationTimer.

4. References

[1] RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] Draft_R2_126_Meeting_Report_v1

[3] Draft_Minutes_report_RAN#117_v020