3GPP TSG-RAN WG2 Meeting #127	R2-2407365

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.1.2.2

Source:	LG Electronics Inc.

Title:	Definition of functionality and Applicabtiliy related report

Document for:	Discussion and Decision

# 1. Introduction

According to the conclusion in the latest post e-mail discussion [1] in RAN2, there are proposals for further discussion.

Definition of functionalities

Proposal 1: Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling). Proposal 3: Applicable functionalities refers to functionalities that the UE is ready to apply for inference. Proposal 5: Activated functionalities refers to functionalities already enabled for performing inference. Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed. Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities. Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities

Applicabtility related report

<Proactive reporting>

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

Step 3:  Network configures UE that it is allowed to provide its applicable functionalities.

Step 4: UE sends applicable functionalities to network upon change of applicable functionality/condition.

Step 5: Network sends inference configuration for the applicable functionalities to the UE.

Step 6: Start inference/monitoring based on network/UE activation/deactivation.

<Reactive reporting>

Step 1: Network sends UECapabilityEnquiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

Step 3:  Network provides network configurations and initiates UE to report its applicable functionalities.

Step 4: UE sends applicable functionalities to network.

Step 5: Network sends updated inference configuration for applicable functionalities reported in Step 4 to the UE. (see Q2-6)

Step 6: Start inference/monitoring based on network/UE activation/deactivation.

Proposal 1: Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. Proposal 2: “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. Proposal 3: UE decides the applicable functionalities based on NW-side additional conditions (if available), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE. Proposal 4: “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition (if available), FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc Proposal 5: Step 5: 1) Network configures full inference configuration to UE after applicable functionality reporting, if configuration of supported functionality is not provided in Step 3 (i.e. full inference configuration is provided in Step 5). 2) If configuration of supported functionality is agreed to be provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. Proposal 6: The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer). FFS on the order of inference configuration and applicable functionality reporting, FFS on functionality activation.

This contribution aims to address issued regarding definition of functionalities, applicability related report, and the functionality-based LCM for UE-sided model focusing on the beam management use case.

# 2. Discussion

## Definition of functionalities

Initially, in the RAN2 study phase, the difference between proactive and reactive reports was the timing of the inference configuration and the applicability related report, which can be expressed as shown in the figure below.

- Proactive reporting                                           (b) Reactive reporting

The terms "proactive report" and "reactive report" might not be explicitly specified in the specifications, but they could be appropriately specified in relevant sections for each report.

- In the case of 'proactive report', it can be specified as a report to be configured for a certain functionality, i.e., it can be specified at the point before inference configuration for a functionality is configured

- In the case of "reactive report", it can be specified as a report on applicable functionality to be re-configured/(de)activated among configured functionalities, i.e., it can be specified at any point after inference configuration for a functionality has been configured

Therefore, there may be a need for a common understanding within RAN2 about the meaning of the configured functionality based on inference configuration. However, according to [1], there are three ambiguities for inference configuration.

- Ambiguity on relation between NW sided additional condition and full inference configuration: For the network-sided additional conditions, whether they are partial/separate configuration of full inference configuration (At least for proactive reporting)

- Ambiguity on timing for full inference configuration: Whether full inference configuration needs to be provided to UE before applicable functionality reporting (At least for reactive reporting)

- Ambiguity on range of full inference configuration: Whether full inference configuration include Beam related configuration like set A and set B, other CSI framework related configuration, or outside of CSI framework (For both proactive and reactive reporting)

During RAN2 discussion, there are no consensus on the above aspects and, it might be RAN1 scope. With regard to defining configured functionality and clarifying relationship between configured functionality and applicable functionality, we propose to wait until above ambiguities become clear from RAN1 and re-discuss them.

Proposal 1. Before (i)idefining configured functionality and (ii)clarifying relationship between configured functionality and applicable functionality, RAN2 to wait RAN1 input to address below ambiguities:

- Ambiguity on relation between NW sided additional condition and full inference configuration: For the network-sided additional conditions, whether they are partial/separate configuration of full inference configuration (At least for proactive reporting)

- Ambiguity on timing for full inference configuration: Whether full inference configuration needs to be provided to UE before applicable functionality reporting (At least for reactive reporting)

- Ambiguity on range of full inference configuration: Whether full inference configuration include beam related configuration like set A and set B, other CSI framework related configuration, or outside of CSI framework (For both proactive and reactive reporting)

## Applicability related report

Initial Applicabiility-Related Report Scenario

During the latest post e-mail discussion [1] in RAN2, RAN2 discuss initial applicability-related report procedure. The signalling framework can be revised to combine proactive and reactive reporting based on the conclusion.

Regarding Step 3, especially for proactive reporting, instead of informing about applicability of all ‘supported functionality’, the UE should notify the network of the functionalities that are intended to be activated/managed by the network to avoid unnecessary transmission of applicability-related reports. Therefore, the network should initially configure the intended functionalities in step 3.

Proposal 2. For step 3, network should configure the intended functionalities to be activated/managed by the network to avoid unnecessary transmission of applicability-related reports. Revised proposal can be:

Proposal 2: “Step 3”: Following configurations are provided from NW to UE:

1) Network may provide the intended functionalities to manage (e.g., activation) on network side

2) UE is allowed to do UAI reporting via OtherConfig.

3) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

4) FFS on configuration (e.g. inference configuration) of supported functionalities.

Regarding step 4, for additional information beyond applicability, it would be useful to provide the associated IDs related to the NW-side additional conditions and/or preferred configurations (e.g. beam sets A/B). Based on this information, the network can configure (at least for proactive reporting) or update (at least for reactive reporting) the inference configuration in step 5. However, due to the ambiguity about the relationship between the NW-side additional conditions and the full inference configuration, and whether the scope of the full inference configuration includes other CSI framework configurations or outside of the CSI framework configuration, it is premature to say what additional information can be included along with the applicability. Therefore, additional information can be left as FFS for further discussion later.

Proposal 3. For step 4, leave it as FFS to provide additional information such as associated ID or preferred configuration for further discussion after ambiguities about relation between NW sided additional condition and full inference configuration and scope of full inference configuration whether it includes CSI framework configuration or outside of CSI framework configuration are clear. Revised proposal can be:

Proposal 4: “Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition (if available), FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

FFS whether associated ID related to NW-side additional condition(if configured) or preferred configuration can be include along with applicability

Regarding Step 6, until the UE sends applicability related report, the network does not know which configuration and/or functionality are applicable. Allowing initial activation of applicable functionality in step 3 would lead to a mismatch between the UE and the network regarding the activated functionality, which is not suitable for NW-side LCM cases. Also, in the absence of updated inference configuration (at least for reactive reporting) in step 5, the network should indicate whether the UE can activate a functionality via L1/L2/L3 signalling. Therefore, we propose to remove the first FFS and add L3 to the second FFS.

Proposal 4. For step 6, to remove first FFS (FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3.) to avoid mismatch between the UE and the network regarding the activated functionality, which is not suitable for NW side LCM case. Revised proposal can be:

Proposal 6: The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2/L3 signaling for activation/deactivation.

Update Aplicabiility-Related Report Scenario

For further discussion, RAN2 need to consider the applicability update scenario.

Regarding step 7, the UE-side additional condition such as memory, battery, and other hardware limitations can change the applicability of functionality from applicable to non-applicable. Note that, in dual connectivity scenarios, the UE can request SCG deactivation due to factors such as battery or thermal issues. Detailed conditions have not been specified in spec, but UE can simply request SCG deactivation based on UE decisions. Similarly, without specifying UE's internal conditions, the UE can make general decision for applicability. A simple way to report updated information could be to use UAI. Based on this updated information, the network can decide to (de)activate a functionality.

Proposal 5. Do not specify UE-side additional condition considering that all UE-side additional condition cannot be specified

Proposal 6. To send UAI to notify of a non-applicable functionality that was previously reported as applicable, or to notify of an applicable functionality that was not reported as applicable

Regarding step 9, when the network sends an LCM command (e.g., activation) for a functionality via L1/L2/L3 signaling, the UE may not be able to activate the functionality due to UE-side additional condition. In this case, RAN2 need to discuss whether the applicability related report can be utilized to reject the LCM command. For example, when the UE is unable to activate the functionality, the UE can inform the network via subsequent UAI that the functionality is not applicable.

Proposal 7. To discuss whether the applicability related report can be utilized to reject the LCM command, e.g., the UE can report non-applicability for a functionality via UAI upon receiving activation command for the functionality

## LCM

During the RAN2 meeting, we have reached agreement regarding functionality management.

Agreement on RAN2 #125bis For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

In the scenario of 'network-decision, UE-initiated', RAN2 assumes that the network may reject the UE’s request. However, it is unclear how the UE should proceed when its requests are either rejected by the network or receive no response. When the network rejects the UE's request, continuing the current AI/ML-related operation could potentially result in inaccurate AI/ML functioning, thereby negatively impacting the stable beam operation of the UE. Additionally, AI/ML operations are not guaranteed to perform in good radio condition. Accordingly, it is necessary to consider scenarios where the UE’s request is not delivered to the network or instructions are not received in the UE. Therefore, RAN2 needs to discuss the UE’s behaviour in cases where its requests are either rejected by the network or receive no response.

Proposal 8. To discuss the UE’s behaviour in cases where its requests are either rejected by the network or receive no response, particularly in the scenario of 'network-decision, UE-initiated'.

# 3. Conclusion

Proposal 1. Before (i)idefining configured functionality and (ii)clarifying relationship between configured functionality and applicable functionality, RAN2 to wait RAN1 input to address below ambiguities:

- Ambiguity on relation between NW sided additional condition and full inference configuration: For the network-sided additional conditions, whether they are partial/separate configuration of full inference configuration (At least for proactive reporting)

- Ambiguity on timing for full inference configuration: Whether full inference configuration needs to be provided to UE before applicable functionality reporting (At least for reactive reporting)

- Ambiguity on range of full inference configuration: Beam related configuration like set A and set B, other CSI framework related configuration, or outside of CSI framework (For both proactive and reactive reporting)

Proposal 2. For step 3, network should configure the intended functionalities to be activated/managed by the network to avoid unnecessary transmission of applicability-related reports. Revised proposal can be:

Proposal 2: “Step 3”: Following configurations are provided from NW to UE:

1) Network may provide the intended functionalities to manage (e.g., activation) on network side

2) UE is allowed to do UAI reporting via OtherConfig.

3) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

4) FFS on configuration (e.g. inference configuration) of supported functionalities.

Proposal 3. For step 4, leave it as FFS to provide additional information such as associated ID or preferred configuration for further discussion after ambiguities about relation between NW sided additional condition and full inference configuration and scope of full inference configuration whether it includes CSI framework configuration or outside of CSI framework configuration are clear. Revised proposal can be:

Proposal 4: “Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition (if available), FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

FFS whether associated ID related to NW-side additional condition(if configured) or preferred configuration can be include along with applicability

Proposal 4. For step 6, to remove first FFS (FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3.) to avoid mismatch between the UE and the network regarding the activated functionality, which is not suitable for NW side LCM case. Revised proposal can be:

Proposal 6: The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2/L3 signaling for activation/deactivation.

Proposal 5. Do not specify UE-side additional condition considering that all UE-side additional condition cannot be specified

Proposal 6. To send UAI to notify of a non-applicable functionality that was previously reported as applicable, or to notify of an applicable functionality that was not reported as applicable

Proposal 7. To discuss whether the applicability related report can be utilized to reject the LCM command, e.g., the UE can report non-applicability for a functionality via UAI upon receiving activation command for the functionality

Proposal 8. To discuss the UE’s behaviour in cases where its requests are either rejected by the network or receive no response, particularly in the scenario of 'network-decision, UE-initiated'.

# 4. References

[1] R2-24xxxxx_[POST126][032][AIML PHY] LCM (Intel_Samsung)_Phase 2_v30_Rapp.docx