3GPP TSG-RAN WG2 Meeting #127	R2-2407366

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.1.3

Source:	LG Electronics Inc.

Title:	NW side data collection

Document for:	Discussion and Decision

# 1. Introduction

Regarding inference data collection, the required latency is ‘Time-critical’, which requires less than a few msecs[2]. Given the latency and processing time in RRC taking up about 10msec or more, the RRC signalling is not suitable for data collection for inference. Also, L1 signalling based report for inference is being discussed in RAN1. Therefore, data collection for inference can be started depending on RAN1 input.

Regarding monitoring data collection, the required latency is near-real time, which requires less than several tens of msecs to a few seconds[2]. Accordingly, RRC signalling may be a candidate solution. Given the complexity and UE’s power consumption, instead of timely reporting, logging based reporting may also be considered. According to RAN1 LS[1][2], data collection for training and monitoring typically requires L1-RSRP and/or beam-ID. Therefore, some signalling/procedure for tragining data collection can be reused for monitoring data collection to report L1 measurement results. Therefore, RAN2 need to define necessary signalling/procedure for training data collection first.

In this contribution, we discuss the necessary signalling/procedure for NW side data collection for training of beam management use case from RAN2 perspective.

# 2. Discussion

Define RAN2 impact

Before we discuss further on data collection, we need to understand the ongoing discussion in RAN1. In RAN1 discussion, it is FFS on additional optimization for contents from multiple time instances in one reporting. If reporting multiple time instances is necessary, logging in RRC becomes inevitable.

RAN1 also discussed how to reduce the overhead of data for training, inference, and monitoring. Three methods are being considered: (i) Omission/selection of measurement quantity, (ii) Reduce the overhead of beam information, and (iii) Quantization of RSRP.

- In the Omission/selection of measurement quantity, only report Top M bemas with highest RSRP and only report the RSRP larger than a threshold or within a threshold are considered.

- In the Reduce the overhead of beam information, bitmap-based, no CRI/SSBRI ID, pattern ID, etc are considered

- In the Quantization of RSRP, larger quantization step for L1-RSRP reporting, differential L1-RSRP based reporting with a smaller quantization range, etc are considered.

If the L1-RSRP and/or beam index information is delivered to the RRC with omission and selection from lower layer, it remains unclear whether additional filtering of data in RRC is necessary. If additional filtering is performed at the RRC level, information required for the RAN1 use case may be deleted. Therefrore, as a starting point, RAN2 can assume that RRC could store all relevant measurement quantities from lower layers for logging without further optimization to reduce logging overhead. This approach, however, can be updated based on additional RAN1 input, allowing RAN2 to start with this preliminary understanding.

Observation 1. RAN1 has discussed how to reduce the overhead of data with omission/selection of measurement quantities (e.g., only report Top M bemas with highest RSRP and only report the RSRP larger than a threshold or within a threshold)

Proposal 1. As a starting point, do not consider optimization for overhead reduction of logging in RRC, i.e., RRC stores the measurenet quantities received from lower layer for data collection

Since the data is delivered through the RRC, the report should be controlled by the RRC layer. Typically, there are three types of reporting: periodic, event-based,and on-demand request. Not that regardless of the report type, as the reported contents consist of logged data, they should not cause memory issues in the UE. Therefore, to avoid memory issues in the UE, it is improtant to consider the report conditions with respect to the amount of data on the UE side. Therefore, RAN2 need to discuss whether there need additional condition to report with respect to UE’s memory size. The straight forward way is to report collected data when the number of data from lower layer is larger than a threshold.

Proposal 2. To report data, to consider report type in RRC: periodic, event-based, on-demand request

Proposal 3. To discuss whether there need additional condition to report with respect to UE’s memory size. The straight forward way is to report collected data when the number of data from lower layer is larger than a threshold.

# 3. Conclusion

Observation 1. RAN1 has discussed how to reduce the overhead of data with omission/selection of measurement quantities (e.g., only report Top M bemas with highest RSRP and only report the RSRP larger than a threshold or within a threshold)

Proposal 1. As a starting point, do not consider optimization for overhead reduction of logging in RRC, i.e., RRC stores the measurenet quantities received from lower layer for data collection

Proposal 2. To report data, to consider report type in RRC: periodic, event-based, on-demand request

Proposal 3. To discuss whether there need additional condition to report with respect to UE’s memory size. The straight forward way is to report collected data when the number of data from lower layer is larger than a threshold.

# 4. References

[1] R1-2308730 RAN2 LS reply on data collection

[2] R1-2310681 Reply LS on Data Collection Requirements and Assumptions

[3] 38843-201