3GPP TSG-RAN WG2 Meeting #127			     R2-2407368

Maastricht, Netherlands, August 19 – 23, 2024

Agenda Item:	8.7.5 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Discussion on details of RLC enhancements for XR

Document for: 	Discussion and Decision

Introduction

In the RAN2#126 meeting, RAN2 summarised possible options for RLC enhancement and, as a next step, the session chair suggested to analyse and compare options for avoiding unnecessary retransmissions and achieving timely retransmission. This contribution discusses proposed solutions for RLC enhancements and present our views on each solution.

Discussion

Avoiding unnecessary retransmissions

The following two approaches were summarised and RAN2 agreed to choose one of two approaches for avoiding unnecessary retransmissions in the last RAN2 meeting.

RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. - Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs. - Tx side stops retransmit obsolete SDUs. - Rx side updates state variables according to the information from Tx side. - Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy. - FFS how Rx side determines that an SDU should be abandoned.

To compare two approaches, we think that whole procedure of each approach should be described first and then it needs to check what is a key element to make it operate correctly and weak point of each approach.

For Tx initiated approach, when the transmitting entity receives a discard indication for a RLC SDU which is already transmitted and awaiting for acknowledgment in the transmitting window, the transmitting entity would stop retransmitting this obsolete RLC SDU and notify the receiving entity about this obsolete RLC SDU, e.g., SN gap report. Then the receiving entity updates state variable based on the information from the transmitting entity. A key element of Tx initiated approach is that how to determine whether the report about obsolete RLC SDUs is successfully transmitted to the receiving entity and how to update state variable about the obsolete RLC SDUs in the transmitting entity.

Most contributions supporting Tx approach in the last RAN2 meeting assume that the receiving entity will transmit a STATUS report after receiving the SN gap report and then state variables in the transmitting entity are updated only based on the STATUS report. However, the problem is that if the SN gap report is missed or the STATUS report from the receiving entity is missed, the transmitting entity cannot update state variable. This means that transmission window stalling would not be solved and follow-up transmission may be also unnecessarily delayed since the transmission window does not move forward. Thus, it is important to guarantee that the SN gap report and the STATUS report from the receiving entity are successfully transmitted.

Observation 1. For Tx initiated approach, if state variables in the transmitting entity is updated only based on the received STATUS report as in legacy, transmission window stalling would not be solved and follow-up transmission is also unnecessarily delayed until another STATUS report is successfully received from the receiving entity.

To resolve this concern on the observation 1, if RAN2 introduce a mechanism to guarantee a successful transmission of the SN gap report like Move Receiving Window (MRW) in UTAR RLC, the Tx approach would be complex and benefits may be insignificant. Thus, we think that RAN2 should not make same mistake on this issue and should find another way to relieve this concern. One possible way is that the transmitting entity updates state variables based on information in the SN gap report, i.e., transmitting window may move forward after the RLC SDU discard. In this condition, even if the SN gap report and the STATUS report from the receiving entity are missed, the transmitting entity can resolve transmitting window stalling problem and perform follow-up transmission. Of course, if the SN gap report is missed, the receiving entity may include a NACK information for the discarded RLC SDUs in the STATUS report, but this may not cause a big problem in the transmitting entity since those RLC SDUs may be already located in the out of transmitting window or the transmitting entity can ignore this NACK for already discarded RLC SDUs. The only thing is needed that the transmitting entity would retransmit the SN gap reporting because NACK for the discarded RLC SDU in the STATUS report means that the previous SN gap report is missed.

Observation 2. Introducing a mechanism to guarantee a successful transmission of the SN gap report like Move Receiving Window (MRW) in UTAR RLC is not beneficial.

Proposal 1. If Tx initiated approach is introduced, the discarded RLC SDU considers positively acknowledged and updates state variables for the transmitting window, i.e., transmitting window move forward based on information in the SN gap report.

Proposal 2. If Tx initiated approach is introduced, the transmitting entity retransmits the SN gap report after receiving the STATUS report including NACK for a discarded RLC SDU already reported in the SN gap report.

For Rx initiated approach, the receiving entity determines whether an SDU should be abandoned or not. Then if the receiving entity abandons SDUs, the receiving entity considers that the abandoned RLC SDU is successfully received and moves the receiving window forward. After that, when the receiving entity generates a STATUS report, this abandoned RLC SDU is positively acknowledged in the STATUS report. The transmitting entity updates state variable based on the received STATUS report as in legacy procedure. A key element of Rx initiated approach is how to determine which SDU should be abandoned as the following RAN2 agreement.

FFS how Rx side determines that an SDU should be abandoned.

As per contributions submitted in the last RAN2 meeting, one company suggests a counter based method, but majority companies suggest a timer based method. For the timer based method, when the receiving entity detects a missing RLC SDU, a timer is started and then if the timer expires, the receiving entity determines a RLC SDU causing the SN gap is abandoned and then moves the receiving window forward. On the other hand, for the counter based method, when the receiving entity detects a missing RLC SDU, the receiving entity increments a counter for the missing RLC SDU and then if the counter of the missing RLC SDU reaches a certain threshold, the receiving entity determines a missed RLC SDU causing the SN gap is abandoned and then moves the receiving window forward. Considering that the main purpose of avoiding unnecessary retransmission is not to retransmit the RLC SDU whose delay already exceeds the PDB, the counter based method would not prevent unnecessary retransmission well since time duration between two consecutive counter increments would not be static and the counter may not reach a certain threshold even after exceeding the PDB of the RLC SDU. Thus, if the Rx initiated approach is introduced, the timer based method should be introduced to determine whether a RLC SDU is abandoned.

Observation 3. A counter based Rx initiated approach would not avoid unnecessary retransmission well.

Proposal 3. If the Rx initiated approach is introduced, the timer based method is introduced to determine whether a RLC SDU is abandoned and the abandoned RLC SDU is positively acknowledged in the STATUS report.

To sum up, the following pros and cons can be identified for each approach as shown in the below table. We think that the Rx initiated approach is much simpler and specification impact is also less than the Tx initiated approach. Someone may have concerns that the Rx initiated approach cannot exactly determine that the RLC SDU exceeds the PDB and the receiving window can be moved forward earlier or later than the exact expected timing. However, if a missing RLC SDU is detected at the receiving entity, the PDCP entity would also detect a SN gap and then start the t-Reordering timer. After expiry of t-Reordering timer, the PDCP entity moves the receiving window forward and RLC SDU having SN lower than RX_DELIV in the PDCP entity would be discarded anyway. Thus, considering the t-Reordering timer in the PDCP, we think that if the new timer value to abandon a RLC SDU is quite aligned with the value of t-Reordering timer in the PDCP, their concerns can be relieved and the Rx initiated approach is better than the Tx initiated approach.

Proposal 4. The timer based Rx initiated approach is introduced to avoid unnecessary retransmission.

|  | Tx initiated approach | Rx initiated approach |
| --- | --- | --- |
| Pros | - Can exactly determine whether the RLC SDU exceeds the PDB. | - Only receiver side operation needs to be changed. |
| Cons | - Both transmitter and receiver side operation need to be changed. - A new SN gap report needs to be introduced and retransmission/confirmation of the new report need to be considered. | - Cannot exactly determine whether the RLC SDU exceeds the PDB. |

Timely RLC retransmissions

- To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

Before diving into each option in the above RAN2 agreements, it should be discussed first whether existing mechanisms can be reused for timely RLC retransmission without any enhancements as FFS indicated.

The opponent companies believe that if smaller values for existing STATUS report related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) are introduced and configured, the STATUS report can be transmitted frequently and there would be no more timely RLC retransmission problem. However, we think that this existing mechanism with smaller timer value would cause resource waste problem and early RLF declaration problem. Specifically, the current minimum value of t-Reassembly and t-StatusProhibit are zero. If zero is configured for both t-Reassembly and t-StatusProhibit, the receiving entity would not wait enough time to receive RLC SDU or RLC SDU segment which is still being transmitted or retransmitted at the lower layer and generate the STATUS report including pre-mature NACK. For example, as shown in the below figure, when the receiving entity detects a SN gap after receiving the RLC SDU5, the receiving entity triggers and transmits the STATUS report including NACK for RLC SDU3/4 to the transmitting entity since t-Reassembly and t-StatusProhibit are configured with zero. Then the receiving entity receives RLC SDU 3/4 successfully after transmitting the STATUS report, but the transmitting entity should retransmit the RLC SDU 3/4 unnecessarily and this resource waste cannot be avoided. We think that even though non-zero value, e.g., smaller value, is configured for these timer, same situation can occur and this resource waste problem may not be avoided due to aggressively frequent STATUS report transmission.

Observation 4. The existing mechanism with smaller values for existing RLC STATUS reporting related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) can generate the STATUS report with pre-mature NACK and this cause unnecessary resource waste problem.

Another problem is that a STATUS report with pre-mature NACK can be transmitted frequently and the pre-mature NACK in each STATUS report can retransmit the RLC SDU with incrementing the ReTx counter. This can reach maximum retransmission threshold more quickly than normal configuration. That is, the possibility of RLF declaration by maximum RLC retransmissions would be increased by a smaller value of RLC STATUS reporting related timers. It is obvious that early RLF by maximum RLC retransmissions is bad for timely RLC retransmission.

Observation 5. The existing mechanism with smaller values for existing RLC STATUS reporting related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) may trigger unnecessary RLF by maximum RLC retransmissions.

Based on the observation 4 and 5, existing mechanism with smaller timer values cannot resolve timely RLC retransmission problem and rather generates another side effects. Therefore, enhancements for timely RLC retransmission should be investigated in Rel-19.

Proposal 5. RAN2 confirm that existing mechanisms with smaller timer values are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19.

As the next step, which enhancements are needed to achieve timely RLC retransmissions for XR traffic?

The basic RLC AM retransmission mechanism is based on a STATUS report. When the transmitting entity receives the STATUS report, the transmitting entity can retransmit only RLC SDU which is indicated by NACK in the STATAU report. This means that the transmitting entity has to delay the retransmission of the RLC SDU until the STATUS report is received. In addition, even though the transmitting entity receives the STATUS report, if the received STATUS report does not include NACK for the RLC SDU in the transmitting window, the transmitting entity cannot retransmit the RLC SDU and should wait for the next STATUS report including NACK for the RLC SDU. For this reason, RAN2 should consider two approaches together for ensuring timely RLC retransmissions: one is how to get a STATUS report for timely retransmission and another is how to retransmit the RLC SDU without the STATUS report since the received STATUS report may not include NACK for the delay-critical RLC SDU timely.

For how to get a STATUS report for timely retransmission, the current STATUS report can be triggered by polling at the transmitting entity or a SN gap detection at the receiving entity. As per the current RLC specification, the transmitting entity can include a poll bit to request a STATUS report when the number of transmitted RLC PDUs or the amount of transmitted bytes reaches a certain threshold, but there is no condition to check delay status of a RLC SDU. In this situation, even though the delay-critical RLC SDU is transmitted, the transmitting entity cannot include a poll bit unless the current condition for polling is satisfied. The transmitting entity may not receive the STATUS report before expiry of discard timer for the delay-critical RLC SDU since there is no poll included in the last transmission and the receiving entity may not detect any missing RLC SDUs. As a result, the delay-critical RLC SDU may not have another retransmission opportunity before exceeding PDB and this becomes an out-dated RLC SDU unnecessarily. Thus, it would be important to receive the STATUS report after the delay-critical RLC SDU transmission to receive another retransmission opportunity and satisfy the PDB of the delay-critical RLC SDU. Thus, the transmitting entity should include a poll when the delay-critical RLC SDU is submitted to the lower layer for transmission.

Proposal 6. The transmitting entity include a poll when the RLC SDU having remaining time lower than a threshold is submitted to the lower layer for transmission.

Another option to receive a STATUS report for timely retransmission is to enhance status report mechanism and followings can be considered for this enhancement.

- Option 1. STATUS report trigger based on bytes/number of PDUs received at the Rx entity;

- Option 2. Periodical status report;

- Option 3. Allow immediate reporting of the RLC status;

For option 1, considering that the transmitting entity already counts bytes and number of PDUs to include a poll for the STATUS report, even though the option 1 is introduced, the receiving entity anyway triggers a STATUS report by the poll bit from the transmitting entity and this duplicate triggering condition at the receiving entity would not be helpful to transmit a STATUS report for timely retransmission.

For option 2, the network can transmit the STATUS report for UL traffic periodically and can include a poll for DL traffic periodically by network implementation. This means that same result can be achieved by network implementation already.

For option 3, there are two proposals for this: one is to stop the t-StatusProhibit timer if it detects an SN gap and another is to ignore the condition (i.e., SN x < RX_Highest_Status or x >= RX_Next + AM_Window_Size) for triggering STATUS report. Regarding immediate STATUS report after detecting SN gap, this can be achieved by configuring smaller value of STATUS report related timers since the zero value is already possible to configure the t-StatusProhibit timer. However, as explained in the observation 4/5, this can cause pre-mature NACK in the STATUS report which can waste radio resource. Just immediate SATAUS report after detecting SN gap may not be helpful and this should be controlled by proper configuration based on the XR traffic characteristic. For ignoring the condition for triggering STATUS report (i.e., SN x < RX_Highest_Status or x >= RX_Next + AM_Window_Size), in our understanding, this condition is to ensure that the RLC Status report is transmitted after HARQ reordering and this would not be long. Thus, it is sceptical how much gain can be achieved from the option 3.

Based on the explanation so far, we think that option 1/2/3 are not essential and may not be needed for timely RLC retransmission.

Proposal 7. Retransmission based on enhanced status report is not supported.

Even if enhancements to receive a STATUS report for timely retransmission is adopted, it should be also considered how to retransmit the RLC SDU without the STATUS PDU since RLC PDUs can be missed anytime and there is still possibility not to receive the STATUS report timely. For instance, if the PDU including a poll is missed, another poll bit can be included again after expiry of a poll retransmission timer or satisfying another triggering condition which would take an unacceptable latency for delay-critical RLC SDUs. In addition, even the PDU including a poll is successfully transmitted, the receiving entity cannot know exactly which RLC SDU is a delay-critical RLC SDU at the transmitting entity. The receiving entity cannot selectively include necessary NACK for the delay-critical RLC SDU and NACK in the STATUS report is determined only based on the identified missing SN from the receiving window. It would be hard to receive the STATUS report at the time point where the transmitting entity wants for retransmission of delay-critical RLC SDU. To overcome this limitation of RLC AM retransmission mechanism, we think that if remaining time of a RLC SDU in the transmitting window becomes below a threshold, e.g., the RLC SDU is indicated as a delay-critical RLC SDU by the PDCP, this delay-critical RLC SDU should be considered for retransmission without receiving NACK for this delay-critical RLC SDU. Furthermore, considering low remaining time of the delay-critical RLC SDU, one time retransmission of the delay-critical RLC SDU would not be sufficient since a retransmission of the delay-critical RLC SDU can be also failed and the transmitting entity may not receive another STATUS PDU before exceeding the PDB of the delay-critical RLC SDU. Therefore, multiple autonomous retransmissions can be considered for the delay-critical RLC SDU to satisfy the PDB.

For concerns on capacity loss due to blind autonomous retransmission, when remaining time of a RLC SDU becomes below a threshold, a delay status report at the MAC layer would be already triggered and transmitted depending on the configured value of the threshold. The network would give the UE both an uplink grant and the STATUS report together after receiving a DSR. In this situation, the transmitting entity can cancel the retransmission of the RLC SDU having lower than the threshold based on the positive acknowledgment in the STATUS report. If a network vendor has a concern to trigger retransmission of a RLC SDU by the UE itself without the network’s order, this autonomous retransmission can be controlled by a STATUS report with a new indication. After receiving the DSR from the UE, the network can transmit a STATUS report with a new indication to make the UE retransmit RLC SDUs having remaining time lower than the threshold and not yet indicated by ACK based on the STATUS report information. This means that capacity loss would be marginal since only RLC SDUs which are not yet positively acknowledged can be retransmitted.

Observation 6. If the transmitting entity considers to retransmit the RLC SDU having remaining time lower than the threshold, capacity loss may be marginal since only RLC SDUs which are not yet positively acknowledged can be retransmitted.

There is another proposal for this issue to retransmit the RLC SDU based on HARQ feedback. However, this proposal requires huge impact to both RAN1 and RAN2 since explicit HARQ ACK/NACK for each uplink transmission should be newly defined. We think that it is unreasonable to introduce explicit HARQ ACK/NACK only for RLC retransmission determination and this should not be considered for this enhancement.

Proposal 8. If remaining time of a RLC SDU in the transmitting window becomes below a threshold, this RLC SDU should be considered for retransmission without receiving NACK for this RLC SDU.

Proposal 9. Multiple autonomous retransmissions of a delay-critical RLC SDU are supported.

Proposal 10. RLC retransmission based on HARQ NACK is not supported.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. For Tx initiated approach, if state variables in the transmitting entity is updated only based on the received STATUS report as in legacy, transmission window stalling would not be solved and follow-up transmission is also unnecessarily delayed until another STATUS report is successfully received from the receiving entity.

Observation 2. Introducing a mechanism to guarantee a successful transmission of the SN gap report like Move Receiving Window (MRW) in UTAR RLC is not beneficial.

Observation 3. A counter based Rx initiated approach would not avoid unnecessary retransmission well.

Observation 4. The existing mechanism with smaller values for existing RLC STATUS reporting related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) can generate the STATUS report with pre-mature NACK and this cause unnecessary resource waste problem.

Observation 5. The existing mechanism with smaller values for existing RLC STATUS reporting related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) may trigger unnecessary RLF by maximum RLC retransmissions.

Observation 6. If the transmitting entity considers to retransmit the RLC SDU having remaining time lower than the threshold, capacity loss may be marginal since only RLC SDUs which are not yet positively acknowledged can be retransmitted.

Proposal 1. If Tx initiated approach is introduced, the discarded RLC SDU considers positively acknowledged and updates state variables for the transmitting window, i.e., transmitting window move forward based on information in the SN gap report.

Proposal 2. If Tx initiated approach is introduced, the transmitting entity retransmits the SN gap report after receiving the STATUS report including NACK for a discarded RLC SDU already reported in the SN gap report.

Proposal 3. If the Rx initiated approach is introduced, the timer based method is introduced to determine whether a RLC SDU is abandoned and the abandoned RLC SDU is positively acknowledged in the STATUS report.

Proposal 4. The timer based Rx initiated approach is introduced to avoid unnecessary retransmission.

Proposal 5. RAN2 confirm that existing mechanisms with smaller timer values are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19.

Proposal 6. The transmitting entity include a poll when the RLC SDU having remaining time lower than a threshold is submitted to the lower layer for transmission.

Proposal 7. Retransmission based on enhanced status report is not supported.

Proposal 8. If remaining time of a RLC SDU in the transmitting window becomes below a threshold, this RLC SDU should be considered for retransmission without receiving NACK for this RLC SDU.

Proposal 9. Multiple autonomous retransmissions of a delay-critical RLC SDU are supported.

Proposal 10. RLC retransmission based on HARQ NACK is not supported.