3GPP TSG-RAN WG2 #127                                             	    R2-2407370

Maastricht, Netherlands, August 19th –23th, 2024

Agenda item:	7.4.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on PRACH occasion validation for LTM

Document for:	Discussion/Decision

# Introduction

When a UE is configured with LTM candidate cells with LTM-Config, the UE can also be provided with EarlyUL-SyncConfig for each of the candidate cells [1], to acquire a valid timing adjustment before the LTM cell switch. Based on the PRACH transmission parameters provided by EarlyUL-SyncConfig, the UE can be triggered a PRACH transmission on the candidate cell by a PDCCH order received on the serving cell. In general, the PRACH transmission parameters include a configuration of PRACH occasions, a preamble subcarrier spacing, and a preamble index, etc. In particular, for a PRACH transmission initiated by a PDCCH order, the preamble index is provided by the PDCCH order.

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                     LTM-CandidateId-r18,

ltm-CandidatePCI-r18                    PhysCellId                         OPTIONAL,    -- Need M

ltm-SSB-Config-r18                      LTM-SSB-Config-r18                 OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                 OCTET STRING (CONTAINING RRCReconfiguration)
                                                                               OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                  ENUMERATED {true}                  OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18              OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)
                                                                               OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18           OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)
                                                                               OPTIONAL,    -- Need R

ltm-TCI-Info-r18                        LTM-TCI-Info-r18                   OPTIONAL,    -- Need M

ltm-NoResetID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)
                                                                               OPTIONAL,    -- Need N

ltm-UE-MeasuredTA-ID-r18                INTEGER (1..maxNrofLTM-Configs-plus1-r18)
                                                                               OPTIONAL,    -- Need N

...

}

As seen in the ASN.1 text above, the configuration for the early UL synchronization is provided separately from the full RRC reconfiguration message of the LTM candidate cell (i.e., ltm-CandidateConfig-r18). Therefore, to obtain the PRACH configuration of the candidate cell, the UE is not required to decode the RRC reconfiguration message of the LTM candidate cell. Thus, a UE implementation can decode the target RRC configuration only after it receives an LTM cell switch command to the LTM candidate cell.

Based on the above explanation, the UE may not know the TDD pattern configuration of an LTM candidate until LTM execution. However, the UE needs this information to validate the RACH occasions for early sync.

In this contribution, we dissuss this issue and suggest a way forward.

# Discussion

According to Clause 8.1 in TS 38.213 [2], for unpaired spectrum, SS/PBCH blocks are mapped to “valid” PRACH occasions, where the valid PRACH occasions are defined as follows:

For unpaired spectrum, - if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block reception symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = "semiStatic" is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213]. - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in clause 4.1 - If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if - it is within UL symbols, or - it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if channelAccessMode = "semiStatic" is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213] - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in clause 4.1.

For unpaired spectrum operation, the determination of valid PRACH occasions depends on whether the TDD pattern configuration, tdd-UL-DL-ConfigurationCommon, is configured or not and, if configured, which symbols are configured as uplink symbols.

Observation 1: To identify valid PRACH occasions for SSB-to-RO mapping in a cell, the UE needs to know the information about the TDD pattern configuration of a candidate cell.

In the legacy RACH-based L3 mobility, within the ReconfigurationWithSync IE, the configurations for the RACH resources (e.g., RACH-ConfigDedicated for contention-free case) and the TDD pattern (tdd-UL-DL-ConfigCommon in ServingCellConfigCommon) of the target cell are provided [1]:

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon       ServingCellConfigCommon                            OPTIONAL,   -- Need M

newUE-Identity           RNTI-Value,

t304                     ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated     CHOICE {

uplink                   RACH-ConfigDedicated,

supplementaryUplink      RACH-ConfigDedicated

}                                                                           OPTIONAL,   -- Need N

...

}

In LTM, as discussed in Section 1, the RACH resources for the early UL synchronization are provided in the EarlyUL-SyncConfig IE, separated from the full RRC reconfiguration message of the LTM candidate cell. However, any information on the TDD pattern of the LTM candidate cell, which is necessary to validate the PRACH occasions for the early UL synchronization, is not separately provided to the UE. Although the UE may acquire it by decoding the system information (i.e., ServingCellConfigCommonSIB) or parsing the full RRC reconfiguration message of the candidate cell before the actual cell switch, these are not viable options for LTM.

Observation 2: In the current LTM configuration, the information about the TDD pattern configuration of an LTM candidate cell cannot be provided outside the RRC configuration of the candidate cell.

Iti is clear that there is a critical problem with the current LTM candidate cell configuration. The logical solution is to add the target cell TDD pattern configurations directly in the LTM-Candidate IE:

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                     LTM-CandidateId-r18,

ltm-CandidatePCI-r18                    PhysCellId                         OPTIONAL,    -- Need M

ltm-SSB-Config-r18                      LTM-SSB-Config-r18                 OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                 OCTET STRING (CONTAINING RRCReconfiguration)
                                                                               OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                  ENUMERATED {true}                  OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18              OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)
                                                                               OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18           OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)
                                                                               OPTIONAL,    -- Need R

ltm-tdd-UL-DL-ConfigurationCommon-r18   TDD-UL-DL-ConfigCommon             OPTIONAL,    -- Cond TDD

ltm-TCI-Info-r18                        LTM-TCI-Info-r18                   OPTIONAL,    -- Need M

ltm-NoResetID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)
                                                                               OPTIONAL,    -- Need N

ltm-UE-MeasuredTA-ID-r18                INTEGER (1..maxNrofLTM-Configs-plus1-r18)
                                                                               OPTIONAL,    -- Need N

...

}

Proposal 1: RAN2 to agree adding a new IE ltm-tdd-UL-DL-ConfigurationCommon-r18 in LTM-Candidate-r18.

If Proposal 1 is agreed, RAN1 needs to update TS 38.213 Clause 8.1 regarding the validation of RACH occasions

Proposal 2: RAN2 to send an LS to RAN1 informing about the new IE and requesting them to udpate their specs accordingly.

# Conclusion

In this contribution, we discussed a critical issue for the RACH configuration for LTM and have the following observations and proposals:

Observation 1: To identify valid PRACH occasions for SSB-to-RO mapping in a cell, the UE needs to know the information about the TDD pattern configuration of a candidate cell.

Observation 2: In the current LTM configuration, the information about the TDD pattern configuration of an LTM candidate cell cannot be provided outside the RRC configuration of the candidate cell.

Proposal 1: RAN2 to agree adding a new IE ltm-tdd-UL-DL-ConfigurationCommon-r18 in LTM-Candidate-r18.

Proposal 2: RAN2 to send an LS to RAN1 informing about the new IE and requesting them to udpate their specs accordingly.

# References

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

- 3GPP TS 38.213: "NR; Physical Layer Procedures for control".