3GPP TSG-RAN WG2 Meeting #127	R2-2407376

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.3.2.1

Source:	InterDigital Inc.

Title:	Initial simulation results for RRM measurement predictions

Document for:	Discussion

# 1. Introduction

In RAN2-126, several agreements were made regarding the simulation assumption for the RRM measurement predictions, and the following prioritization were made regarding the different scenarios [1]:

Table 1: Evaluation Scenario Combinations

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In addition, three RRM prediction sub-cases were introduced [2]:

- Sub-case 1: To predict beam level results, then generate cell level results based on the predicted beam results

- Sub-case 2: To directly predict cell level results based on cell level results

- Sub-case 3: Directly predict cell level results based on beam level results

In this contribution, we provide some initial simulation results for the highlighted scenarios in the table above (case 2 and case 4) , focusing on sub-case 1 and sub-case 3 as indirect and direct approaches for predicting L3 cell-level measurements.

# 2. Discussion

### 2.1 Intra-cell intra-frequency FR1 to FR1 temporal domain case B

In this simulation case, we consider predicting L3 cell-level RSRPs directly and indirectly according to sub-cases 1 and 3 where:

- Sub-case 1 (Indirect Approach): L1 beam-level RSRPs from 3 time instances (T-3, T-2, and T-1) are input to AIML model to predict L1 beam-level RSRPs at time instance T. Then, L3 cell-level RSRP at time instance T is evaluated using the L1 beam-level RSRPs at time instance T that were obtained from the AIML model.

- Sub-case 3 (Direct Approach): L1 beam-level RSRPs from 3 time instances (T-3, T-2, and T-1) are input to AIML model to predict L3 cell-level RSRP at time instance T directly.

We consider a sampling period of 20 ms with 50% measurement reduction rate. For predicting cell-level or beam-level RSRPs at time instance T, we use input from 3 previous time instances. It is necessary to use input from multiple time instances to enable the AIML model to learn information about UE trajectory. The RSRPs at time instances T-3 and T-1 are measured RSRPs, while RSRPs at time instance T-2 are predicted RSRPs from a previous iteration using the same AIML model (i.e., given RSRPs from time instances T-5, T-4, T-3).

Note that we assume that there are 3 time instances with measured RSRPs only in the first iteration of using the AIML model, however, we exclude this iteration from evaluation KPI calculations for consistency with the following iterations that use a combination of 2 time instances with measured RSRPs and 1 time instance with predicted RSRPs.

Simulation parameters from [2] and [3] are listed in Table A.1. Evaluation results are shown in Table 2.

Table 2: Evaluation results for intra-cell intra-frequency FR1 to FR1 temporal domain case B

| Parameters | Sub-case 3, 30 Km/h | Sub-case 1, 30 Km/h | Sub-case 3, 60 Km/h | Sub-case 1, 60 Km/h |
| --- | --- | --- | --- | --- |
| UE trajectory option [2] | Option 1 | Option 1 | Option 1 | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 | Option 3 | Option 3 | Option 3 |
| UE speed | 30 Km/h | 30 Km/h | 60 Km/h | 60 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A | N/A | N/A | N/A |
| Measurement reduction rate | 50% | 50% | 50% | 50% |
| Measurement window | N/A | N/A | N/A | N/A |
| Prediction window | N/A | N/A | N/A | N/A |
| Any other parameters | N/A | N/A | N/A | N/A |
| Number of training examples | 14700 | 14700 | 14700 | 14700 |
| Number of validation examples | 2100 | 2100 | 2100 | 2100 |
| Number of testing examples | 4200 | 4200 | 4200 | 4200 |
| AIML model input | L1 Beam-level RSRPs from 3 time instances T-3, T-2, T-1 | L1 Beam-level RSRPs from 3 time instances T-3, T-2, T-1 | L1 Beam-level RSRPs from 3 time instances T-3, T-2, T-1 | L1 Beam-level RSRPs from 3 time instances T-3, T-2, T-1 |
| AIML model output | L3 Cell-level RSRPs at 1 time instance T | L1 Beam-level RSRPs at 1 time instance T (then L3 cell-level RSRP is evaluated) | L3 Cell-level RSRPs at 1 time instance T | L1 Beam-level RSRPs at 1 time instance T (then L3 cell-level RSRP is evaluated) |
| AIML model type | ResNet | ResNet | ResNet | ResNet |
| AIML model number of parameters (M) | 2.65 M | 2.65 M | 2.65 M | 2.65 M |
| Average L3 cell-level RSRP difference (dB) | 0.9744 dB | 0.4581 dB | 1.1434 dB | 0.5832 dB |
| Average L1 beam-level RSRP difference (dB) | N/A | 0.8157 dB | N/A | 0.9358 dB |

We notice that indirect L3 cell-level RSRP prediction via an AIML model that predicts L1 beam-level RSRPs (i.e., sub-case 1) achieves a better performance than direct prediction (i.e., sub-case 3). This is because the direct AIML model is required to predict a filtered L3 cell-level measurement given partial previous history (i.e., information from only 3 previous time instances). The performance of the direct model may be enhanced by providing either L1 beam-level RSRP input from more previous instances or previous measured and/or predicted L3 cell-level measurements. We also notice that the AIML model achieve a lower prediction error as the UE speed becomes lower.

On the other hand, if we focus on the first two iterations of inference at the AIML model, we notice the following:

- Iteration 1 (Excluded from KPI evaluation): beam-level or cell-level RSRPs at time instance T3 are evaluated using measured RSRPs at time instances T0, T1, and T2 (Input Window = 3 time instances)

- Iteration 2 (and similarly following iteration): beam-level or cell-level RSRPs at time instance T5 are evaluated using measured RSRPs at time instances T2 and T4 along with predicted RSRPs at time instance T3 from iteration 1 (Input Window = 3 time instances)

| Parameters | Sub-case 3, 30 Km/h | Sub-case 1, 30 Km/h | Sub-case 3, 60 Km/h | Sub-case 1, 60 Km/h |
| --- | --- | --- | --- | --- |
| Average L3 cell-level RSRP difference (dB) at 1st iteration | 0.9218 dB | 0.3697 dB | 0.9956 dB | 0.4803 dB |
| Average L3 cell-level RSRP difference (dB) at 2nd iteration | 0.9787 dB | 0.4300 dB | 1.1426 dB | 0.5524 dB |

As expected, we notice that prediction error in the first iteration where only measured RSRPs were used as input is less than the prediction error using both measured and predicted RSRPs in the second iteration.

Observation 1: For temporal domain case B, as UE speed increases, the average L3 cell-level RSRP difference increases, i.e., AIML model prediction accuracy decreases for both direct and indirect prediction.

Observation 2: For temporal domain case B, the indirect prediction approach outperforms the direct prediction approach when given the same input (i.e., only L1 beam-level RSRPs) since the direct approach tries to predict a value that depends on a long history (due to L3 filtering impact) using partial information on that history (i.e., a few time instances).

Proposal 1: RAN2 to study the impact of how the measured RSRPs and predicted RSRPs are combined in cell level measurement derivation.

Proposal 2: RAN2 to study the impact of different input window lengths in temporal domain case B.

### 2.2 Intra-cell intra-frequency FR2 to FR2 temporal domain case A

In this simulation case, we consider predicting L3 cell-level RSRPs directly according to sub-case 3 [2] where:

- Sub-case 3 (Direct Approach): L1 beam-level RSRPs from 20 time instances (T-20, T-19, …, T-1) are input to AIML model to predict L3 cell-level RSRP for time instances (T, T+1, …, T+19) directly.

We consider a sampling period of 20 ms, a measurement/observation window of 200 ms and a prediction window of 200 ms. For predicting cell-level RSRPs at time instances (T, T+1, …, T+19), we use input from 20 previous time instances (T-20, T-19, …, T-1) as shown in figure below. We evaluate the average L3 cell-level RSRP difference for each time instance in the prediction window.

Simulation parameters from [2] and [3] are listed in Table A.2. Evaluation results are described in Table 3 and the figure below.

Table 3: Evaluation results for intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Parameters | Sub-case 3, 60 Km/h | Sub-case 3, 120 Km/h |
| --- | --- | --- |
| UE trajectory option [2] | Option 1 | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 | Option 3 |
| UE speed | 60 Km/h | 120 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A | N/A |
| Measurement reduction rate | N/A | N/A |
| Measurement window | 400 ms | 400 ms |
| Prediction window | 400 ms | 400 ms |
| Any other parameters | N/A | N/A |
| Number of training examples | 14334 | 14574 |
| Number of validation examples | 2047 | 2082 |
| Number of testing examples | 4094 | 4162 |
| AIML model input | L1 Beam-level RSRPs from 20 time instances T-20, T-19, …, T-1 | L1 Beam-level RSRPs from 3 time instances T-20, T-19, …, T-1 |
| AIML model output | L3 Cell-level RSRPs at 20 time instances T, T+1, …, T+19 | L3 Cell-level RSRPs at 20 time instances T, T+1, …, T+19 |
| AIML model type | ResNet | ResNet |
| AIML model number of parameters (M) | 0.86 M | 0.86 M |
| Average L3 cell-level RSRP difference (dB) | Refer to figure | Refer to figure |
| Average L1 beam-level RSRP difference (dB) | N/A | N/A |

We notice that the AIML model achieve a lower prediction error as the UE speed becomes lower. We also note that a longer prediction window can be considered as the UE speeds gets lower given a fixed difference error requirement. For example, if it is required that the average L3 cell-level RSRP difference should not exceed 1 dB, the prediction window (at UE speed= 60 Km/h) will be 17*20ms = 340 ms, while it will be only 8*20ms = 160ms only at UE speed= 120 Km/h.

Observation 3: For temporal domain case A, as UE speed increases, the average L3 cell-level RSRP difference increases, i.e., AIML model prediction accuracy decreases.

Observation 4: For temporal domain case A, the prediction window can be determined based on UE speed. As UE speed gets higher, a shorter prediction window may be considered to meet a certain prediction error requirement.

Proposal 3: RAN2 to study the impact of prediction performance requirement on the length of prediction window at different UE speeds in temporal domain case A.

# 4. Conclusion

In this contribution, initial simulation results for the RRM measurement prediction use case were discussed and the following observations and proposals were made:

Observation 1: For temporal domain case B, as UE speed increases, the average L3 cell-level RSRP difference increases, i.e., AIML model prediction accuracy decreases for both direct and indirect prediction.

Observation 2: For temporal domain case B, the indirect prediction approach outperforms the direct prediction approach when given the same input (i.e., only L1 beam-level RSRPs) since the direct approach tries to predict a value that depends on a long history (due to L3 filtering impact) using partial information on that history (i.e., a few time instances).

Observation 3: For temporal domain case A, as UE speed increases, the average L3 cell-level RSRP difference increases, i.e., AIML model prediction accuracy decreases.

Observation 4: For temporal domain case A, the prediction window can be determined based on UE speed. As UE speed gets higher, a shorter prediction window may be considered to meet a certain prediction error requirement.

Proposal 1: RAN2 to study the impact of how the measured RSRPs and predicted RSRPs are combined in cell level measurement derivation.

Proposal 2: RAN2 to study the impact of different input window lengths in temporal domain case B.

Proposal 3: Study the impact of prediction performance requirement on the length of prediction window at different UE speeds in temporal domain case A

# 5. References

[1]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2]	R2-2404955: Summary of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO)

[3]	R2-2406095: Summary of [POST126][031][AIMob] Simulations

[4]	TR 38.901: Study on channel model for frequencies from 0.5 to 100 GHz (3GPP TR 38.901 version 16.1.0 Release 16)

# Appendix

### A.1 Simulation assumptions of Intra-cell intra-frequency FR1 to FR1 temporal domain case B

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 – 4 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 500 m |
| Channel model | According to TR 38.901 [4] |
| System BW | 20 MHz |
| Sub-carrier Spacing | 30 kHz |
| UE Speed | 30 Km/h, 60 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,1,1), (dH,dV) = (0.5, 0.8)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (2,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44 dBm |
| UE receiver Noise Figure | 9 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [4] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| nrofSS-BlocksToAverage | 1 |
| absThreshSS-BlocksConsolidation | -110 dBm |
| Number of Beams | 8 Tx Beams (4 Azimuth angles x 2 Elevation angles) |

### A.2 Simulation assumptions of Intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 – 30 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 200 m |
| Channel model | According to TR 38.901 [4] |
| System BW | 80 MHz |
| Sub-carrier Spacing | 120 kHz |
| UE Speed | 60 Km/h, 120 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (4,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | 2 panels (left, right) |
| BS Tx Power | 40 dBm |
| UE receiver Noise Figure | 10 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [4] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| nrofSS-BlocksToAverage | 3 |
| absThreshSS-BlocksConsolidation | -110 dBm |
| Number of Beams | 64 Tx Beams (16 Azimuth angles x 4 Elevation angles) |