3GPP TSG-RAN WG2 Meeting #127	R2-2407377

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.3.2.2

Source:	InterDigital Inc.

Title:	Other aspects of RRM measurement predictions

Document for:	Discussion

# 1. Introduction

In RAN2-126, several agreements were made regarding the simulation assumption for the RRM measurement predictions [1].

In this contribution, we provide discussion regarding on beam level prediction considerations and comparison of beam level measurement prediction and cell level measurement prediction.

# 2. Discussion

## 2.1 Beam level measurement prediction

A UE may be configured to include beam level results in the measurement reports (e.g., measurement results of the X best beams). The cell level measurement reports will help the network identify the best cell to hand over the UE to, and the beam level measurements associated with that best cell could be used by the network to determine which beam to assign to that UE at the target upon handover.

Our understanding is that the inter-cell beam prediction that is part of the objective of this study item is not just for the sake of cell level measurement derivation as in RRM use case 1, but also for identifying the best beam(s) in the target cell.

Observation 1: For RRM use case 1, the predicted beams could be used not only as input for the derivation of the cell level measurements, but also to identify the best beam.

In the email discussion [2][3], there seems to be a consensus that for the cell level measurement results, RSRP difference between actual and predicted values is the key metric/KPI to be used for evaluating prediction results (even though there are different preferences regarding the details of that, for example, if the average RSRP difference is sufficient or the full RSRP difference CDF should be considered).

For the case the beam level predictions that are going to be used to identify the top beam(s) for the target cell, we think it is sufficient to adopt the same metric that is used for the beam prediction use case in the RAN1 AIML work, as captured in the AIML PHY TR [4]. Specifically, we recommend the following KPIs to be considered:

- Average L1-RSRP difference of Top-1 predicted beam:

- Top-1 (%): the percentage of "the Top-1 genie-aided beam is Top-1 predicted beam"

- Top-K/1 (%): the percentage of "the Top-1 genie-aided beam is one of the Top-K predicted beams"

Proposal 1: For the evaluation of the beam predictions, RAN2 to adopt the KPIs/metrics used in the RAN1 work on beam prediction (e.g., average L1-RSRP difference of Top-1 predicted beam, Top-1(%), Top-K/1(%)).

## 2.2 Comparison of beam prediction KPIs and cell level KPIs

We consider intra-cell intra-frequency FR1 to FR1 temporal domain case B evaluations in [5], where L3 cell-level RSRP is directly and indirectly predicted from L1 beam-level RSRPs according to sub-cases 1 and 3 as follows:

- Sub-case 1 (Indirect Approach): Past L1 beam-level RSRPs are input to AIML model to predict future L1 beam-level RSRPs. Then, future L3 cell-level RSRP is evaluated.

- Sub-case 3 (Direct Approach): Past L1 beam-level RSRPs are input to AIML model to predict future L3 cell-level RSRP directly.

We compare the average RSRP difference of L1 beam-level prediction and L3 cell-level prediction as shown in the following table.

Table 1: Evaluation results for intra-cell intra-frequency FR1 to FR1 temporal domain case B

| Parameters | Sub-case 3, 30 Km/h | Sub-case 1, 30 Km/h | Sub-case 3, 60 Km/h | Sub-case 1, 60 Km/h |
| --- | --- | --- | --- | --- |
| AIML model input | L1 Beam-level RSRPs from 3 time instances T-3, T-2, T-1 | L1 Beam-level RSRPs from 3 time instances T-3, T-2, T-1 | L1 Beam-level RSRPs from 3 time instances T-3, T-2, T-1 | L1 Beam-level RSRPs from 3 time instances T-3, T-2, T-1 |
| AIML model output | L3 Cell-level RSRPs at 1 time instance T | L1 Beam-level RSRPs at 1 time instance T (then L3 cell-level RSRP is evaluated) | L3 Cell-level RSRPs at 1 time instance T | L1 Beam-level RSRPs at 1 time instance T (then L3 cell-level RSRP is evaluated) |
| AIML model type | ResNet | ResNet | ResNet | ResNet |
| Average L3 cell-level RSRP difference (dB) | 0.9744 dB | 0.4581 dB | 1.1434 dB | 0.5832 dB |
| Average L1 beam-level RSRP difference (dB) | N/A | 0.8157 dB | N/A | 0.9358 dB |

We note that AIML models that predict L1 beam-level measurements achieve lower RSRP estimation error than AIML models that predict L3 cell-level measurement if the input to both is L1 beam-level measurements. This is because the AIML model that predicts L3 cell-level measurement is required to predict a filtered L3 cell-level measurement given partial previous history (i.e., information from limited previous time instances) [5].

Also, when we compare indirect prediction at different UE speeds, lower average L1 beam-level RSRP difference indicates a lower average L3 cell-level RSRP difference.

Observation 2: For temporal prediction case B, the RSRP estimation error for L3 cell level measurements is lower than the L1 beam measurements.

# 3. Conclusion

In this contribution, initial simulation results for the RRM measurement prediction use case were discussed and the following observations and proposals were made:

Observation 1: For RRM use case 1, the predicted beams could be used not only as input for the derivation of the cell level measurements, but also to identify the best beam.

Observation 2: For temporal prediction case B, the RSRP estimation error for L3 cell level measurements is lower than the L1 beam measurements.

Proposal 1: For the evaluation of the beam predictions, RAN2 to adopt the KPIs/metrics used in the RAN1 work on beam prediction (e.g., average L1-RSRP difference of Top-1 predicted beam, Top-1(%), Top-K/1(%)).

# 4. References

[1] RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2] R2-2404955, Summary of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO),

[3] R2-2406095, Summary of [POST126][031][AIMob] Simulations, OPPO, Fukuoka, Japan, May 2024

[4] TS 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[5] R2-2407377, Initial simulation results for RRM measurement predictions, RAN2-127, Maastricht, Netherlands, August 2024