3GPP TSG-RAN2 Meeting #127	R2-2407379

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on RRM enhancements for NR FR2 HST

Agenda item:	7.25.1

Document for: 	Discussion and Decision

- Introduction

In the RAN2 #125bis meeting, we sent an LS to RAN4 to inquiry that whether the presence of highSpeedMeasFlagFR2 should be the same for all serving cells in the same frequency band. Now, we receive the reply, and the content is as follows [1]:

RAN4 extensively discussed the issue raised by RAN2 and agreed that: - In FR2 HST, gNB should enable RRM enhancements for all serving cells in the same frequency band (i.e. intra-band CA) in consistent manner, i.e., - The presence of highSpeedMeasFlagFR2 should be the same for all serving cells in the same frequency band - If highSpeedMeasFlagFR2 is present, the configured parameters for FR2 HST, i.e., set 1 or set 2, should be the same for all serving cells in the same frequency band

In this contribution, we’d like to share our views on this.

- Discussion

In the current specification, the field description of highSpeedMeasFlagFR2 is as follows:

highSpeedMeasFlagFR2 If the field is present and UE supports ue-PowerClass-v1700 set to pc6, the UE shall apply enhanced intra-frequency RRM requirement to the serving frequency of SpCell to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14]. If the field is present and the UE supports enhanced inter-frequency RRM requirements for FR2 HST in RRC_IDLE and RRC_INACTIVE, the UE shall apply enhanced interfrequency RRM requirement to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_IDLE and RRC_INACTIVE. If the field is present for SpCell and the UE supports measEnhCAInterFreqFR2-r18, the UE shall apply enhanced inter-frequency RRM requirement to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_CONNECTED. If the field is present for SCell(s) and and the UE supports measEnhCAInterFreqFR2-r18, the UE shall apply enhanced intra-frequency RRM requirements to the serving frequency of the corresponding SCell to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_CONNECTED. The field value, set1 or set2, is applied as specified in TS38.133 [14].

Based on the highlighted part, the enhanced RRM requirements for intra-freq measurement on SCell are configured by the flag highSpeedMeasFlagFR2 of each SCell.

Observation 1: In the current specification, the enhanced RRM requirements for intra-freq measurement on SCC are configured by the flag highSpeedMeasFlagFR2 of each SCell.

In the RAN2 125bis meeting, we sent an LS to RAN4 to ask whether gNB should enable RRM enhancements for all serving cells in the same frequency band in consistent manner. And now, we get the reply. Based on RAN4’s reply, the presence of highSpeedMeasFlagFR2 and the configured value (set 1 or set 2) for all serving cells in the same band should be the same. But this restriction has not been captured by the RAN2 specification.

Observation 2: Based on RAN4’s reply, the presence of highSpeedMeasFlagFR2 and the configured value (set 1 or set 2) for all serving cells in the same band should be the same, which is not captured by RAN2 specification.

Therefore, we suggest to update the field description of highSpeedMeasFlagFR2 to capture that the presence of highSpeedMeasFlagFR2 and the configured value (set 1 or set 2) for all serving cells in the same band should be the same.  The corresponding TP can be seen below:

highSpeedMeasFlagFR2 If the field is present and UE supports ue-PowerClass-v1700 set to pc6, the UE shall apply enhanced intra-frequency RRM requirement to the serving frequency of SpCell to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14]. If the field is present and the UE supports enhanced inter-frequency RRM requirements for FR2 HST in RRC_IDLE and RRC_INACTIVE, the UE shall apply enhanced interfrequency RRM requirement to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_IDLE and RRC_INACTIVE. If the field is present for SpCell and the UE supports measEnhCAInterFreqFR2-r18, the UE shall apply enhanced inter-frequency RRM requirement to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_CONNECTED. If the field is present for SCell(s) and and the UE supports measEnhCAInterFreqFR2-r18, the UE shall apply enhanced intra-frequency RRM requirements to the serving frequency of the corresponding SCell to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_CONNECTED. The presence and configured value for all serving cells in the same band should be the same. The field value, set1 or set2, is applied as specified in TS38.133 [14].

Proposal 1: Update the field description of highSpeedMeasFlagFR2 to capture that the presence of highSpeedMeasFlagFR2 and the configured value (set 1 or set 2) for all serving cells in the same band should be the same.

Proposal 2: Adopt the TP above.

- Conclusion

Based on the above discussions, we have the following observations and proposals:

Observation 1: In the current specification, the enhanced RRM requirements for intra-freq measurement on SCC are configured by the flag highSpeedMeasFlagFR2 of each SCell.

Observation 2: Based on RAN4’s reply, the presence of highSpeedMeasFlagFR2 and the configured value (set 1 or set 2) for all serving cells in the same band should be the same, which is not captured by RAN2 specification.

Proposal 1: Update the field description of highSpeedMeasFlagFR2 to capture that the presence of highSpeedMeasFlagFR2 and the configured value (set 1 or set 2) for all serving cells in the same band should be the same.

Proposal 2: Adopt the TP above.

- References

[1] R4-2410285  Reply LS to RAN2 on RRM enhancements for NR FR2 HST (R4-2410285, Contact: Samsung)  Rel-18 NR_HST_FR2_Enh