3GPP TSG-RAN2 Meeting #127	R2-2407380

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on ATG

Agenda item:	7.25.1

Document for: 	Discussion and Decision

- Introduction

In the last meeting, companies discussed how to handle switching of SCS via BWP switching for TA reporting, but no conclusion was achieved [1]. In this contribution, we’d like to share our views on it.

- Discussion

In current specification, the configuration/parameter for ATG TA reporting are in the unit of symbols, whose absolute value will change with the SCS change due to BWP swtiching. These configuration/parameter include:

- offsetThresholdTA-r18 in TAR-Config

- Timing Advance field in TAR MAC CE

The offsetThresholdTA-r18 is used to indicate the variation threshold between the current estimate of the TA value and the last reported TA value. If the SCS changes due to BWP switching, the absolute value of offsetThresholdTA-r18 is also changed. If the BWP switching is not triggered via RRC signaling, the RRC layer is unclear that which BWP is activated. So, when the absolute value of offsetThresholdTA-r18 changes, the network may not be able to re-configure it.

Observation 1: The network can not re-configure offsetThresholdTA-r18 to keep the absolute value unchanged in time if the BWP change is not triggered via RRC.

The SCS of ATG system can only be 15kHz or 30kHz. If the offsetThresholdTA-r18 is configured in the case of 15kHz SCS, when the SCS changes to 30kHz, the absolute value of variation threshold is changed to 1/2 of the original value, which results in unnecessary TA reporting. If the offsetThresholdTA-r18 is configured in the case of 30kHz SCS, when the SCS changes to 15kHz, the absolute value of variation threshold is changed to twice of the original value, which causes  TA reporting is not triggered and TA value can not be adjusted by the network in time.

Observation 2: The change of SCS will cause unnecessary TA reporting (15kHz--> 30kHz) or too late TA reporting (30kHz--> 15kHz).

- For Timing Advance field in TAR MAC CE, it is used to report the current estimiated TA value to the network. If the BWP change occurs when TAR MAC CE is sent or re-sent, the reported timing advance value may be ambiguous for the network.

- Observation 3: With the unit of symbol, the reported timing advance value may be ambiguous for the network.

- To solve the problem above, the following solutions can be considered:

- 1)  Update the unit to the symbol with SCS of initial BWP;

- 2)  Update the unit to the symbol with SCS of 30kHz;

- With 1), the network needs to check the SCS of the initial BWP before generating configuration, and the UE also needs to check the SCS of the initial BWP when appling the network configuration. This increases implementation complexity. So, 2) is preferred. We provide the corresponding TP in the Annex.

- Observation 4: In order to handle the absolute value change of TA reporting configuration/parameter due to SCS switching, consider the following solutions:

- 1)  Update the unit to the symbol with SCS of initial BWP;

- 2)  Update the unit to the symbol with SCS of 30kHz;

- Proposal 1: Suggest to update that the unit of offsetThresholdTA-r18 in TAR-Config and Timing Advance field in TAR MAC CE is symbol with 30kHz SCS.

- Proposal 2: Adopt the TP in the Annex.

- Conclusion

Based on the above discussions, we have the following observations and proposals:

Observation 1: The network can not re-configure offsetThresholdTA-r18 to keep the absolute value unchanged in time if the BWP change is not triggered via RRC.

Observation 2: The change of SCS will cause unnecessary TA reporting (15kHz--> 30kHz) or too late TA reporting  (30kHz--> 15kHz).

- Observation 3: With the unit of symbol, the reported timing value may be ambiguous for the network.

- Observation 4: In order to handle the absolute value change of TA reporting configuration/parameter due to SCS switching, consider the following solutions:

- 1)  Update the unit to the symbol with SCS of initial BWP;

- 2)  Update the unit to the symbol with SCS of 30kHz;

- Proposal 1: Suggest to update that the unit of offsetThresholdTA-r18 in TAR-Config and Timing Advance field in TAR MAC CE is symbol with 30kHz SCS.

- Proposal 2: Adopt the TP in the Annex.

- References

[1] Report of 3GPP TSG RAN WG2 meeting #126

- Annex

## 5.1 TP for offsetThresholdTA-r18

–	TAR-Config

The IE TAR-Config is used to configure Timing Advance reporting in non-terrestrial networks and ATG network.

TAR-Config information element

-- ASN1START

-- TAG-TAR-CONFIG-START

TAR-Config-r17 ::=                      SEQUENCE {

offsetThresholdTA-r17               ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, ms11, ms12,

ms13, ms14, ms15, spare13, spare12, spare11, spare10, spare9, spare8, spare7,

spare6, spare5, spare4, spare3, spare2, spare1}          OPTIONAL,    -- Need R

timingAdvanceSR-r17                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

TAR-Config-r18 ::=                      SEQUENCE {

offsetThresholdTA-r18               INTEGER (1..56)                                                     OPTIONAL,    -- Need R

timingAdvanceSR-r18                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

-- TAG-TAR-CONFIG-STOP

-- ASN1STOP

TAR-Config field descriptions

offsetThresholdTA Offset for TA reporting as specified in TS 38.321 [3]. Network only configures this parameter for MCG. For ATG, network only configures offsetThresholdTA-r18, which is in unit of symbols using subcarrier spacing of 30kHz.

timingAdvanceSR Used to configure whether a Timing Advance report may trigger a Scheduling Request as specified in TS 38.321 [3].

- 5.2 TP for Timing Advance field in TAR MAC CE

6.1.3.56	Timing Advance Report MAC CE

The Timing Advance Report MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-2. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.56-1):

-	R: Reserved bit, set to 0;

-	Timing Advance: Except for ATG in FR1, the Timing Advance field indicates the least integer number of slots, using subcarrier spacing of 15 kHz, greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1).

For ATG in FR1, the Timing Advance field indicates the least integer number of symbols, using subcarrier spacing of 30kHz, greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). The symbol duration is based on the subcarriers spacing the UE is currently configured with. In this release of the specification, only 15 kHz and 30 kHz SCS are applicable and only values 1 … 56 are used.

The length of the field is 14 bits.

Figure 6.1.3.56-1: Timing Advance Report MAC CE