3GPP TSG-RAN WG2 Meeting #127	R2-2407386

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	vivo

Title:	MRO for Rel-18 mobility features

Agenda Item:	8.10.2

Document for:	Discussion and Decision

- Introduction

At the RAN2#126 meeting, RAN2 reached the following agreements regarding the MRO for Rel-18 mobility features:

LTM If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD. Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field. Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure. CHO with candidate SCG RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report. Subsequent CPAC RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

In this contribution, we will provide further analysis of the remaining issues.

- Discussion

  - LTM

At the RAN2#126 meeting, it was agreed to log the LTM cell ID upon performing LTM-based recovery. However, whether to reuse the cell ID for CHO or introduce a new one is FFS.

The field of choCellID is as follows. As another HO type will be introduced for LTM, it is feasible to reuse the choCellID field from ASN.1 perspective.

choCellId-r17 CHOICE { cellGlobalId-r17 CGI-Info-Logging-r16, pci-arfcn-r17 PCI-ARFCN-NR-r16 } OPTIONAL,

choCellId This field is used to indicate the candidate target cell for conditional handover included in condRRCReconfig that the UE selected for CHO based recovery while T311 is running.

However, reusing the field may cause confusion as the field name includes CHO. Besides, only one extra bit is needed if we introduce a separate field. Therefore, we prefer to introduce a separate field to indicate the LTM cell ID in LTM-Candidate that the UE selected for LTM-based recovery.

Proposal 1: Introduce a separate field to indicate the LTM cell ID in LTM-Candidate that the UE selected for LTM-based recovery.

Another aspect that should be considered is RACH-less HO in LTM. Specifically, two approaches for UE to obtain the TA during RACH-less HO are supported:

- Early sync: the UE sends a preamble to the target cell and the target cell forwards the TA value to the source cell. Upon LTM HO is triggered, the source cell indicates the TA value to UE in the LTM cell switch command.

- UE-based TA management: NW indicates UE whether UE-based TA measurement can be supported, and UE performs UE-based TA measurement (e.g., get the TA value of the target cell based on the RSTD between the source cell and target cell).

For the first approach, the TA may be invalid due to the UE moves after performing the early sync. In this case, the NW may optimize the early sync configuration, e.g., configure more frequent early sync or use RACH-based HO.

For the second approach, if the source cell and the candidate target cell are out of sync, the UE-based TA management may lead to HO failure. In this case, the NW may optimize the UE-based TA management configuration, e.g., adjust the ltm-ServingCellUE-MeasuredTA-ID associated with the cells.

To differentiate the above two approaches, how UE obtains the TA during RACH-less HO should be indicated in the RLF report.

Proposal 2: Record how UE obtains the TA during RACH-less HO in the RLF report, e.g., Early-sync, UE-based TA management.

  - CHO with candidate SCG

At the RAN2#125bis meeting, it was agreed to study failure and near-failure scenarios for CHO with candidate SCGs.

RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

2.2.1	Failure CHO with candidate SCGs

For CHO with candidate SCG, UE cannot execute CHO with candidate SCG(s) until the execution conditions for both the candidate PCell and the associated candidate PSCell are met. Non-optimal configuration of CHO with candidate SCGs may result in MCG/SCG failure due to UE not executing CHO with candidate SCG(s) if only PCell’s execution condition is fulfilled or only associate PSCell’s execution condition is fulfilled.

For MCG failure, the SON for CHO in the RLF report can be taken as the baseline for enhancement.

[[ choCandidate-r17 ENUMERATED {true} OPTIONAL, choConfig-r17 SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16 OPTIONAL, triggeredEvent-r17 SEQUENCE { timeBetweenEvents-r17 TimeBetweenEvent-r17 OPTIONAL, firstTriggeredEvent-r17 ENUMERATED {condFirstEvent, condSecondEvent} OPTIONAL } OPTIONAL ]],

choCandidate This field indicates whether the associated cell is a candidate target cell for conditional handover or conditional PSCell change or addition. This field may be included only in the SuccessHO-Report or SuccessPSCell-Report within UEInformationResponse message.

choConfig If the associated cell is a candidate target cell for conditional handover, this field indicates the conditional handover execution condition for each measId within condTriggerConfig associated to the cell. This field may be included only in the rlf-report within UEInformationResponse message.

firstTriggeredEvent This field is set to condFirstEvent if the execution condition associated to the first entry of choConfig was fulfilled first in time. This field is set to condSecondEvent if the execution condition associated to the second entry of choConfig was fulfilled first in time. This field may be included in rlf-report within UEInformationResponse message or in SCGFailureInformation message.

timeBetweenEvents Indicates the time elapsed between fulfilling the conditional execution conditions included in choConfig. Value in milliseconds. The maximum value 1023 means 1023ms or longer. This field may be included in the reports associated to UEInformationResponse message, e.g., rlf-Report or in the SCGFailureInformation message.

From our understanding, the following information should be included in the MeasResultNR in RLF report:

- The CPAC configuration. Similar to CHO configuration, the CPAC configuration should also be included.

- The association between the candidate PCell for CHO and candidate PSCell for CPAC. With the association, the network may figure out whether to adjust the configuration of CHO or CPAC.

Proposal 3: For the MCG failure scenario of CHO with candidate SCGs, introduce the following information in the MeasResultNR within RLF report:

- CPAC configuration.

- Association between the candidate PCell for CHO and candidate PSCell for CPAC.

For SCG failure, the UE will send the SCGFailureInformation message to the serving gNB. As the serving gNB has the configuration of CHO with candidate SCGs, with the measurement in the SCGFailureInformation, the serving gNB can optimize the configuration of CHO with candidate SCGs. However, we are open to the essential enhancement, if any. Thus no proposal for SCG failure scenario from our perspective.

2.2.2	Near-failure CHO with candidate SCGs

From the NW perspective, the NW expects T1 and T2 to be as close as possible, where T1 is the time when PCell's execution condition is satisfied and T2 is the time when the associate PSCell’s execution condition is satisfied. So if PCell changes with PSCell addition/change is successfully executed but if the time difference between T1 and T2 is large, the NW needs to know that situation, UE should report some information to assist the NW in optimizing the configuration.

For the above near-failure scenario, the SHR can be taken as the baseline for enhancement. The most straightforward way is introducing another triggering event for the SHR report, i.e., the time difference between T1 and T2 exceeds a configured threshold. Correspondingly, the time difference between T1 and T2 should be introduced in the SHR. In addition, which one is fulfilled at first should also be indicated.

Proposal 4: Introduce another triggering event for the SHR report, i.e., the time difference between T1 and T2 exceeds a configured threshold, where T1 is the time when PCell's execution condition is fulfilled and T2 is the time when the associate PSCell’s execution condition is fulfilled.

Proposal 5: For the near-failure scenario of CHO with candidate SCGs, introduce the following information in the SHR report:

- Time difference between T1 and T2, where T1 is the time when PCell's execution condition is fulfilled and T2 is the time when the associate PSCell’s execution condition is fulfilled.

- The first fulfilled event, CHO or CPAC.

  - Subsequent CPAC

From our understanding, for failure and near-failure scenarios for subsequent execution of SCPAC, the existing SCGFailureInformation message and SPR can be reused, and no other enhancements are needed. However, we are open to the essential enhancement, if any. Thus no proposal for subsequent CPAC from our perspective.

- Conclusion

Based on the above analysis, we make the following proposals:

LTM

Proposal 1: Introduce a separate field to indicate the LTM cell ID in LTM-Candidate that the UE selected for LTM-based recovery.

Proposal 2: Record how UE obtains the TA during RACH-less HO in the RLF report, e.g., Early-sync, UE-based TA management.

CHO with candidate SCG

Proposal 3: For the MCG failure scenario of CHO with candidate SCGs, introduce the following information in the MeasResultNR within RLF report:

- CPAC configuration.

- Association between the candidate PCell for CHO and candidate PSCell for CPAC.

Proposal 4: Introduce another triggering event for the SHR report, i.e., the time difference between T1 and T2 exceeds a configured threshold, where T1 is the time when PCell's execution condition is fulfilled and T2 is the time when the associate PSCell’s execution condition is fulfilled.

Proposal 5: For the near-failure scenario of CHO with candidate SCGs, introduce the following information in the SHR report:

- Time difference between T1 and T2, where T1 is the time when PCell's execution condition is fulfilled and T2 is the time when the associate PSCell’s execution condition is fulfilled.

- The first fulfilled event, CHO or CPAC.

- Reference