3GPP TSG-RAN WG2 Meeting #127	R2-2407387

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	vivo

Title:	Discussion on RACH optimization for SDT

Agenda Item:	8.10.5

Document for:	Discussion and Decision

- Introduction

At the RAN2#126 meeting, the following agreements were made regarding RACH optimization for SDT:

For the purpose of SON enhancements for SDT, include RSRP/data volume related information Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT. When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

In this contribution, we will provide further analyses of the RACH optimization for SDT.

- Discussion

For SDT, the UE may initiate the SDT if the following criteria are met:

- sdt-Config is configured and SIB1 includes sdt-ConfigCommon;

- all the pending data in UL is mapped to the radio bearers configured for SDT;

- data volume of the pending UL data across all RBs configured for SDT is less than or equal to a threshold;

- RSRP of the downlink pathloss reference is higher than a threshold (if configured).

In Rel-18, RAN2 mainly focuses on RACH report enhancement for RA-SDT:

- UE includes RA and SDT information in RA report when an SDT operation fails.

- Include a single flag indicating whether the SDT failed or not.

#### 5.7.10.4 Actions for the Random Access report determination Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, or upon failed or successfully completed RA-SDT operation as specified in clause 5.3.13.5

To assist the network in optimizing the RSRP threshold, the UE should record the RSRP measurement upon RA-SDT failure in the RA Information. Specifically, if the RSRP threshold is too low, the UE may fail to perform RA-SDT towards the camping cell due to poor signal quality. In this case, the gNB should increase the RSRP threshold for SDT.

Proposal 1: To assist the network in optimizing the RSRP threshold for SDT, upon RA-SDT failure, the UE may record the RSRP measurement in the RA-InformationCommon.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Proposal 1: To assist the network in optimizing the RSRP threshold for SDT, upon RA-SDT failure, the UE may record the RSRP measurement in the RA-InformationCommon.

- Reference