3GPP TSG-RAN WG2 #127	R2-2407390

Maastricht, Netherlands, 19th – 23th August 2024

Agenda Item:	8.13.1

Source:	InterDigital, LG Electronics, FirstNet, Ericsson, AT&T, Qualcomm, Samsung

Title:	Discussion on Working Assumptions for Multi-hop Relay Mechanisms

Document for:	Discussion, Decision

# 1	Introduction

As mentioned in the NR sidelink multiple-hop relay WID [1]:

single hop U2N relays that have been developed in Rel-17 and Rel-18 have limited applicability because of the range limitation of a single hop sidelink relay. Thus, additional sidelink relay enhancements are needed to support some remaining (and new) requirements for Proximity Services captured in TS 22.278 and TS 22.261, e.g. multi-hop support for UE-to-Network Relay.

More specifically, for some public safety use cases [2], mechanisms to support at least two additional hops relays (in Release 19) on top of Rel-17 U2N relay is critical to extend the out-of-network coverage for public safety users.

NR sidelink multiple-hop relay WID [1] also says:

A study item for ProSe phase 3 has been approved in SA in order to investigate further 5G system enhancements to support Proximity Services in Rel-19. RAN-side enhancements for sidelink relay, particularly multi-hop Layer-2 UE-to-Network relay, is necessary in accordance with the SA work.

Work in SA [3] has already investigated and identified 5G system enhancements to support scalable multi-hop relay mechanisms (such as Relay Discovery, Relay (re)selection, and so on) which can serve as basis to develop mechanisms for two additional hops L2 relays (in Release 19) on top of Rel-17/Rel-18 L2 U2N relay.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

In this paper, we discuss and encourage companies to bring solutions to support two additional hop relays in R19 (and be forward compatible for future extensions for additional relays) from the start. Solutions should target minimizing the impact of hop-count on the developed mechanisms as much as possible.

# 2	Discussion

Figure 1: An example of a multi-hop L2 UE-to-Network (U2N) Relay with two additional hops relays on top of Rel-17/Rel-18 U2N Relay.

According to the WI objectives, RAN2 needs to specify mechanisms to support up to two additional hops relays in R19. RAN2 may focus on one additional hop relay mechanisms until RAN#107 and then extend these mechanisms to two additional hops relays after RAN#107. WID further specifies a necessary criterion for the mechanisms developed until RAN#107 to be easily extensible to support two additional hop relays in R19 and forward compatible for future extensions for additional relays. Therefore, we believe that RAN2 shall focus on developing mechanisms aiming to minimize the impact of hop count on the developed mechanisms from the start of the work in this WI. When developing mechanisms (such as Relay discovery, Relay (re)selection, and control plane procedures) for NR sidelink multiple-hop relay, RAN2 shall focus on supporting at least 2 additional hops relays in R19 from the start.

Therefore, we would like RAN2 to agree on following Working Assumptions:

RAN2 to focus on developing mechanisms aiming to minimize the impact of hop count on the developed multi-hop relay mechanisms.

RAN2 to develop mechanism from the onset of Rel19 work which are applicable to at least 2 additional hop relays.

# 3	Conclusion

Based on the discussion in the previous sections, we propose RAN2 to agree on the following Working Assumptions:

- RAN2 to focus on developing mechanisms aiming to minimize the impact of hop count on the developed multi-hop relay mechanisms.

RAN2 to develop mechanism from the onset of Rel19 work which are applicable to at least 2 additional hop relays.

# 4	References

- RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.

- RP-232896: Release-19 NR Sidelink Relay Enhancements for Public Safety, 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023.

- 3GPP TR 23.700-03 V1.0.0 (2024-06), 3GPP TSG SA;Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 3 (Release 19).