3GPP TSG-RAN WG2 Meeting #127		                               R2-2407391

Maastricht, Netherlands, 19-23 August, 2024

Agenda Item:		8.7.5

Source:			MediaTek Inc.

Title:				Discussion on RLC enhancements on small packet delay budget scenario

Document:			Discussion and decision.

### 1	Introduction

This document discusses the enhancement directions of fourth objective in R19 XR WID [1].

-	Specify the following user plane enhancements [RAN2]

-	RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

In last meeting (RAN2 #126), we reach following agreements:

- For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives:

  - Rx initiated approach

  - Tx initiated approach

- RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

- For Tx initiated approach:

  - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs

  - Tx side stops retransmit obsolete SDUs

  - Rx side updates state variables according to the information from Tx side

- For Rx initiated approach:

  - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter.

  - Tx side just processes the status report as in legacy

  - FFS how Rx side determines that an SDU should be abandoned

- To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options:

  - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered)

  - Retransmission based on enhanced status report

  - Retransmission based on enhanced polling

- FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism

- Impact on capacity should be considered

- RAN2 focuses on the enhancements for UL traffic

### 2	Discussion

## 2.1	Avoiding unnecessary RLC AM retransmissions

In last meeting (RAN2 #126), we discuss two mechanisms to avoiding unnecessary RLC AM retransmissions, one is Tx initiated approach, the other one is Rx initiated approach. We think the only feasible way is Tx initiated approach, because only transmitter side will know the remaining time of an SDU. There is still one FFS point in Rx initiated approach, which is how Rx side determines that an SDU should be abandoned. The only reason to abandon an SDU is due to out of date, Rx side is unable to know this information, only Tx side can get and aware this information easily.

Proposal 1: RAN2 to support Tx initiated approach to avoid unnecessary RLC AM retransmissions.

## 2.2	Timely retransmissions for XR traffic in RLC layer

In last meeting (RAN2 #126), we discuss three mechanisms to achieve timely retransmissions, among these three mechanisms, we can further categorize them into two types. One is retransmission without feedback, the other one is retransmission with feedback.

We use one example to show why we prefer the retransmission should based on feedback from receiver side. Assume each RLC SDU size is 1500 bytes and SN is 18 bits. Transmitter side already send SDU 1~7 to receiver side. Assume SDU 1, 3, 5, 7 are correctly received. The STATUS PDU is at most 31bytes. With the feedback, transmitter side only need to retransmit SDU 0, 2, 4, 6. (4*1500 = 6000 bytes). However, if there is no feedback, transmitter side needs to retransmit SDU 1~7 again, which cost (7*1500 = 10500bytes).

Capacity consumption of retransmission with feedback in this example is (6000 + 31) bytes which is less than retransmission without feedback (15000 bytes). It is clearly that retransmission based on feedback has less negative impact on capacity.

| SDU SN | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 (ACK_SN) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Correct received or not | N (at most ~7bytes) | Y | N (at most ~7bytes) | Y | N (at most ~7bytes) | Y | N (at most ~7bytes) | Y | N (~3bytes) |

Observation 1: Retransmission according to feedback has less negative impact on capacity.

Proposal 2: RAN2 to consider the following two options:

- Retransmission based on enhanced status report

- Retransmission based on enhanced polling

In the current RLC spec [2], only RLC SDU discard behavior is specified as shown in Excerpt 1 below. RLC PDU (RLC SDU or RLC SDU segment) discard behavior is not specified. As explained in Section 2.1, repeated attempts to retransmit lost RLC PDUs that have exceeded their PDB leads to head-of-line blocking and queuing delays at the transmitter. Therefore, we propose to define a mechanism to discard RLC PDUs that have exceeded their PDB.

5.4 SDU discard procedures

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

Excerpt 1: Discard procedure in RLC

Proposal 3: Introduce a mechanism to discard RLC PDUs that have exceeded their delay budget.

### 3	Conclusion

This document promulgated observation and following proposals:

Observation 1: Retransmission according to feedback has less negative impact on capacity.

Proposal 1: RAN2 to support Tx initiated approach to avoid unnecessary RLC AM retransmissions.

Proposal 2: RAN2 to consider the following two options:

- Retransmission based on enhanced status report

- Retransmission based on enhanced polling

Proposal 3: Introduce a mechanism to discard RLC PDUs that have exceeded their delay budget.

### 4	Reference

[1]	RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3.

[2]       38.322, NR Radio Link Control (RLC) protocol specification, v18.0.0