3GPP TSG-RAN WG2 Meeting #127		                               R2-2407392

Maastricht, Netherlands, 19-23 August, 2024

Agenda Item:		8.7.4

Source:			MediaTek Inc.

Title:				Discussion on UL scheduling enhancements

Document:			Discussion and decision.

### 1	Introduction

This document discusses the enhancement directions of third objective in R19 XR WID [1].

-	Specify Enhancements for Scheduling, as follows:

-	For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2].

NOTE:	LCP implementation complexity need to be taken into account when evaluating solutions.

NOTE:	Check in RAN#105

In last meeting, we reach following agreements:

- For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

  - Use additional priority configured to LCHs in case of these LCHs with delay-critical data

- FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

- Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG.

- FFS whether DSR triggering is impacted

- FFS whether PDU set importance needs to be included

### 2	Discussion

In Rel-18 XR WI, we introduce DSR MAC CE for UE to report the delay-critical buffer size information to gNB. The purpose is to only indicate delay-critical buffer status, the non-delay critical data can be handled by legacy BSR or refined BSR.

Figure 1: DSR MACE CE from MAC spec

Observation 1: In Rel-18 XR WI, RAN2 only consider information related to delay-critical data in DSR. Non-delay critical data can be handled by legacy BSR or refined BSR.

PSI is introduced in Rel-18 XR, its purpose is to further distinguish the different level of importance within PDU set, although, PSI has 16 level difference importance level. It is unclear how to use the PSI information other than discard mechanism when timer expires in PDCP layer. Before RAN2 agree to introduce importance information into DSR, companies need to show the benefit of DSR enhancement with PSI information.

Proposal 1: RAN2 need to show the benefit before introducing importance information into DSR.

Another enhanced direction for UL scheduling with delay/deadline information is that during logical channel selection [section 5.4.3.1.2], UE only select the logical channel with delay-critical data when DSR is triggered. An example TP is provided below:

-- MAC spec --

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and

2>	allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and

2>	allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant; and

2>	allowedHARQ-mode, if configured, includes the allowed UL HARQ mode for the HARQ process associated to the UL grant.

2>	allowedDelay-Critrical-mode, if configured, includes the logical channels trigger DSR to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration, Cell information, and priority index are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

--

Proposal 2: Introduce new condition (e.g., logical channels which trigger DSR) during logical channel selection.

Proposal 3: The new condition is activated/deactivated via RRC message in a semi-static way, once activate, the new condition applies to the whole LCH.

### 3	Conclusion

This document promulgated the following proposals:

Observation 1: In Rel-18 XR WI, RAN2 only consider information related to delay-critical data in DSR. Non-delay critical data can be handled by legacy BSR or refined BSR.

Proposal 1: RAN2 need to show the benefit before introducing importance information into DSR.

Proposal 2: Introduce new condition (e.g., logical channels which trigger DSR) during logical channel selection.

Proposal 3: The new condition is activated/deactivated via RRC message in a semi-static way, once activate, the new condition applies to the whole LCH.

### 4	Reference

[1]	RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3.