3GPP TSG-RAN WG2 Meeting #127	 R2-2407393

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.3 Measurement event evaluation

Source:	KDDI Corporation

Title:	Discussion on measurement event evaluation

WID:       	NR_Mob_Ph4-Core

Tdoc Type:	Discussion

Document for: 	Discussion and decision

- Introduction

In RAN#104 meeting, a revised Rel-19 WID of ‘NR mobility enhancements Phase 4’ was approved [1].

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In this contribution, we will discuss and share our views on event triggered L1 measurement reporting for “Specify necessary components to support event triggered L1 measurement reporting” described in the second bullet of the WID objective above.

- Discussion

What beam(s) of the serving cell and neighboring cell is used for event evaluation

Measurement events could be defined at the beam level support the generation of event triggered L1 measurement reports. This refers to the process where specific conditions related to the quality and characteristics of individual beams within the network can initiate the reporting process.

1. Optimization of Quality

- Best Beam Selection**: By selecting the most representative and highest quality beam from the candidate cell, the network can ensure that the service quality provided to the user is maximized. This results in better connectivity and more stable communication for end-users.

2. Efficient Resource Utilization

- Prioritizing the use of high-quality beams helps in optimizing the use of network resources. This minimizes unnecessary handovers and reduces unnecessary consumption of resources, which improves overall network efficiency.

3. Seamless Service Continuity

- The current serving beam, as indicated by the TCI (Transmission Configuration Indication) state, represents the beam to which the user is currently connected. Evaluating based on the current serving beam ensures that handovers or switching between beams cause minimal service interruption, leading to a smoother transition.

4. Improved Reliability and Stability

- Considering both the current serving beam and the best representative beam of the candidate cell ensures that the transition is made to a highly reliable and stable beam. This increases the likelihood of maintaining a stable connection post-transition, thereby enhancing the overall user experience.

5. Simplification and Clarity

- Focusing on the most representative beams simplifies the criteria for beam selection, making the event evaluation process clearer and more straightforward. This leads to easier network operations and maintenance, thus achieving efficient network management.

Strengthening the criteria for beam selection significantly improves network performance and service quality. By focusing the criteria on the "most representative quality beams," the event evaluation process becomes more efficient and effective.

Proposal 1: The selection of beams for event evaluation should focus on the most representative beams in terms of quality (i.e., best beam) for candidate cell and the current serving beam as indicated by TCI state and used for serving cell.

Need of cell level measurement result for event evaluation

In RAN2#126 meeting, proposals from reporters were not reached to agreement below [2].

Cell level measurement (P3: R2-2405063: ZTE) Proposal 3: For the input of the L1 event evaluation, RAN2 to consider both beam-level measurement result and cell-level measurement result (e.g. based on the consolidation of multiple beams of a cell).

For event evaluation, beam level measurements may be sufficient for event-triggered L1 measurement reports, but in some cases, such as conditional LTM, measurement events defined at cell level taking into account multiple beams may be required, and cell level measurements can be used to prevent ping-pong handovers.

1. Cell-Level Measurement

- Avoiding Ping-Pong Effect: The ping-pong effect occurs when a mobile device frequently switches back and forth between two or more cells. This can lead to instability in the connection and inefficient use of network resources. By implementing cell-level measurements, a more stable and accurate evaluation of the cell's performance can be achieved, thereby reducing the likelihood of unnecessary handovers and avoiding the ping-pong effect.

2. Need for Cell-Level Measurement

- Enhanced Stability: Cell-level measurements provide a broader and more consistent view of the network's performance at the cell level rather than at the beam level. This leads to more stable connections and better overall network performance.

- Efficient Resource Utilization: By accurately measuring and evaluating the cell performance, the network can make more informed decisions regarding handovers and resource allocations, optimizing the use of available network resources.

3. Further Discussions on Beam Consolidation

- Consolidation Strategy: Beams need to be consolidated in a way that the information gathered from multiple beams within a cell can be integrated into a single, cohesive assessment. This requires discussions on the methodology, algorithms, and criteria for consolidating beam measurements.

- Improved Decision-Making: Consolidating beam measurements will help in making better decisions related to handovers, thus reducing the probability of the ping-pong effect. This integration ensures that the overall cell performance is taken into account rather than just relying on individual beam measurements.

Proposal 2: Cell level measurement is needed to avoid ping-pong and should further discuss how beams will be consolidated.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: The selection of beams for event evaluation should focus on the most representative beams in terms of quality (i.e., best beam) for candidate cell and the current serving beam as indicated by TCI state and used for serving cell.

Proposal 2: Cell level measurement is needed to avoid ping-pong and can further discuss how beams will be consolidated.

References

[1] RP-241515 Revised Work Item: NR mobility enhancements Phase 4 (Apple Inc, China Telecom)

[2] R2-2403731 Report from session on LTE V2X and NR SL (Vice Chairman: Samsung)