3GPP TSG-RAN WG2 Meeting #127	 R2-2407394

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.4 Measurement reporting

Source:	KDDI Corporation

Title:	Discussion on event triggered L1 measurement reporting

WID:       	NR_Mob_Ph4-Core

Tdoc Type:	Discussion

Document for: 	Discussion and decision

- Introduction

In RAN#104 meeting, a revised Rel-19 WID of ‘NR mobility enhancements Phase 4’ was approved [1].

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In this contribution, we will discuss and share our views on event triggered L1 measurement reporting for “Specify necessary components to support event triggered L1 measurement reporting” described in the second bullet of the WID objective above.

- Discussion

MAC CE or UCI for measurement reporting

In RAN2#126, proposals from reporters were not discussed below [2].

MAC CE (P6: R2-2404463: Ericsson, T-Mobile) Proposal 6: Event-triggered L1-measurements are reported by the UE to the network via a MAC CE. UCI (P14: R2-2405492: LG) Proposal 14: RAN2 to consider transmission procedure adopted for UE-initiated/event-driven beam reporting in R19 MIMO as baseline for event-triggered L1 report for LTM. That is, Mode A (dynamically scheduling UCI by gNB) and Mode B (UCI in pre-configured resource(s) for second UL channel) are supported.

Regarding the medium/container for sending L1 LTM measurement reports, MAC-CE, UCI, or other options can be considered, and it has been agreed to use L1 UCI with dynamic resource allocation for the Rel-19 UE initiated/event-driven beam management framework for MIMO enhancements.

MAC CE is harmonized with L3 measurement design and can be flexible. On the other hand, UCI was agreed to use L1 UCI in the Rel-19 UE-initiated/event-driven beam management framework for MIMO extensions, with dynamic resource allocation and report format in Rel-18 LTM can be reused.

1. Transmission Method Decision

- Dependent on RAN1 Progress: The decision on whether to transmit the information via L1 UCI or L2 MAC CE depends on the progress of discussions in RAN1.

2. Resource Requirements

- Varying Needs: The resources required for transmission will vary based on the content and format of the beam report. For example, more detailed or complex beam reports will demand more resources for transmission.

3. L1 UCI

- Limitations: L1 UCI is used for transmitting control information on the physical layer. It is typically suitable for sending simple control information with limited resource usage. However, if the required resources exceed the capacity of L1 UCI, then it will not be adequate.

4. L2 MAC CE

- Enhanced Capability: L2 MAC CE is used for transmitting control information on the data link layer. This method can handle larger data sizes and more complex control information compared to L1 UCI. Therefore, if the beam report requires substantial resources or contains detailed information, MAC CE will be necessary for transmission.

To decide the most suitable transmission method, it is crucial to monitor the progress of discussions in RAN1 and evaluate the required resources based on the beam report's content and format. If it is determined that more resources are needed, L1 UCI will not suffice, and transmission via MAC CE will be required.

In summary, the choice between L1 UCI and L2 MAC CE for transmitting beam reports depends on the specific resource requirements, which in turn depend on the report’s content and format. This decision should be made based on ongoing technical discussions in RAN1. If additional resources are necessary, MAC CE will be the preferred method of transmission.

Proposal 1: Whether it is carried via L1 UCI or L2 MAC CE can be wait for discussions RAN1 progress since required resources will vary depending on the beam report content and format. If more resources are necessary, it would not fit in L1 UCI, and transmission via MAC CE will be required.

Need of measurement reporting once leaving condition is met, etc.

In RAN2#126, several FFS were agreed of [2].

Agreements at RAN2#126 - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

If cell level measurements are considered, a cell specific offset should be applied.

1. Account for Different Cell Conditions

- Varying Conditions: Cells within a network can experience different levels of interference, load, and environmental factors. By applying a cell-specific offset, these unique conditions can be accounted for, leading to more accurate measurements and evaluations.

2. Enhanced Handover Decisions

- Better Decision-Making: Cell-specific offsets help in making more informed handover decisions by adjusting the measurement results to reflect the actual performance and quality of different cells. This ensures that handovers are triggered only when they are truly beneficial to the user.

3. Reduction of Unnecessary Handover Events

- Minimizing Ping-Pong Effects: Often, cells are close in performance metrics, leading to repeated handovers (ping-pong effect). Applying cell-specific offsets can help differentiate these metrics more clearly, reducing the likelihood of unnecessary handover events.

4. Fair Comparison Across Cells

- Standardization: Offsets allow for a standardized comparison of measurements across different cells. This compensates for inherent differences in cell performance, ensuring a fair evaluation process.

5. Network Efficiency

- Optimized Resource Allocation: By applying cell-specific offsets, the network can more efficiently allocate resources, as it accurately understands the performance of each cell in relation to its specific conditions. This leads to optimal utilization of network capacity and better user experience.

Applying cell-specific offsets to cell level measurements introduces a more nuanced and accurate evaluation process. This adjustment accounts for the unique conditions of each cell, leading to improved decision-making, reduced unnecessary handovers, and overall better network performance.

Proposal 2: If cell level measurement is considered, cell specific offset should be applied.

By allowing the network to know when the event conditions are not met, ping-pong handover can be prevented.

1. Preventing Ping-Pong Effect

- Ping-Pong Effect: The ping-pong effect occurs when a mobile device frequently switches back and forth between two or more cells, leading to unnecessary handovers. This can cause connection instability and degrade the user experience. By reporting measurements once the leaving condition is met, the network can make more informed decisions to prevent these frequent switches.

2. Ensuring Stability in Connections

- Stable Handover Decisions: Measurement reporting upon meeting the leaving condition ensures that handover decisions are based on the most current and relevant data. This increases the likelihood of a stable connection after the handover, reducing the chances of the device quickly switching back to the previous cell.

3. Optimized Resource Utilization

- Efficient Resource Use: Frequent handovers consume network resources and can lead to increased signaling overhead. By reporting measurements when the leaving condition is met, the network can avoid unnecessary handovers, thus optimizing resource utilization and improving overall network efficiency.

4. Improvement of User Experience

- Seamless Connectivity: By avoiding frequent and unnecessary handovers, users experience fewer disruptions and enjoy more seamless connectivity. This leads to higher satisfaction and better user experiences, especially in mobile environments.

Enabling measurement reporting once the leaving condition is met is crucial for avoiding the ping-pong effect and ensuring stable, efficient handovers. This approach optimizes resource utilization, enhances network efficiency, and significantly improves the user experience by maintaining seamless connectivity. By making handover decisions based on the most current and relevant data, the network can effectively manage connections and provide high-quality service to users.

Proposal 3: Measurement reporting is needed once leaving condition is met to avoid ping-pong.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: Whether it is carried via L1 UCI or L2 MAC CE can be wait for discussions R1 progress since required resources will vary depending on the beam report content and format. If more resources are necessary, it would not fit in L1 UCI, and transmission via MAC CE will be required.

Proposal 2: If cell level measurement is considered, cell specific offset should be applied.

Proposal 3: Measurement reporting is needed once leaving condition is met to avoid ping-pong.

References

[1] RP-241515 Revised Work Item: NR mobility enhancements Phase 4 (Apple Inc, China Telecom)

[2] R2-2405701 Report from session on V2X/SL, R19 NES and MOB	Vice Chairman (Samsung)