3GPP TSG-RAN2 Meeting #127	R2-2407395

Maastricht, The Netherlands, 19 - 23 August 2024

Agenda Item:	7.11.2 Corrections

Source: 	Ericsson

Title:  	Validity of PTM configuration in RRCRelease

Document for:	Discussion and Decision

# Introduction

In this contribution the validity of inactivePTM-Config IE in RRCRelease is discussed further.

# Discussion

When the UE is suspended with a PTM configuration of an active session and the UE selects the serving cell (i.e. cell where it received the session in connected), then the UE can use this PTM configuration to receive the session in RRC_INACTIVE:

The PTM configuration is not valid when the UE selects a neighbour cell.

However what is the intended behavior, i.e. what is the validity of the PTM configuration when:

- The UE selects the serving cell but the session is inactive and the UE receives group paging much later.

- The UE selected a neighbour cell when it was released, but the UE reselects to the serving cell much later while the session is active.

In our view, in both cases, the UE should use the received PTM configuration in RRCRelease, unless MCCH is configured in the cell, in which case the UE should use the PTM configuration from MCCH. In case the MCCH is not configured, then the PTM configuration received in RRCRelease is not supposed to change, and the UE can use for case 1 and 2, i.e. the UE does not resume.

What does the specification say?

Inactive session (case 1)

When the UE receives group Paging in RRC_INACTIVE the UE either resumes or stays:

5.3.2.3	Reception of the Paging message by the UE or PagingRecord by the L2 U2N Remote UE

Upon receiving the Paging message by the UE or receiving PagingRecord from its connected L2 U2N Relay UE by a L2 U2N Remote UE, the UE shall:

…

1>	if in RRC_INACTIVE and the UE has joined one or more MBS session(s) indicated by the TMGI(s) included in the pagingGroupList:

2>	if PagingRecordList is not included in the Paging message; or

2>	if none of the ue-Identity included in any of the PagingRecord matches the UE identity allocated by upper layers or the UE's stored fullI-RNTI:

3>	if the UE is not configured to receive multicast in RRC_INACTIVE for at least one of the MBS sessions indicated by the TMGI(s) that the UE has joined; or

3>	if inactiveReceptionAllowed is not included for at least one of the MBS sessions indicated by the TMGI(s) that the UE has joined:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set as below:

…

3>	else:

4>	start monitoring the G-RNTI(s), if configured, corresponding to the TMGI(s);

4> if the UE was notified to stop monitoring the G-RNTI(s) for all the joined multicast sessions that are configured for reception in RRC_INACTIVE:

5>	if multicast MCCH is present:

6>	start monitoring the Multicast MCCH-RNTI;

6>	acquire the MBSMulticastConfiguration message on multicast MCCH;

4>	else if the UE was notified to stop monitoring the G-RNTI for at least one multicast session for which the PTM configuration was not included in RRCRelease message:

5>	acquire the MBSMulticastConfiguration message on multicast MCCH;

…

Intermezzo about “if the UE is not configured to receive multicast in RRC_INACTIVE”. The UE is configured to receive multicast in RRC_INACTIVE when multicastConfigInactive IE is included in the RRCRelease message:

RRCRelease ::=                      SEQUENCE {

…

SuspendConfig ::=                   SEQUENCE {

…

multicastConfigInactive-r18 SetupRelease { MulticastConfigInactive-r18 }         OPTIONAL

…

MulticastConfigInactive-r18::=         SEQUENCE {

inactivePTM-Config-r18   OCTET STRING (CONTAINING MBSMulticastConfiguration-r18) OPTIONAL,

inactiveMCCH-Config-r18  OCTET STRING (CONTAINING SystemInformation)             OPTIONAL

}

SIB24-r18 ::= SEQUENCE {

multicastMCCH-Config-r18       MCCH-Config-r17         OPTIONAL,  -- Need S

cfr-ConfigMCCH-MTCH-r18        CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

lateNonCriticalExtension       OCTET STRING            OPTIONAL,

...

}

multicastConfigInactive Indicates whether the UE is configured to receive MBS multicast in RRC_INACTIVE. The presence of this field indicates the UE is configured to receive MBS multicast in RRC_INACTIVE; otherwise, the UE is not configured to receive MBS multicast in RRC_INACTIVE.

inactivePTM-Config Indicates the multicast session(s) that can be received in RRC_INACTIVE and optionally the corresponding PTM configuration (which includes mrb-ListMulticast, pdsch-ConfigIndex, mtch-SSB-MappingWindowIndex, etc.) for the cell where the multicast session(s) was received in RRC_CONNECTED. If absent, UE considers that all joined multicast sessions can be received in RRC_INACTIVE.

inactiveMCCH-Config Indicates multicast MCCH/MTCH configuration for MBS multicast reception in RRC_INACTIVE in the serving cell. Only SIB24 is allowed to be included.

In our understanding the inactivePTM-Config IE should always be present in the multicastConfigInactive IE, i.e. when omitted the UE is not configured to receive multicast in RRC_INACTIVE:

Clarify that inactivePTM-Config IE is always included in multicastConfigInactive IE.

In case this is not agreeable, i.e. when the UE is also configured when only the MCCH configuration is present in the release, then the field description of multicastConfigInactive IE needs to be updated and further corrections may be needed.

In case the UE is released with an active session and selects the serving cell where it received the session in connected and the multicastConfigInactive IE includes the inactivePTM-Config IE then the NW may choose to include only cfr-ConfigMCCH-MTCH IE in SIB24. In such case the UE is not required to obtain SIB24 before applying the configuration received in the release, i.e. it may shorten the interruption during the release:

RAN2 confirms that inactiveMCCH-Config IE may only include cfr-ConfigMCCH-MTCH IE.

End of Intermezzo.

The UE resumes when it does not have a PTM configuration or the UE is not allowed to receive the session in RRC_INACTIVE when it receives the group paging.

Else the UE starts monitoring the G-RNTI(s) included in the PTM configuration for the session(s) that are activated, i.e. the UE starts using the (old) PTM configuration, without using the PTM configuration on the MCCH, if configured.

If the UE was not already monitoring the MCCH for an active session, then the UE will start monitoring the MCCH. The UE monitors the MCCH for an activate session to be notified about PTM configuration change or session deactivation. Thus if the PTM configuration on the MCCH is different from the PTM configuration in RRCRelease, the UE will apply the new configuration from MCCH.

If the MCCH is not configured then the UE will use the (old) PTM configuration received in RRCRelease. When the MCCH is not configured, then the PTM configuration is not supposed to change, i.e. the UE can use the old PTM configuration received in RRCRelease.

Mobility case (case 2)

After cell reselection to a new cell with at least one active MBS session:

5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

…

4>	if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers:

5>	if the serving cell does not belong to the configured ran-NotificationAreaInfo:

6>	initiate an RNA update as specified in 5.3.13.8;

5>	if configured to receive MBS multicast in RRC_INACTIVE and not indicated to stop monitoring G-RNTI for at least one MBS multicast session:

6>	if SIB24 is not scheduled in SIB1 in the new cell after cell selection or cell reselection:

7>	initiate an RRC connection resume procedure for multicast reception as specified in 5.3.13.1d;

…

If the MCCH is not present after cell reselection for an active session then the UE resumes:

In case of cell reselection the UE does not use the PTM configuration received in RRCRelease when MCCH is absent. But when the session is activated via group paging and the UE is in the serving cell where it received the session in connected then the UE uses the PTM configuration received in RRCRelease when MCCH is absent.

It seems that the PTM configuration received in RRCRelease is not used after cell reselection when MCCH is absent. However it is used in case the session is activated via group paging and the MCCH is absent:

RAN2 to discuss if it needs to clarified when the UE can use the PTM configuration received in RRCRelease.

A TP for proposal 1 is provided for discussion and agreement in the Annex.

# Summary

RAN2 is kindly asked to discuss the validity of the PTM configuration received in RRCRelease:

Observation 1	In case of cell reselection the UE does not use the PTM configuration received in RRCRelease when MCCH is absent. But when the session is activated via group paging and the UE is in the serving cell where it received the session in connected then the UE uses the PTM configuration received in RRCRelease when MCCH is absent.

Proposal 1	Clarify that inactivePTM-Config IE is always included in multicastConfigInactive IE.

Proposal 2	RAN2 confirms that inactiveMCCH-Config IE may only include cfr-ConfigMCCH-MTCH IE.

Proposal 3	RAN2 to discuss if it needs to clarified when the UE can use the PTM configuration received in RRCRelease.

# Annex: TP 38.331 section 6.2.2

MulticastConfigInactive field descriptions

inactivePTM-Config Indicates the multicast session(s) that can be received in RRC_INACTIVE and optionally the corresponding PTM configuration (which includes mrb-ListMulticast, pdsch-ConfigIndex, mtch-SSB-MappingWindowIndex, etc.) for the cell where the multicast session(s) was received in RRC_CONNECTED. If absent, UE considers that all joined multicast sessions can be received in RRC_INACTIVE. This field is present when is MulticastConfigInactive is configured.

inactiveMCCH-Config Indicates multicast MCCH/MTCH configuration for MBS multicast reception in RRC_INACTIVE in the serving cell. Only SIB24 is allowed to be included.