3GPP TSG-RAN2 Meeting #127	R2-2407396

Maastricht, The Netherlands, 19 - 23 August 2024	Revision of R2-2404996

Agenda Item:	8.4.2 Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Source: 	Ericsson

Title:  	LP-WUS in Idle and Inactive

Document for:	Discussion and Decision

# Introduction

In this contribution the LP-WUS in Idle and Inactive are discussed.

For the WID objectives and RAN1/RAN2 agreements the reader is referred to the Annex.

# Discussion

## Entry/exit condition

It is FFS whether the entry/exit condition for LP-WUS monitoring is always configured:

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

  - LP-SS configuration

  - LP-WUS configuration

  - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured)

The use of LP-WUS should not decrease the paging performance, e.g. missed paging probability. In case LP-WUS does not provide full cell coverage, i.e. LP-WUS does not provide similar performance as reception of Paging PDCCH (missed detection target of 10-2), then an entry condition must be configured.

In our view OFDM-based WUR can provide full LP-WUS cell coverage and provide similar performance as Paging PDCCH. Rel-16 PEI (PDCCH-based) provides full cell coverage, and there is no entry/exit condition for PEI. When pei-Config is present in SIB1 the UE is allowed to use PEI. The entry condition may increase the time paging reception is interrupted during cell reselection, and increase the missed paging probability:

For OFDM-based WUR the entry/exit condition is optional.

For OFDM-based WUR it should of course be allowed to configure an entry/exit condition, e.g. in case the OFDM-based LP-WUS performance is not expected to be similar to Paging PDCCH (e.g. due to configured repetitions/bandwidth/power).

The OOK-based LP-WUS performance is not expected to meet the Paging PDCCH requirements, i.e. configuration of an entry/exit condition for OOK-based WUR is mandatory:

For OOK-based WUR the entry/exit condition is mandatory.

It should be possible to configure a different entry/exit configuration for OFDM and OOK-based WUR. For example different threshold values can be configured:

Separate entry/exit conditions can be configured for OFDM-based and OOK-based WUR.

It is FFS whether the entry condition includes LR measurements:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

MR measurements are of course a good indication whether LP-WUS monitoring can be allowed. But these are different physical signals with different performance and characteristics. Therefore we think that an extra check whether the LP-SS measurements actually provide good results should be supported. This can prevent scenarios where the LP-SS / LP-WUS performance turns out to be insufficient and the UE needs to activate the MR again. This ping pong between MR and LR may increase the missed paging probability. Furthermore when the MR threshold is set very high, then it might unnecessarily limit the use of LP-WUS:

For OOK-based WUR the LR entry condition may include an LP-SS threshold.

We do not see a (strong) need for LR measurements on SSS/PSS, i.e. we expect a similar results from both measurements.

The UE should not perform the MR and LR measurements in sequence, i.e. this would further delay the LR entry. But it is beneficial when the MR and LR measurements are performed in parallel:

If an LP-SS threshold is configured, a threshold below the MR serving cell threshold of the entry condition can be configured where the UE performs LP-SS measurements.

In Idle/Inactive the UE is required to perform Idle mode tasks as defined in 38.304, e.g. paging monitoring, RRM measurements, cell reselection, SI acquisition, provide information to upper layers for PLMN selection, etc. Furthermore the UE may receive MBS broadcast/multicast data or SDT unicast data. When the UE is using WUR/LR then the MR can potentially be deactivated for a longer time because the LR can perform certain MR tasks like serving cell measurements and paging monitoring:

When the LR detects LP-WUS and there is a subgroup match, then the MR shall perform legacy PO monitoring. The UE then needs to receive the Paging PDCCH including any short message indications like SI-change or ETWS/CMAS indication. If the Paging PDCCH indicates a following Paging PDSCH, then the UE needs to receive the Paging message. If it was a false alarm the legacy PO monitoring task is terminated, otherwise the UE needs to act on the Paging message content.

When there is a subgroup match detected by the LR, then there is no need to deactivate the LR, even though the UE implementation may decide to deactivate the LR. It is possible that there is a false paging alarm, Paging PDCCH only (SI-change or ETWS/CMAS indication) or MBS group paging indication for MBS multicast reception in RRC_INACTIVE, in which case the UE remains in Idle/Inactive and continues to monitor paging:

When LP-WUS is detected and there is a subgroup match then the MR shall perform legacy paging monitoring.

With WUR it is possible to implement and enable a new sleep state, i.e. ultra-deep sleep. This new state can be much more power efficient than deep sleep. But it costs a more energy and time to enter/exit ultra-deep sleep state, i.e. entering ultra-deep is only beneficial when the MR remains deactivated for at least 10 – 20 seconds. However there is no inter-operability issue when the UE enters ultra-deep sleep but needs to wake up after 2 seconds again, but the UE power consumption is increased. There are also power saving gains with WUR when the MR enters deep sleep, because the LR can be used for serving cell measurements and paging monitoring instead of MR. The MR would need to be activated in fewer cases to monitor legacy PO:

If the UE is not required to perform any idle mode tasks, then it is left to UE implementation which sleep state to select and when to enter deep sleep, if at all.

## Configuration

It is FFS whether there is a need for dedicated RRC configuration:

The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed.

We think that the LP-WUS configuration is common, i.e. cell specific, and there is no need for RRC signalling to enable UE specific LP-WUS configurations. Also note that the SIB can include LP-WUS configurations for different types of UEs, e.g. OFDM and OOK-based WUR:

Dedicated RRC signalling is not needed for LP-WUS configuration.

The following type of UEs can be identified:

- OFDM-based WUR:

  - LR uses PSS/SSS for sync and serving/neighbour RRM measurements

  - LR uses LP-WUS for paging monitoring

- OOK-based WUR:

  - LR uses LP-SS for sync and serving cell RRM measurements

  - LR uses LP-WUS for paging monitoring

- UE supports both OFDM-based and OOK-based WUR

There are differences between OFDM-based and OOK-based WUR in terms of coverage, accuracy, latency and need for LP-SS. RAN1 is discussing whether there is a benefit if an OFDM-based WUR UE in addition supports LP-SS for sync and serving cell RRM measurements.

The LP-SS is a periodic signal that increases the required system resources, and LP-SS provides lower accuracy and longer delay for sync and serving cell measurements compared to PSS/SSS. Therefore it should be possible for the gNB to decide whether only OFDM-based or only OOK-based WUR or both types of WUR are used in the cell:

gNB can configure in SIB whether only OFDM-based or only OOK-based WUR or both types of WUR is supported.

The RAN2 working assumption is that the LP-WUS configuration at least includes:

Working assumption: the LP-WUS configuration in SIB at least includes the following information:

  - LP-SS configuration

  - LP-WUS configuration

  - FFS on Entry/exit condition for LP-WUS monitoring

However OFDM-based WUR uses PSS/SSS for the LR serving cell measurement, i.e. the LP-SS configuration is not needed in that case:

Dependent on RAN1 progress the LP-SS configuration is optional or absent for OFDM-based WUR.

The LP-WUS configuration parameters need to be configured in SIB, e.g. time offset between LP-WUS occasion (LO) and Paging Occasion (PO), the bits/codepoints used for CN-assigned and UE-ID based subgroups, the mapping of subgroups onto bits or codepoints, LO window duration including LP-WUS MOs, mapping of LO to one or more POs, etc. For most of the LP-WUS configuration details RAN2 should wait for RAN1 progress, but some high level agreements can be reached in RAN2.

System resources are wasted when the NW has to transmit LP-WUS two times, i.e. for UEs in ultra-deep sleep and deep sleep, while most of the UEs can be assumed to be in ultra-deep sleep. Furthermore the UE that decided to enter deep sleep, can enter deep sleep again between LO and PO when LP-WUS is detected.

The LR requires a long wake-up time (between 400-800 ms) compared to waking up from deep sleep (20 ms). During the study phase, as a baseline a transition time of 400 ms was assumed in FR1, with an alternative value of 800 ms:

For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows,

-	Alt 1: (15000, 400ms) as baseline

-	Alt 2: (40000, 800ms)

Company to report which alternative they use for which use cases.

The minimum value of the time offset can be discussed and agreed in RAN1. The minimum value has to accommodate different UE implementations, which implies that some UE implementations may need to wake-up a little earlier then required. But this has a minor impact on the power consumption because the LR power consumption is very low:

A single time offset between LP-WUS occasion (LO) and Paging Occasion (PO) can be configured in SIB assuming that the UE needs to wake-up from ultra-deep sleep.

The time offset should allow the UEs supporting LP-WUS to wake-up from ultra-deep sleep before the PO starts, in case LP-WUS is detected and there is a subgroup match. In case the UE supports LP-WUS it is fair to assume that the UE typically enters ultra-deep sleep, because this provides the most power saving gains. An LP-WUS UE could also enter deep sleep, similar as a UE supporting PEI, but in such case the additional implementation complexity is not well justified:

|  | Relative Power (unit) | Transition time (ms) | Transition energy (ms) |
| --- | --- | --- | --- |
| UL | 250 (0 dBm) - 700 (23 dBm) |  |  |
| PDCCH + PDSCH | 300 |  |  |
| SSB | 100 |  |  |
| PDCCH-only | 100 |  |  |
| Micro sleep | 45 | 0 | 0 |
| Light Sleep | 20 | 6 | 100 |
| Deep Sleep | 1 | 20 | 450 |
| Ultra deep sleep (MR) | 0.015 | 400 (800) | 15000 (40000) |
| On (LR) | 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 | FFS | TLR, ramp-up *(PON-POFF)/2 |
| Off (LR) | 0.001/ 0.02/ 0.1 |  |  |

The power consumption of MR active (PDCCH-only e.g. using PEI) compared to LR active with OFDM-based WUR and relative power unit ~10 (factor 10x) or OOK-based WUR and relative power unit ~1 (factor 100x).

The same configuration flexibility as with PEI for the time offset can be re-used for LP-WUS:

The time offset between LP and PO consists of a frame offset (lpwus-FrameOffset) between the start of the Paging Frame including the PO and frame including the LO, and the number of symbols (firstPDCCH-MonitoringOccasionOfLP-WUS-LO) between the start of the frame including the LO and the start of the first MO included in the MO.

## Subgrouping

RAN2 assumes that there are at least 8 subgroups to reduce the false paging alarms:

RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

This is in line with RAN1 agreement where the number of subgroups is between 8 and 256:

Agreement

For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256.

- FFS the exact value of X.

We think that typically 8 bits are needed to reduce the false paging alarms. It is not unlikely that more than one UE (in different subgroups) in a PO needs to be paged. Afterall up to 32 paging records can be included in the Paging message. In case only 8 codepoints are available, then every time when multiple subgroups need to be paged, the feature is useless because all UEs need to be woken up (assuming that one codepoint is used to wake up all UEs), and the NW has wasted an LP-WUS transmission.

Perhaps in some scenarios where the paging rate is very low, and the probability that multiple subgroups need to be paged is also very low, and 16 or more codepoints are available, then the use of codepoints could be applied. The use of codepoint can reduce the LP-WUS signal duration, i.e. improve power saving. However in some cases either the paging message is delayed with one DRX cycle or all UEs need to be woken up.

Given that there are different views on the use of bits or codepoints for subgrouping we think that the following proposal meets both views:

A subgroup can be mapped onto a bit or a codepoint dependent on the configuration in SIB.

Indicate to RAN1 that it should be possible to configure 8 bits or less for subgrouping.

RAN1 is considering the options to support a 1:1 and 1:N mapping of LO to PO, i.e. option 1 and 2 below:

Agreement

It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options:

- Option 1: UEs monitoring the same PO monitor the same LO

- Option 2: UEs corresponding to different POs monitor the same LO

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

- Combinations of the above options can be considered.

In case a single LO points to multiple POs (1:N mapping), then there are less LP-WUS transmissions to wake-up the UEs for paging, which is good from a system perspective. However the false alarm rate would increase because more UEs will wake-up from a single LP-WUS transmission. But the false alarm rate does not increase when the number of UEs per PO is reduced i.e. the number of UEs per subgroup remains the same. For example a 1:1 mapping with a quarter of the frames of the DRX cycle configured as paging frames, can be expected to provide a similar false paging alarm rate as a 1:2 mapping with half of the frames configured as paging frames:

A single LO can be mapped onto a single PO or multiple POs (further details are left to RAN1 to decide).

With PEI a single PEI can be mapped up to 8 POs with the POs across 2 paging frames at most.

## CN-assigned and UE-ID based subgrouping

RAN2 agreed to use the PEI subgrouping as a baseline:

The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

With PEIPS (Paging Early Indication with Paging Subgrouping) signalling, the CN can assign a CN-assigned subgroup to the UE during registration:

It is proposed to adopt the PEIPS signalling framework to assign a CN-assigned subgroup to the UE for LP-WUS monitoring:

The UE can indicate to support CN-assigned subgrouping with LP-WUS in the REGISTRATION REQUEST message. The CN can assign a CN-assigned subgroup for LP-WUS monitoring to the UE in the REGISTRATION ACCEPT message.

The UE indicates support of UE-ID based subgrouping (per band) in the paging capabilities included in the UERadioPagingInfo IE in UECapabilityInformation message. The CN conveys the UE capabilities to RAN to enable RAN to use UE-ID based PEI with RAN paging. When there is CN paging for UEs in CM_IDLE, then the UE-ID based PEI support is included in the UE Radio Capability for Paging IE in the PAGING message from CN. When the gNB supports PEI and pages the UE, either triggered by RAN paging or CM_IDLE paging, the gNB takes the UEs UE-ID based PEI support and whether the UE has a CN-assigned subgroup into account. If the UE supports PEI then the gNB has to transmit PEI prior to the PO. If the UE supports PEI it is up to UE implementation to monitor PEI or monitor PO directly:

The UE-ID based subgroup is not signalled, but the UE determines the UE-ID based subgroup based on its UE identity. Instead there is signalling between UE, gNB and CN whether the UE supports UE-ID based subgrouping:

The UE can indicate to support UE-ID based subgrouping with LP-WUS in the UERadioPagingInfo IE in UECapabilityInformation message.

The UE-ID based and CN-assigned subgrouping capabilities were discussed in different working groups. And for PEI the support of UE-ID based subgroup can be indicated per band, while this is not possible for CN-assigned subgrouping. In our understanding it is not well motivated to have different granularity. To save signalling and reduce complexity:

The UE-ID based subgrouping with LP-WUS is not signalled per band.

## LP-WUS and mobility

From a network perspective two cases w.r.t. the UE mobility can be identified:

- The UE is in the same cell as it was the time when it last accessed the network.

- The UE is not in the same cell as it was the time when it last accessed the network.

In case 2 the LP-WUS overhead is significantly higher than in case 1. For example: in the first paging attempt the network pages the UE in the last used cell (and some surrounding cells) to limit paging resources. Then if the UE does not respond to the first paging attempt the network escalates the paging to the complete TA/RNA to find the UE as soon as possible. In the second paging attempt there will be a high number of LP-WUS transmissions when the TA/RNA consists of a large number of cells.  The UE will only be found in a single cell, i.e. there are a lot of “useless” LP-WUS transmissions. Similar as with PEI (lastUsedCellOnly flag in SIB1) the use of LP-WUS with or without mobility should be configurable in SIB:

It is indicated in SIB whether LP-WUS is only used in the last used cell or in any cell.

In case LP-WUS is used in any cell, then this is beneficial for the UE because LP-WUS can always be used. But in that case the NW will transmit LP-WUS more often and the UE will receive LP-WUS also more often, i.e. more frequent wake-up calls. In such case it is beneficial to have one bit in LP-WUS (“mobility” bit) indicating that the network is paging a UE that has moved, i.e. the network is looking for a UE that did not reply when paged in its last used cell. Then UEs that have not moved, i.e. UEs camped on the last used cell, can ignore the wake up call for the following PO:

LP-WUS can indicate when there is paging because the UE is not found in the last used cell. UE monitoring LP-WUS in the last used cell is not required to monitor the following PO.

## LP-WUS monitoring rules

The same monitoring principles as for PEI should apply to LP-WUS:

If the UE detects LP-WUS and the subgroup ID of the UE is included in LP-WUS then the UE monitors the following PO.

If the UE does not detect LP-WUS or subgroup ID of the UE is not included in LP-WUS then the UE is not required to monitor the following PO.

If the UE is not able to monitor the LP-WUS (details FFS) then the UE is required to monitor the following PO.

For example when during cell re-selection the UE is unable to monitor the LP-WUS occasion, then the UE should monitor the associated PO.

Concerning LP-WUS Occasion (LO) including multiple Monitoring Occasions (MO) RAN1 agreed:

Agreement

LP-WUS occasions (LOs) are defined for LP-WUS monitoring.

- Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs.

  - Different LP-WUS MOs may correspond to different beams in multi-beam operation

  - FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs

  - FFS details

- It is at least supported that a UE monitors LOs with a configured periodicity.

- FFS eDRX, if supported

In case of multibeam operation it seems straightforward that each beam includes at least one MO or the LO, and that the information in the MO is the same:

But for reliability and scheduling flexibility it is also beneficial to support multiple MOs per LO, e.g. an LO window including multiple MOs. In such case the UE is required to monitor all MOs of the LO:

In case the LO consists of a window including multiple MOs then the UE shall monitor all MOs in the window (of the selected beam in case of multibeam operation).

When the UE uses both LP-WUS and PEI (see section 2.8) then there are additional monitoring requirements. For example when LP-WUS indicates that the UE shall monitor the PO, but PEI indicates that this is not needed, then the UE is not required to monitor the PO:

When the UE uses both LP-WUS and PEI and LP-WUS indicates that the UE has to monitor paging, then this wake-up call is cancelled when UE receives PEI indicating that the UE does not have to monitor paging. When the UE does not detect PEI the UE monitors the following PO.

Pending progress in RAN1 RAN2 assumes that the LP-WUS missed and false detection probability does not impact the Paging performance. The same applies for when LP-WUS and PEI are used together.

## Enabling/disabling

There can be a need to disable LP-WUS in a specific UE, for example when the NW observes that:

- the missed paging rate is increased for the UE.

- the missed paging rate is increased for the UE using PEI and LP-WUS.

CN-assigned based subgrouping using LP-WUS can be disabled by the CN, i.e. the CN can decide not to configure a CN subgroup in the registration accept. However the UE-ID based subgrouping is configured in the RAN and is transparent to the CN. But the NAS and CN signalling can easily be extended to enable the CN to enable/disable UE-ID based LP-WUS for a specific UE:

- The UE can indicate in the registration request that it supports UE-ID based LP-WUS

- The CN can indicate in the registration accept whether the UE is allowed to use UE-ID based LP-WUS

- In case UE-ID based LP-WUS is disabled in the UE, the CN informs RAN:

  - Using the INITIAL CONTEXT SETUP REQUEST message for RAN paging

  - Using the PAGING message for CN paging

Given that the existing signalling already enables the CN to disable CN-assigned LP-WUS for a specific UE, we think it is easy to extend the existing signalling with UE-ID based LP-WUS. It is more complex using RRC signalling to disable UE-ID based subgrouping in RAN for a specific UE:

The CN has the ability to disable/enable LP-WUS in the UE during initial registration, but also has the opportunity to change this in the UE during registration triggered by mobility or periodic location update:

The signalling for CN assigned subgrouping is extended to enable the CN to enable/disable UE-ID based LP-WUS in the UE.

## LP-WUS and PEI

RAN1 agreed that LP-WUS and PEI can be used together:

Agreement

For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Monitoring PEI after LP-WUS has been detected can potentially further reduce the false paging alarm for UEs monitoring both LP-WUS and PEI. However using both LP-WUS and PEI also increases the false paging alarm for UEs that are only monitoring PEI, because the subgroup ID sent via PEI will also wake-up UEs that only support PEI. Furthermore the population of UEs supporting both LP-WUS and PEI may be small, which means that the benefit is not large. However the network is required to send “additional” PEI transmissions for every LP-WUS transmissions, when both LP-WUS and PEI are configured:

When both LP-WUS and PEI are configured, it is indicated in SIB whether the UE supporting both LP-WUS and PEI is allowed to use LP-WUS and PEI simultaneously.

In case the UE is not allowed to use LP-WUS and PEI together, the network indicates in SIB which one the UE shall use to avoid useless transmissions:

When both LP-WUS and PEI are configured, and it is indicated that the UE is not allowed to use LP-WUS and PEI together, then it is indicated in SIB which one the UE shall use.

To ensure that simultaneous monitoring of LP-WUS and PEI is beneficial it needs to be ensured that UEs monitoring LP-WUS are randomly distributed over the PEI subgroups when monitoring PEI after LP-WUS. This can be achieved by re-using the existing formula but taking into account the number of PEI subgroups (#PEIsubgroups) when calculating the LP-WUS subgroup ID:

PEISubgroupID = (floor(UE_ID/(N*Ns)) mod #PEIsubgroups)

LP-WUSSubgroupID = (floor(UE_ID/(N*Ns* #PEIsubgroups)) mod #LP-WUSsubgroups)

For example with 4 PEI subgroups and 3 LP-WUS subgroups and UE_ID: 0, 1, 2, 3, 4, 5, 6, 7, 8, …

UE-ID          LP-WUS Gr 1	LP-WUS Gr 2   	LP-WUS Gr 3   	LP-WUS Gr 1

PEI Group 1: 	0,1,2,		12,13,14,	24,25,26,	36,37,38, …

PEI Group 2: 	3,4,5,		15,16,17,	27,28,29,	39,40,41, …

PEI Group 3: 	6,7,8,		18,19,20,	30,31,32,	42,43,44, …

PEI Group 4: 	9,10,11, 	21,22,23,	33,34,35,	45,46,47, …

This (obviously) means that the LP-WUS subgroup ID is different when using LP-WUS standalone or LP-WUS together with PEI:

When LP-WUS and PEI are used together the UE calculates the subgroup used for LP-WUS (floor (UE_ID / (N * Ns * #PEIsubgroups)) mod #LP-WUSsubgroups).

## LP-WUS and eDRX

LP-WUS can be supported with eDRX with limited specification impact, similar to PEI and eDRX, provided that for PO monitoring within the PTW the same monitoring rules apply as for normal DRX. The only change to the specification agreed for PEI and eDRX was to allow for a larger UEID value (5G-S-TMSI mod 32768 instead of 1024) in 38.304 to properly distribute the subgroups with a larger DRX cycle.

However the use of LP-WUS with eDRX may not always be beneficial as simulations results have shown [1]. For example when the eDRX cycle is very long (e.g. larger than 500 sec), then the MR ultra-deep sleep power consumption (Relative Power unit 0,015) becomes the determining factor for the battery lifetime and it does not matter whether LP-WUS is used or not.

Perhaps the UE implementation decides to switch of the modem, i.e. the power consumption drops to zero, when the eDRX cycle is very long. However this may increase the sync delay and time to wake up, because there was not low power low accuracy clock running:

eDRX is supported with LP-WUS when it can be supported without additional changes, i.e. no separate time offset for eDRX and the UE monitors LP-WUS and POs during PTW similar as during normal DRX.

## Paging delay

When LP-WUS is used then the average paging delay is increased due to the time offset between LP-WUS and PO. When a paging message is ready to be transmitted during the time offset, then the paging message is delayed with one DRX cycle. The time offset for LP-WUS is considerably larger (400-800 ms) compared to the time offset for PEI (configurable offset up to 160 ms e.g. 40 ms):

Paging delay evaluation:

- When LP-WUS is not configured, a Paging message experiences on average half the DRX cycle delay between the time the Paging message is ready for transmission and when the Paging message can be transmitted in the next PO. The maximum paging delay is one DRX cycle.

- When LP-WUS is configured and the Paging message is ready for transmission during the time offset then the Paging message is transmitted in the next DRX cycle, i.e. delay with one DRX cycle. When the time offset to DRX cycle length ratio increases the probability is increased that the Paging message is delayed with one DRX cycle, i.e. the average paging delay is increased.

- The maximum paging delay is increased further when a single LO is used for multiple POs and the POs cover more than one Paging Frame (PF). For example with PEI there can be a 1:8 mapping and POs may belong to at most 2 Paging Frames. When 1/16 of the frames are configured as PFs, then the maximum delay can be 1 DRX cycle + 16 frames.

During the study phase (TR 38.869) 400 ms transition time for MR ramp-up/down has been used as a baseline, but 800 ms was mentioned as an alternative. In RRC_IDLE mode a DRX cycle of 640 ms can be observed in the field, and in RRC_INACTIVE a short DRX cycle down to 320 ms can be configured. However when the time offset exceeds the DRX cycle then the next PO cannot be used for paging:

Thus when the time offset exceeds the DRX cycle, the average paging delay is half the time offset and the maximum delay equals the time offset plus DRX cycle. For example the average paging delay with 640 ms DRX is 320 ms without LP-WUS. When LP-WUS with time offset 800 ms is used, then the average paging delay is increased to 400 ms. Perhaps some of the Paging Frame can be considered “wasted”, i.e. not used. However using a DRX cycle of 1.28 DRX cycle would increase the average paging delay to 640 ms:

The LP-WUS time offset may exceed the DRX cycle, i.e. the LP-WUS periodicity is equal to the DRX cycle.

In RAN1 the use of “dynamic” PO compared to legacy PO is still FFS:

Agreement

It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up.

- FFS: support of UE monitoring dynamic PO

With legacy PO and LP-WUS, the UE monitors the LP-WUS in front of the PO the UE monitors in the DRX cycle. From a UE perspective the LP-WUS periodicity equals the DRX cycle. But with dynamic PO the UE monitors the LP-WUS in front of every PO of the DRX cycle. The intention of dynamic PO is to reduce the paging delay. Instead of waiting for the PO of the UE to arrive the UE can be paged in every PO during the DRX cycle:

As can be seen from the figure above, dynamic PO reduces the paging delay provided that the time offset is rather short, e.g. with PEI. However with LP-WUS the time offset is rather large which reduces the effectiveness of dynamic PO because the probability is high that the paging message arrives during the time offset and the paging message is delayed. But dynamic PO does reduce the latency when the UE needs to be paged in the next DRX cycle, i.e. the UE can be paged in the next PO after the end of the time offset (instead of the designated PO of the UE with legacy paging). When the time offset is larger than the DRX cycle, the paging delay is mainly determined by the time off, even with dynamic PO.

To reduce the paging delay it is also possible to just configure a shorter DRX. For example a shorter default DRX for all UEs in RRC_IDLE, or shorter UE specific DRX via NAS signalling in RRC_IDLE. The DRX cycle in RRC_INACTIVE is configured per UE in RRCRelease. With a shorter DRX the UE has to monitor more often, but with LP-WUS the power consumption can be reduced significantly.

With dynamic PO the number of LP-WUS and paging transmissions are increased significantly. Because the Paging messages will be distributed more evenly over the POs of the DRX cycle, i.e. the number of UEs paged in a single Paging message is reduced. With legacy PO all the paging messages for all the UEs monitoring the same PO are collected during a DRX cycle:

Dynamic PO is not supported.

## Emergency PDU session

When the UE has an emergency PDU session then eDRX is not used because eDRX may delay an (emergency) callback. Furthermore CN-assigned PEI is also not allowed when the UE has an emergency PDU session (see 24.501 section 5.3.25) for latency reasons. However UE-ID based PEI is allowed during an emergency PDU session, because the required signalling was overlooked initially, and the additional latency introduced by PEI was deemed tolerable.

As explained in section 2.10 the additional delay introduced by LP-WUS is much larger than PEI, however it remains small compared to the potential eDRX latency.

In our view CN-assigned and UE-ID based LP-WUS should be treated the same during an emergency session, because the additional latency is the same:

CN-assigned and UE-based LP-WUS are treated the same when the UE has an emergency PDU session, i.e. either both are allowed or neither are allowed during an emergency PDU session.

RAN2 to discuss whether the CN can indicate in PAGING message that UE-ID based LP-WUS is not allowed (e.g. during an emergency PDU session).

## NES

The paging occasions are uniformly distributed over the DRX cycle, e.g. every frame of the DRX cycle can be configured as a Paging Frame (PF) or every other, fourth, eight or sixteenth frame can be configured as a PF. To enable further Network Energy Saving (NES) POs can be grouped in time to enable longer sleep period in the gNB [2]:

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

Longer gNB sleeping periods can be achieved by configuring the paging frames in a DRX cycle consecutively together. Or by increasing the number of POs that can be configured in a single Paging Frame, such that the number of Paging Frames within the DRX cycle can be reduced:

RAN2 should wait for NES progress to discuss potential impact on the LP-WUS configuration e.g. time offset. But RAN2 can agree the LP-WUS can be used when NES is configured i.e. enable power saving in both UE and NW:

LP-WUS can be used when NES is configured.

# Summary

RAN2 is kindly asked to discuss WUR in Idle and Inactive:

Proposal 1	For OFDM-based WUR the entry/exit condition is optional.

Proposal 2	For OOK-based WUR the entry/exit condition is mandatory.

Proposal 3	Separate entry/exit conditions can be configured for OFDM-based and OOK-based WUR.

Proposal 4	For OOK-based WUR the LR entry condition may include an LP-SS threshold.

Proposal 5	If an LP-SS threshold is configured, a threshold below the MR serving cell threshold of the entry condition can be configured where the UE performs LP-SS measurements.

Proposal 6	When LP-WUS is detected and there is a subgroup match then the MR shall perform legacy paging monitoring.

Proposal 7	If the UE is not required to perform any idle mode tasks, then it is left to UE implementation which sleep state to select and when to enter deep sleep, if at all.

Proposal 8	Dedicated RRC signalling is not needed for LP-WUS configuration.

Proposal 9	gNB can configure in SIB whether only OFDM-based or only OOK-based WUR or both types of WUR is supported.

Proposal 10	Dependent on RAN1 progress the LP-SS configuration is optional or absent for OFDM-based WUR.

Proposal 11	A single time offset between LP-WUS occasion (LO) and Paging Occasion (PO) can be configured in SIB assuming that the UE needs to wake-up from ultra-deep sleep.

Proposal 12	The time offset between LP and PO consists of a frame offset (lpwus-FrameOffset) between the start of the Paging Frame including the PO and frame including the LO, and the number of symbols (firstPDCCH-MonitoringOccasionOfLP-WUS-LO) between the start of the frame including the LO and the start of the first MO included in the MO.

Proposal 13	A subgroup can be mapped onto a bit or a codepoint dependent on the configuration in SIB.

Proposal 14	Indicate to RAN1 that it should be possible to configure 8 bits or less for subgrouping.

Proposal 15	A single LO can be mapped onto a single PO or multiple POs (further details are left to RAN1 to decide).

Proposal 16	The UE can indicate to support CN-assigned subgrouping with LP-WUS in the REGISTRATION REQUEST message. The CN can assign a CN-assigned subgroup for LP-WUS monitoring to the UE in the REGISTRATION ACCEPT message.

Proposal 17	The UE can indicate to support UE-ID based subgrouping with LP-WUS in the UERadioPagingInfo IE in UECapabilityInformation message.

Proposal 18	The UE-ID based subgrouping with LP-WUS is not signalled per band.

Proposal 19	It is indicated in SIB whether LP-WUS is only used in the last used cell or in any cell.

Proposal 20	LP-WUS can indicate when there is paging because the UE is not found in the last used cell. UE monitoring LP-WUS in the last used cell is not required to monitor the following PO.

Proposal 21	If the UE detects LP-WUS and the subgroup ID of the UE is included in LP-WUS then the UE monitors the following PO.

Proposal 22	If the UE does not detect LP-WUS or subgroup ID of the UE is not included in LP-WUS then the UE is not required to monitor the following PO.

Proposal 23	If the UE is not able to monitor the LP-WUS (details FFS) then the UE is required to monitor the following PO.

Proposal 24	In case the LO consists of a window including multiple MOs then the UE shall monitor all MOs in the window (of the selected beam in case of multibeam operation).

Proposal 25	When the UE uses both LP-WUS and PEI and LP-WUS indicates that the UE has to monitor paging, then this wake-up call is cancelled when UE receives PEI indicating that the UE does not have to monitor paging. When the UE does not detect PEI the UE monitors the following PO.

Proposal 26	The signalling for CN assigned subgrouping is extended to enable the CN to enable/disable UE-ID based LP-WUS in the UE.

Proposal 27	When both LP-WUS and PEI are configured, it is indicated in SIB whether the UE supporting both LP-WUS and PEI is allowed to use LP-WUS and PEI simultaneously.

Proposal 28	When both LP-WUS and PEI are configured, and it is indicated that the UE is not allowed to use LP-WUS and PEI together, then it is indicated in SIB which one the UE shall use.

Proposal 29	When LP-WUS and PEI are used together the UE calculates the subgroup used for LP-WUS (floor (UE_ID / (N * Ns * #PEIsubgroups)) mod #LP-WUSsubgroups).

Proposal 30	eDRX is supported with LP-WUS when it can be supported without additional changes, i.e. no separate time offset for eDRX and the UE monitors LP-WUS and POs during PTW similar as during normal DRX.

Proposal 31	The LP-WUS time offset may exceed the DRX cycle, i.e. the LP-WUS periodicity is equal to the DRX cycle.

Proposal 32	Dynamic PO is not supported.

Proposal 33	CN-assigned and UE-based LP-WUS are treated the same when the UE has an emergency PDU session, i.e. either both are allowed or neither are allowed during an emergency PDU session.

Proposal 34	RAN2 to discuss whether the CN can indicate in PAGING message that UE-ID based LP-WUS is not allowed (e.g. during an emergency PDU session).

Proposal 35	LP-WUS can be used when NES is configured.

# References

- R1-24xxxxx, LP-WUR operation in IDLE and INACTIVE modes, Ericsson, DISC, RAN1#118

# Annex: WID and agreements

## WID

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

## RAN1 agreements

R1-2401631	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreements

Multi-beam operations are supported for LP-WUS and LP-SS for idle mode

LP-WUS occasions (LOs) are defined for LP-WUS monitoring.

- Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitor for LP-WUS transmission in each of the LP-WUS MOs.

  - Different LP-WUS MOs may correspond to different beams in multi-beam operation

  - FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs

  - FFS details

- It is at least supported that a UE monitors LOs with a configured periodicity.

- FFS eDRX, if supported

For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up.

- FFS: support of UE monitoring dynamic PO

Conclusion

For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

R1-2402895	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

For multi-beam operation of LP-WUS, UE assumes the same LP-WUS information payload is repeated in all transmitted beams corresponding to LP-WUS

- the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation

Agreement

Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam.

- Option 1: K = 1

- Option 2: K can be larger than or equal to 1

  - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Conclusion

LP-SINR is not considered further as a metric for RRM serving cell measurement for OOK-based receiver.

R1-2405681	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Conclusion

Regarding the “FFS: How to determine the received power of LP-SS in OOK ON symbols” for LP-RSRP, no additional work in RAN1.

R1-2404300	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options:

- Option 1: UEs monitoring the same PO monitor the same LO

- Option 2: UEs corresponding to different POs monitor the same LO

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

- Combinations of the above options can be considered.

R1-2404301	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth.

Agreement

For LP-RSSI definition for LP-RSRQ, the following is agreed

- Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols.

Note: Above does not constrain LP-SS sequence design for OOK

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

R1-2405681	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16].

- FFS the exact value of Z

Agreement

For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256.

- FFS the exact value of X.

## RAN2 agreements

The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed.

Working assumption: the LP-WUS configuration in SIB at least includes the following information:

  - LP-SS configuration

  - LP-WUS configuration

  - FFS on Entry/exit condition for LP-WUS monitoring

The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

Entry/exit condition

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

  - LP-SS configuration

  - LP-WUS configuration

  - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured)

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

Sub-grouping

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

SI reception

From RAN2 perspective, no new procedure is introduced for SI reception/updates.